| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 8th Meeting: San José, CA, USA, 1–10 February, 2012 | Document: JCTVC-H0231 |
| --- | --- |

| Title: | CE4: Cross check of CE4 Subset2.4 on JCTVC-Hxxxx |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Cross check |  |  |
| Author(s) or Contact(s): | Yoshitaka Morigami Junichi Tanaka Teruhiko Suzuki 2-10-1, Osaki, Shinagawa-ku , Tokyo, 141-8610 JAPAN | Tel: Email: | +81-50-3750-2740 Yoshitaka.Morigami@jp.sony.com Tanaka.Junichi@jp.sony.com teruhikos@jp.sony.com |
| Source: | Sony Corporation |  |  |

_____________________________

# Abstract

This contribution is cross verification for part of CE4 Subset2.4 on JCTVC-Hxxx.  Using an experiment condition of JCTVC- G1204, It is confirmed that there is no mismatch between reference results and test result. In addition, the content of the source provided by the proponent was investigated in detail.

# Introduction

The proposal in CE4 Subset2.4 uses layered coding structure, and the proposal encodes each frame respectively. Coding parameters of each layer can be set at QMSizexCodeLevels(4x4,8x8,16x16,32x32), and the residul can be send optionally. Prediction modes are as follows,

- Copy mode and down-sampling

- Symmetry mode

- Residue Coding

- Differential coding

# Simulation Results

The coding performance of matrix itself is measured. Encoding and decoding time was measured using class D sequence. To evaluate the most critical case, simulation time was compared for low complexity intra coding for 1 frame. The detailed results are shown in the attached excel-file.

Table 1:The results of lossless coding

| (Subtest2.0, anchor) | symmetry1 | symmetry2 | symmetry3 | symmetry4 | symmetry5 | symmetry6 |
| --- | --- | --- | --- | --- | --- | --- |
| Total Bit | 8779 | 5037 | 12157 | 15677 | 8739 | 10863 |
| MaxError | 0 | 0 | 0 | 0 | 0 | 0 |
| AvgError | 0 | 0 | 0 | 0 | 0 | 0 |
| Enc Time[sec] | 0.7 | 0.66 | 0.73 | 0.7 | 0.66 | 0.67 |
| Dec Time[sec] | 0.01 | 0.02 | 0.02 | 0.01 | 0.02 | 0.01 |
| (subtest2.4),( lossless) | symmetry1 | symmetry2 | symmetry3 | symmetry4 | symmetry5 | symmetry6 |
| Total Bit | 3367 | 2184 | 5209 | 8678 | 3377 | 4704 |
| MaxError | 0 | 0 | 0 | 0 | 0 | 0 |
| AvgError | 0 | 0 | 0 | 0 | 0 | 0 |
| Enc Time[sec] | 0.7 | 0.66 | 0.73 | 0.7 | 0.66 | 0.67 |
| Dec Time[sec] | 0.01 | 0.02 | 0.02 | 0.01 | 0.02 | 0.01 |
| Ratio | 38.4% | 43.4% | 42.8% | 55.4% | 38.6% | 43.3% |
| (Subtest2.0, anchor) | asymmetry1 | asymmetry2 | asymmetry3 | asymmetry4 | asymmetry5 | asymmetry6 |
| Total Bit | 9443 | 5053 | 12707 | 24825 | 9357 | 11259 |
| MaxError | 0 | 0 | 0 | 0 | 0 | 0 |
| AvgError | 0 | 0 | 0 | 0 | 0 | 0 |
| Enc Time[sec] | 0.7 | 0.7 | 0.75 | 0.69 | 0.73 | 0.72 |
| Dec Time[sec] | 0.02 | 0.01 | 0.02 | 0.01 | 0.02 | 0.01 |
| (subtest2.4),( lossless) | asymmetry1 | asymmetry2 | asymmetry3 | asymmetry4 | asymmetry5 | asymmetry6 |
| Total Bit | 5609 | 3744 | 9076 | 23942 | 5652 | 8012 |
| MaxError | 0 | 0 | 0 | 0 | 0 | 0 |
| AvgError | 0 | 0 | 0 | 0 | 0 | 0 |
| Enc Time[sec] | 0.7 | 0.7 | 0.75 | 0.69 | 0.73 | 0.72 |
| Dec Time[sec] | 0.02 | 0.01 | 0.02 | 0.01 | 0.02 | 0.01 |
| Ratio | 59.4% | 74.1% | 71.4% | 96.4% | 60.4% | 71.2% |

Table 2: The results of lossy coding (option1)

| (Subtest2.0, anchor) | symmetry1 | symmetry2 | symmetry3 | symmetry4 | symmetry5 | symmetry6 |
| --- | --- | --- | --- | --- | --- | --- |
| Total Bit | 8779 | 5037 | 12157 | 15677 | 8739 | 10863 |
| MaxError | 0 | 0 | 0 | 0 | 0 | 0 |
| AvgError | 0 | 0 | 0 | 0 | 0 | 0 |
| Enc Time[sec] | 0.7 | 0.66 | 0.73 | 0.7 | 0.66 | 0.67 |
| Dec Time[sec] | 0.01 | 0.02 | 0.02 | 0.01 | 0.02 | 0.01 |
| (subtest2.4),( lossy_opt1) | symmetry1 | symmetry2 | symmetry3 | symmetry4 | symmetry5 | symmetry6 |
| Total Bit | 1535 | 1001 | 2198 | 2751 | 1529 | 1968 |
| MaxError | 23 | 11 | 46 | 27 | 15 | 23 |
| AvgError | 0.55 | 0.26 | 1.03 | 3.75 | 0.55 | 0.69 |
| Enc Time[sec] | 0.68 | 0.66 | 0.71 | 0.69 | 0.7 | 0.7 |
| Dec Time[sec] | 0.02 | 0.01 | 0.02 | 0.02 | 0.01 | 0.02 |
| Ratio | 17.5% | 19.9% | 18.1% | 17.5% | 17.5% | 18.1% |
| (Subtest2.0, anchor) | asymmetry1 | asymmetry2 | asymmetry3 | asymmetry4 | asymmetry5 | asymmetry6 |
| Total Bit | 9443 | 5053 | 12707 | 24825 | 9357 | 11259 |
| MaxError | 0 | 0 | 0 | 0 | 0 | 0 |
| AvgError | 0 | 0 | 0 | 0 | 0 | 0 |
| Enc Time[sec] | 0.7 | 0.7 | 0.75 | 0.69 | 0.73 | 0.72 |
| Dec Time[sec] | 0.02 | 0.01 | 0.02 | 0.01 | 0.02 | 0.01 |
| (subtest2.4),( lossy_opt1) | asymmetry1 | asymmetry2 | asymmetry3 | asymmetry4 | asymmetry5 | asymmetry6 |
| Total Bit | 2615 | 1550 | 3827 | 7016 | 2571 | 3355 |
| MaxError | 8 | 4 | 16 | 105 | 7 | 8 |
| AvgError | 0.40 | 0.19 | 0.79 | 11.78 | 0.39 | 0.54 |
| Enc Time[sec] | 0.7 | 0.7 | 0.74 | 0.69 | 0.7 | 0.7 |
| Dec Time[sec] | 0.01 | 0.01 | 0.01 | 0.02 | 0.01 | 0.01 |
| Ratio | 27.7% | 30.7% | 30.1% | 28.3% | 27.5% | 29.8% |

Table 3: The results of lossy coding (option2)

| (Subtest2.0, anchor) | symmetry1 | symmetry2 | symmetry3 | symmetry4 | symmetry5 | symmetry6 |
| --- | --- | --- | --- | --- | --- | --- |
| Total Bit | 8779 | 5037 | 12157 | 15677 | 8739 | 10863 |
| MaxError | 0 | 0 | 0 | 0 | 0 | 0 |
| AvgError | 0 | 0 | 0 | 0 | 0 | 0 |
| Enc Time[sec] | 0.7 | 0.66 | 0.73 | 0.7 | 0.66 | 0.67 |
| Dec Time[sec] | 0.01 | 0.02 | 0.02 | 0.01 | 0.02 | 0.01 |
| (subtest2.4),( lossy_opt2) | symmetry1 | symmetry2 | symmetry3 | symmetry4 | symmetry5 | symmetry6 |
| Total Bit | 606 | 505 | 997 | 826 | 733 | 903 |
| MaxError | 21 | 14 | 58 | 30 | 24 | 29 |
| AvgError | 1.10 | 0.62 | 2.72 | 5.23 | 1.57 | 1.68 |
| Enc Time[sec] | 0.66 | 0.67 | 0.7 | 0.7 | 0.68 | 0.66 |
| Dec Time[sec] | 0.02 | 0.01 | 0.01 | 0.02 | 0.01 | 0.01 |
| Ratio | 6.9% | 10.0% | 8.2% | 5.3% | 8.4% | 8.3% |
| (Subtest2.0, anchor) | asymmetry1 | asymmetry2 | asymmetry3 | asymmetry4 | asymmetry5 | asymmetry6 |
| Total Bit | 9443 | 5053 | 12707 | 24825 | 9357 | 11259 |
| MaxError | 0 | 0 | 0 | 0 | 0 | 0 |
| AvgError | 0 | 0 | 0 | 0 | 0 | 0 |
| Enc Time[sec] | 0.7 | 0.7 | 0.75 | 0.69 | 0.73 | 0.72 |
| Dec Time[sec] | 0.02 | 0.01 | 0.02 | 0.01 | 0.02 | 0.01 |
| (subtest2.4),( lossy_opt2) | asymmetry1 | asymmetry2 | asymmetry3 | asymmetry4 | asymmetry5 | asymmetry6 |
| Total Bit | 1025 | 761 | 1651 | 2066 | 1235 | 1469 |
| MaxError | 10 | 7 | 32 | 112 | 10 | 16 |
| AvgError | 0.98 | 0.48 | 2.22 | 15.34 | 1.14 | 1.32 |
| Enc Time[sec] | 0.7 | 0.69 | 0.73 | 0.69 | 0.72 | 0.72 |
| Dec Time[sec] | 0.01 | 0.01 | 0.02 | 0.02 | 0.02 | 0.01 |
| Ratio | 10.9% | 15.1% | 13.0% | 8.3% | 13.2% | 13.0% |

Table 4: The results of lossy coding (option3)

| (Subtest2.0, anchor) | symmetry1 | symmetry2 | symmetry3 | symmetry4 | symmetry5 | symmetry6 |
| --- | --- | --- | --- | --- | --- | --- |
| Total Bit | 8779 | 5037 | 12157 | 15677 | 8739 | 10863 |
| MaxError | 0 | 0 | 0 | 0 | 0 | 0 |
| AvgError | 0 | 0 | 0 | 0 | 0 | 0 |
| Enc Time[sec] | 0.7 | 0.66 | 0.73 | 0.7 | 0.66 | 0.67 |
| Dec Time[sec] | 0.01 | 0.02 | 0.02 | 0.01 | 0.02 | 0.01 |
| (subtest2.4),( lossy_opt3) | symmetry1 | symmetry2 | symmetry3 | symmetry4 | symmetry5 | symmetry6 |
| Total Bit | 345 | 329 | 543 | 391 | 419 | 493 |
| MaxError | 20 | 14 | 58 | 32 | 29 | 47 |
| AvgError | 2.38 | 0.96 | 5.09 | 5.81 | 2.82 | 3.59 |
| Enc Time[sec] | 0.7 | 0.66 | 0.69 | 0.69 | 0.68 | 0.69 |
| Dec Time[sec] | 0.02 | 0.01 | 0.02 | 0.01 | 0.02 | 0.01 |
| Ratio | 3.9% | 6.5% | 4.5% | 2.5% | 4.8% | 4.5% |
| (Subtest2.0, anchor) | asymmetry1 | asymmetry2 | asymmetry3 | asymmetry4 | asymmetry5 | asymmetry6 |
| Total Bit | 9443 | 5053 | 12707 | 24825 | 9357 | 11259 |
| MaxError | 0 | 0 | 0 | 0 | 0 | 0 |
| AvgError | 0 | 0 | 0 | 0 | 0 | 0 |
| Enc Time[sec] | 0.7 | 0.7 | 0.75 | 0.69 | 0.73 | 0.72 |
| Dec Time[sec] | 0.02 | 0.01 | 0.02 | 0.01 | 0.02 | 0.01 |
| (subtest2.4),( lossy_opt3) | asymmetry1 | asymmetry2 | asymmetry3 | asymmetry4 | asymmetry5 | asymmetry6 |
| Total Bit | 573 | 471 | 869 | 913 | 673 | 799 |
| MaxError | 11 | 7 | 32 | 113 | 20 | 19 |
| AvgError | 1.97 | 0.88 | 4.80 | 16.66 | 2.83 | 2.94 |
| Enc Time[sec] | 0.69 | 0.68 | 0.74 | 0.68 | 0.69 | 0.71 |
| Dec Time[sec] | 0.02 | 0.01 | 0.02 | 0.02 | 0.01 | 0.01 |
| Ratio | 6.1% | 9.3% | 6.8% | 3.7% | 7.2% | 7.1% |

# Conclusion

This contribution reports the cross check result of Layered quantization matrices compression by CE4 Subset2.4 on JCTVC-Hxxx. The simulation results match closely with those provided by the proponents.

# Patent rights declaration(s)

Sony Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).