| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 8th Meeting: San José, CA, USA, 1–10 February, 2012 | Document: JCTVC-H0233 |
| --- | --- |

| Title: | CABAC bypass for coefficient data |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jani Lainema Kemal Ugur Antti Hallapuro | Email: | jani.lainema@nokia.com |
| Source: | Nokia |  |  |

_____________________________

# Abstract

This contribution proposes an alternative mode for coding transform coefficient data utilizing CABAC bypass. The intention is to allow HEVC codecs to operate in higher throughput mode in use cases requiring low complexity operation especially at high bitrates. In the case encoder decides to operate on high throughput mode the transform coefficient bins are coded in the equal probability bypass mode of CABAC. It is reported that the proposed high throughput mode reduces the average number of context coded bins by 79 % in low QP range and 50 % in common conditions range. The effect of the approach on objective compression efficiency is reported to vary from +5.4 % (RA_HE) to +11.3 % (LB_LC) in common conditions simulations and from +6.7 % (LB_LC) to +11.1 % (AI_HE) on low QP range.

# Introduction

This contribution relates to Core Experiment 1 Subtest D1 that evaluated throughput improvements provided by utilizing CABAC bypass coding with VLC-like binarization of transform coefficients. Instead of proposing modifications to the HM 5.0 binarization this contribution reports results of a scheme that simply applies bypass coding to the most frequently used transform coefficient bins significantly reducing HEVC decoder’s throughput requirements.

# Technical description

The proposed technique is enabled by slice level signalling of the binarization mode. One bit is used to identify whether high efficiency binarization (HEB) referring to the context based HM 5.0 coding of coefficients, or high throughput binarization (HTB) referring to bypass coding of transform coefficient data is utilized. In the case HTB is signalled, the following syntax elements are CABAC bypass coded:

- significant_coeff_group_flag

- significant_coeff_flag

- coeff_abs_level_greater1_flag

- coeff_abs_level_greater2_flag

Required changes to the HEVC WD/CD as well as a software package implementing the method are provided as an attachment to this contribution.

# Throughput improvements

The savings in number of bins going through full context based coding is reported in the tables below. The values for each QP and test configuration show the ratio of full context coded bins in high throughput mode versus those in the anchor, counting all the bins in the bitstream. The “Average” column shows the averages for each QP and the “Saving” column the reduction in the number of bins a decoder has to decode with full CABAC processing in HTB mode. The proposed approach reduces the number of coded bins by 79 % in the low QP range (QP 1 to 13) and 50 % in the common conditions range of QPs (QP 22 to 37). Similarly, the average worst cases savings are 80 % in the low QP range and 64 % in the common conditions range of QPs.

Table 1: Summary of the throughput improvements provided by CABAC HTB binarization with respect to the HM 5.0 anchor.

| QP | AI_HE | RA_HE | LB_HE | RA_10 | AI_LC | RA_LC | LB_LC | Average | Saving |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 16 % | 18 % | 19 % | 13 % | 17 % | 20 % | 21 % | 18 % | 82 % |
| 5 | 19 % | 20 % | 21 % | 16 % | 19 % | 23 % | 23 % | 20 % | 80 % |
| 9 | 21 % | 22 % | 22 % | 17 % | 22 % | 26 % | 26 % | 22 % | 78 % |
| 13 | 23 % | 25 % | 25 % | 20 % | 24 % | 30 % | 29 % | 25 % | 75 % |
| 22 | 29 % | 39 % | 38 % | 25 % | 32 % | 43 % | 42 % | 35 % | 65 % |
| 27 | 35 % | 51 % | 52 % | 32 % | 38 % | 54 % | 54 % | 45 % | 55 % |
| 32 | 42 % | 60 % | 63 % | 43 % | 45 % | 63 % | 64 % | 54 % | 46 % |
| 37 | 50 % | 68 % | 72 % | 58 % | 53 % | 72 % | 74 % | 64 % | 36 % |

Table 2: Summary of the worst-case throughput improvements provided by CABAC HTB binarization with respect to the HM 5.0 anchor.

| QP | AI_HE | RA_HE | LB_HE | RA_10 | AI_LC | RA_LC | LB_LC | Average | Saving |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 16 % | 19 % | 19 % | 15 % | 16 % | 19 % | 19 % | 18 % | 82 % |
| 5 | 17 % | 20 % | 19 % | 17 % | 17 % | 20 % | 21 % | 19 % | 81 % |
| 9 | 19 % | 21 % | 22 % | 19 % | 19 % | 23 % | 23 % | 21 % | 79 % |
| 13 | 20 % | 24 % | 24 % | 21 % | 21 % | 26 % | 26 % | 23 % | 77 % |
| 22 | 26 % | 30 % | 28 % | 21 % | 27 % | 34 % | 32 % | 28 % | 72 % |
| 27 | 32 % | 34 % | 31 % | 22 % | 33 % | 37 % | 37 % | 32 % | 68 % |
| 32 | 38 % | 41 % | 40 % | 24 % | 39 % | 47 % | 46 % | 39 % | 61 % |
| 37 | 44 % | 55 % | 59 % | 28 % | 46 % | 64 % | 64 % | 51 % | 49 % |

# Objective coding results

Tables below summarize the objective coding results both in common conditions (QPs 22, 27, 32 and 37) and at higher bitrates (QPs 1, 5, 9, 13).

Common conditions

Low QP testing (QPs 1, 5, 9, 13)

# Conclusions

The benefits of the proposed approach could be summarized as:

- Removes HEVC throughput bottlenecks making it suitable for the lowest complexity use cases

  - Reduced number of context coded bins by 79 % in low QP range, 50 % in common conditions

- The complexity of the CABAC with HTB is very close to CAVLC complexity as significant portion of the bitstream is coded with bypass mode.

- Allows multiple bins to be decoded together

- Allows encoder to dynamically change the complexity/efficiency trade-off

The technique proposed in this contribution provides significant parsing throughput improvement for HEVC bitstreams with moderate impact on coding efficiency and with minimal implementation burden. Thus, it is proposed to be adopted to the next HEVC draft.

# Patent rights declaration(s)

Nokia may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).