| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 8th Meeting: San José, CA, USA, 1–10 February, 2012 | Document: JCTVC-H0237 |
| --- | --- |

| Title: | Non-CE4 Subtest 2: Improvement on quantization matrix signaling |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Sung-Chang Lim Hui Yong Kim Jinho Lee Jin Soo Choi 218 Gajeong-ro, Yuseong-gu, Daejeon, 305-700, Korea | Tel: Email: | +82-42-860-5772 {sclim, hykim5, jinosoul, jschoi} @etri.re.kr |
| Source: | ETRI (Electronics and Telecommunications Research Institute) |  |  |

_____________________________

# Abstract

This contribution presents an improvement on quantization matrix signaling. The proposed method restricts quantization matrix signaling by minimum and maximum TU sizes and sends a flag that indicates whether to use default matrix or proprietary matrix for each SizeID and MatrixID.

# Problem Statement

In order to control subjective quality of video sequence in HEVC [1], simplified quantization matrix signaling like AVC [2] and quantization matrix representation at APS level are adopted in the last meeting. However, the current quantization matrix signaling has some issue as in the following clauses.

## Problem 1

Due to large block size transform such as 16x16 and 32x32 transforms supported in HEVC, signaling the quantization matrices in bitstream could lead to a significant overhead. Thus, it is desirable to signal the quantization matrices that is needed at the decoder. However, as can be seen from Table 1, although minimum and maximum TU sizes are signaled in SPS, quantization matrices for all TU size are signaled regardless of minimum and maximum TU sizes restriction.

Table 1. Quantization matrix signaling regardless of min/max TU sizes restriction in HM5

SizeID represents the size of quantization matrix as described in Table 2. For example, even though Log2MaxTrafoSize is equal to 4, that is maximum TU size is equal to 16x16, quantization matrices for 32x32 TU have to be signaled. Thus, large quantization matrices are signaled unnecessarily in the current HM5 syntax.

Table 2. Mapping table of SizeID in HM5

## Problem 2

use_default_scaling_list_flag is signaled to use default matrix or proprietary matrix for all quantization matrices in the current APS syntax as described in Table 1. That is, if use_default_scaling_list_flag is equal to 1, the value of quantization matrices is not present and the values of all matrices are set equal to the default matrix. Therefore, mixed use of default and proprietary quantization matrices depending on TU sizes results in unnecessary signaling of default QM. Table 3 illustrates some cases when the unnecessary signaling occurs. For example, for slice #3, if 16x16 TU uses default matrix and 32x32 TU uses proprietary matrix and use_default_scaling_list_flag is set equal to 0, then default matrix for 16x16 TU has to be signaled in APS while default matrix for 16x16 TU is existed in decoder.

Table 3. Unnecessary signaling of default QM in HM5

# Proposal

## Proposal 1

In the proposal 1, signaling of quantization matrix is restricted by minimum and maximum TU sizes which are signaled in SPS. Table 4 and Table 5 show the proposed syntax.

Table 4. Restricted quantization matrix signaling by min/max TU sizes (variant 1)

Table 5. Restricted quantization matrix signaling by min/max TU sizes (variant 2)

For example of Table 4, when Log2MinTrafoSize is equal to 2, that is minimum TU size is equal to 4x4 and Log2MaxTrafoSize is equal to 3, that is maximum TU size is equal to 8x8, quantization matrices for 16x16 and 32x32 TUs are not signaled in the proposed syntax. Therefore, signaling of unnecessary matrix for TUs that are not supported at the decoder is avoided.

## Proposal 2

In the proposal 2, a flag that indicates whether to use default matrix or proprietary matrix for each SizeID and MatrixID is signaled in APS. Table 6 shows the proposed syntax.

Table 6. Sending a flag to use default matrix or proprietary matrix for each SizeID and MatrixID

For example, when the encoder wants to use default matrix for 16x16 TU and proprietary matrix for 32x32 TU, sid_mid_use_default_scaling_list_flag is set equal to 0 for 16x16 TU and sid_mid_use_default_scaling_list_flag is set equal to 1 for 32x32 TU. Therefore, the degree of freedom to select between default matrix and proprietary matrix for each quantization matrix within a slice is increased.

## Combination of proposal 1 and proposal 2

Combined syntax of proposal 1 (variant 1) and proposal 2 and Combined syntax of proposal 1 (variant 2) and proposal 2 are proposed as described in Table 7 and Table 8, respectively.

Table 7. Combined syntax of proposal 1 (variant 1) and proposal 2

Table 8. Combined syntax of proposal 1 (variant 2) and proposal 2

## Combination of JCTVC-H0237 proposal 2 and JCTVC-H0255

Combined syntax of JCTVC-H0237 proposal 2 and JCTVC-H0255 are proposed as described in Table 9. Blue color text in Table 9 and Red color text in Table 9 is proposed in this proposal and JCTVC-H0255, respectively. scaling_list_sub_present_flag[SizeID][MatrixID] specifies whether the quantization matrix parameters where an index of transform block size is equal to SizeID and an matrix type index is equal to MatrixID exist in this APS or not. Also, the previously coded quantization matrix with SizeID and MatrixID can be replaced by the quantization matrix with SizeID and MatrixID in this APS. Therefore, combination of JCTVC-H0237 proposal 2 and JCTVC-H0255 can update the quantization matrices in APS partially and avoid unnecessary signaling of default QMs when both the default and proprietary matrices are used for a slice.

Table 9. Combined syntax of JCTVC-H0237 proposal 2 and JCTVC-H0255

# Conclusions

In this contribution, an improvement on quantization matrix signaling is presented. By the proposed method, signaling of unnecessary matrix for TU that is not supported at the decoder is avoided and the degree of freedom to select between default matrix and proprietary matrix for each quantization matrix within a slice is increased. It is recommended to adopt the proposed quantization matrix signaling into HM.

# Reference

- K. Sato and H. Sakurai, "Necessity of Quantization Matrices Compression in HEVC," JCTVC-E056, Mar. 2011.

- Y. Morigami, J. Tanaka, and T. Suzuki, "CE4 subtest 3: Quantization matrix for HEVC based on JCTVC-F362 and F475,", JCTVC-G434, Nov. 2011.

- S. Wenger, "JCT-VC break-out report: Adaptation Parameter Set issues,", JCTVC-G1016, Nov. 2011.

# Patent rights declaration(s)

ETRI may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Annex: Proposed text changes (proposal 1 and proposal 2)

### Scaling list data syntax

| scaling_list_param( ) { | Descriptor |
| --- | --- |
| use_default_scaling_list_flag | u(1) |
| if( !use_default_scaling_list_flag ) |  |
| for( SizeID = Log2MinTrafoSize-2; SizeID < Log2MaxTrafoSize-1; SizeID++ ) |  |
| for( MatrixID = 0; MatrixID < (SizeID = = 3) ? 2:6; MatrixID++ ) { |  |
| sid_mid_use_default_scaling_list_flag[SizeID][MatrixID] | u(1) |
| if (!sid_mid_use_default_scaling_list_flag[SizeID][MatrixID]) { |  |
| pred_mode_flag | u(1) |
| if( !pred_mode_flag ) |  |
| pred_matrix_id_delta | ue(v) |
| else |  |
| scaling_list( QuantMatrix[ SizeID ][ MatrixID ], ( 1 << ( 4 + ( SizeID << 1) ) ) ) |  |
| } |  |
| } |  |
| } |  |

### Scaling list parameter semantics

sid_mid_use_default_scaling_list_flag[SizeID][MatrixID] equal to 0 specifies that the value of scaling list specified by SizeID and MatrixID is present. sid_mid_use_default_scaling_list_flag equal to 1 specifies that the value of scaling list specified by SizeID and MatrixID is not present and the values of scaling list specified by SizeID and MatrixID are set equal to the default scaling list.