| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 8th Meeting: San José, CA, USA, 1–10 February, 2012 | Document: JCTVC-H0238 |
| --- | --- |

| Title: | Non-CE6: Simplification of intra vertical/horizontal prediction |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jinho Lee, Sung-Chang Lim, Hui Yong Kim, Jin Soo Choi. Realistic Media Research Team, ETRI 218 Gajeongno, Yuseong-gu Daejeon, South Korea | Tel: Email: | +82-42-860-5656 jinosoul@etri.re.kr sclim@etri.re.kr hykim5@etri.re.kr jschoi@etri.re.kr |
| Source: | ETRI (Electronics and Telecommunications Research Institute) |  |  |

_____________________________

# Abstract

This contribution presents simplification of intra vertical and horizontal prediction by removing division operation in the secondary boundary filtering process. The proposed method replaces the division operation by right shift operation in HM, and removes comparator in WD. It is reported that the average BD-rate is zero in Intra, both High-Efficiency (HE) and Low-Complexity (LC).

# Introduction

In the last meeting, boundary filtering of intra vertical and horizontal predicted samples was adopted in WD5.0[1]. It reflects variation of the secondary boundary reference samples to the predicted boundary samples as an example of vertical prediction in Fig 1.

Vertical prediction:predSamples[x,y]=p[x,−1], with x=1~nS−1, y=0~nS−1predSamples[x,y]=Clip1Yp[x,−1]+p[−1,y]−p[−1,−1]2, with x=0, y=0~nS−1

Figure 1 : Vertical prediction of HM5.0.

The division operation in the equation is described in WD as the following equations considering right shift operation when the value of d[y] or d[x] is negative.

Vertical prediction: predSamples[x,y]=Clip1Yp[x,−1]+d[y]+d[y]<0?1:0>>1where d[y]=p[−1,y]−p[−1,−1], x=0, y=0~nS-1

Horizontal prediction: predSamples[x,y]=Clip1Yp[−1,y]+d[x]+d[x]<0?1:0>>1where d[x]=p[x,−1]−p[−1,−1], x=0~nS-1, y=0

# Proposed method

The intra vertical and horizontal filtering includes division operation in HM, and it can be implemented with comparator and shift operation. However, it requires checking the sign of the difference value of neighboring samples. If the value is negative, 1 is added to the difference value before performing right shift operation.

This contribution proposes a simple filtering method which is replacing division operation by shift operation as illustrated in Fig 2. The description of WD text for the filtering is as same as the equation in Fig 2.

Vertical prediction:predSamples[x,y]=p[x,−1], with x=1~nS−1, y=0~nS−1predSamples[x,y]=Clip1Yp[x,−1]+p[−1,y]−p[−1,−1]>>1, with x=0, y=0~nS−1

Figure 2 : Vertical prediction using proposed method.

# Simulation results

The proposed method was performed based on HM5.0 software under common test condition [2]. The experiments are conducted on the platform of Intel Core i7 CPU 990X @ 3.46GHz with 24GB RAM. The method is cross-checked by Mitsubishi (JCTVC-H0614).

Table 1 shows the performance of the proposed method.

Table 1 : Performance of the proposed method.

|  | All Intra HE |  |  | All Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A (8bit) | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class C | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class D | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class E | 0.0% | 0.1% | 0.1% | 0.0% | 0.0% | 0.0% |
| Overall | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class F | 0.0% | 0.0% | 0.1% | 0.1% | 0.3% | -0.1% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 99% |  |  | 100% |  |  |

- Conclusions

The proposed method simplifies intra vertical and horizontal filtering by removing division operation without coding loss. We suggest the simplified method to be adopted into the HM.

# Patent rights declaration(s)

ETRI (Electronics and Telecommunications Research Institute) may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Reference

- JCT-VC, “High Efficiency Video Coding (HEVC) text specification Working Draft 5”, JCTVC-G1103, JCT-VC Meeting, Geneva, November 2011.

- Frank Bossen, "Common test conditions and software reference configurations," JCTVC-G1200, JCT-VC Meeting, Geneva, November 2011.

# Annex - Proposed text changes (Based on WD5_d8)

        - Specification of Intra_Vertical prediction mode

Inputs to this process are:

–	neighbouring samples p[ x, y ], with x, y = -1..2*nS-1,

–	a variable nS specifying the prediction size

–	a variable cIdx specifying the chroma component of the current block

Output of this process is:

–	predicted samples predSamples[ x, y ], with x, y =0..nS-1.

This intra prediction mode is invoked when intraPredMode is equal to 0.

The values of the prediction samples predSamples[ x, y ], with x, y = 0..nS-1, are derived as follows:

–	If cIdx is equal to 0,

predSamples[ x, y ] = p[ x, -1 ], with x = 1..nS-1, y = 0..nS-1	(8-1)

predSamples[ x, y ] = Clip1Y( p[ x, -1 ] + ( ( d[ y ] + ( d[ y ] < 0 ? 1 : 0 ) )>> 1 ) ), with x = 0, y = 0..nS-1

where d[ y ] = p[ -1, y ] – p[ -1, -1 ]	(8-2)

predSamples[ x, y ] = Clip1Y( p[ x, -1 ] + ( ( p[ -1, y ] - p[ -1, -1 ] )>> 1 ) ), with x = 0, y = 0..nS-1

–	Otherwise,

predSamples[ x, y ] = p[ x, -1 ], with x, y = 0..nS-1		(8-3)

        - Specification of Intra_Horizontal prediction mode

Inputs to this process are:

–	neighbouring samples p[ x, y ], with x, y = -1..2*nS-1,

–	a variable nS specifying the prediction block size.

–	a variable cIdx specifying the chroma component of the current block

Output of this process is:

–	predicted samples predSamples[ x, y ], with x, y =0..nS-1.

This intra prediction mode is invoked when intraPredMode is equal to 1.

The values of the prediction samples predSamples[ x, y ], with x, y = 0..nS-1, are derived as follows:

–	If cIdx is equal to 0,

predSamples[ x, y ] = p[ -1, y ], with x = 0..nS-1, y = 1..nS-1	(8-4)

predSamples[ x, y ] = Clip1Y( p[ -1, y ] + ( ( d[ x ] + ( d[ x ] < 0 ? 1 : 0 ) ) >> 1 ), with x = 0..nS-1, y = 0

where d[ x ] = p[ x, -1 ] – p[ -1, -1 ]	(8-5)

predSamples[ x, y ] = Clip1Y( p[ -1, y ] + ( ( p[ x, -1 ] – p[ -1, -1 ] ) >> 1 ), with x = 0..nS-1, y = 0

–	Otherwise,

predSamples[ x, y ] = p[ -1, y ], with x, y = 0..nS-1		(8-6)