| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 8th Meeting: San José, CA, USA, 1–10 February, 2012 | Document: JCTVC-H0239 |
| --- | --- |

| Title: | Non-CE9: Simplification on AMVP list construction |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Sung-Chang Lim Hui Yong Kim Jinho Lee Jin Soo Choi 218 Gajeong-ro, Yuseong-gu, Daejeon, 305-700, Korea | Tel: Email: | +82-42-860-5772 {sclim, hykim5, jinosoul, jschoi} @etri.re.kr |
| Source: | ETRI (Electronics and Telecommunications Research Institute) |  |  |

_____________________________

# Abstract

This contribution presents a simplification on AMVP list construction. The proposed method always add a zero motion vector into the motion vector candidate list when the number of motion vector candidate in the motion vector candidate list is less than the maximum number of motion vector candidate, equal to 2 in HM5.0, so that the list empty check and the duplicate check in motion vector derivation process can be removed. The experimental results show that the proposed method does not change the coding efficiency because HM5.0 encoder always select the first zero motion vector candidate when there are two zero motion vectors in the motion vector candidate list.

# Introduction

In the last meeting, merge candidate duplicate check for combined merge candidates, non-scaled merge candidates, and zero merge candidates in merge mode is adopted into HM5.0 in order to reduce the computational complexity [1][2].

In HM5.0, the motion vector candidate list empty check and the duplicate check of zero motion vector are performed in the motion vector derivation process for AMVP as described in Fig. 1. The list empty check is performed in order to add a zero motion vector after motion vector candidate pruning process. Also, the duplicate check of zero motion vector is performed in order to add a zero motion vector when the number of motion vector candidate numMVPCandLX in the motion vector candidate list is less than the maximum number of motion vector candidate maxNumMVPCand, equal to 2 in HM5.0. Thus, only one zero motion vector is existed in the motion vector candidate list. However, the list empty check and the duplicate check increase some computational complexity in the motion vector derivation process for AMVP.

Fig. 1. Motion vector derivation process for AMVP in HM5.0

# Proposal

In this contribution, the removal of abovementioned motion vector candidate list empty check and zero motion vector duplicate check are proposed. And, when the number of motion vector candidate numMVPCandLX in the motion vector candidate list is less than the maximum number of motion vector candidate maxNumMVPCand, a zero motion vector is always added in the motion vector candidate list. Thus, maximum two zero motion vectors are existed in the motion vector candidate list by the proposed method. This is the only difference result of the motion vector derivation process between HM5.0 and the proposed method. The proposed simplification on AMVP list construction is described in Fig. 2.

Fig. 2. Proposed motion vector derivation process for AMVP

# Experimental results

The proposed method was implemented on HM5.0 software. The experiments are conducted on the platform of Intel Core i7 CPU 990X @ 3.46GHz with 24GB RAM. Table 1 shows the summary result of the proposed method compared to HM 5.0 anchor under common test conditions [3].

Table 1. Summary result of the proposed method

The result of proposed method is same as that of HM5.0 anchor. The reason is as follows. Only one zero motion vector can be existed in the motion vector candidate list of AMVP in HM5.0. However, in the proposed method, maximum two zero motion vector can be existed in the motion vector candidate list of AMVP. And, in HM5.0 encoder, the best motion vector candidate index is selected by rate-distortion optimization. The rate term in R-D cost equation is calculated by fixed-length code of motion vector candidate index. The length of fixed-length code of motion vector candidate index is always equal to 1 because the maximum motion vector candidate list size is equal to 2. The proposed method does not change the coding efficiency because HM5.0 encoder always selects the first zero motion vector candidate as the best motion vector candidate when R-D cost of first zero motion vector and R-D cost of second zero motion vector are the same.

# Conclusions

The proposed method always add a zero motion vector into the motion vector candidate list when the number of motion vector candidate in the motion vector candidate list is less than the maximum number of motion vector candidate so that the list empty check and the duplicate check in motion vector derivation process can be removed. Thus, the AMVP list construction is simplified. The experimental results show that the proposed method does not change the coding efficiency because HM5.0 encoder always selects the first zero motion vector candidate as the best motion vector candidate when R-D cost of first zero motion vector and R-D cost of second zero motion vector are the same. It is recommended to adopt the proposed method into HM.

# Reference

- B. Li, J. Xu, and H. Li, "Non-CE9/Non-CE13: Simplification of adding new merge candidates," JCTVC-G397, Nov. 2011.

- T. Sugio and T. Nishi, "Non-CE9/Non-CE13: Simplification on AMVP/Merge," JCTVC-G542, Nov. 2011.

- F. Bossen, "Common HM test conditions and software reference configurations," JCTVC-G1200, Nov. 2011.

# Patent rights declaration(s)

ETRI may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Annex: Proposed text changes based on WD5_d6

- 8.4.2.1.6	Derivation process for luma motion vector prediction

Inputs to this process are

- a luma location ( xP, yP ) specifying the top-left luma sample of the current prediction unit relative to the top-left sample of the current picture,

- variables specifying the width and the height of the prediction unit for luma, nPSW and nPSH.

- the reference index of the current prediction unit partition refIdxLX (with X being 0 or 1).

Output of this process is

- the prediction mvpLX of the motion vector mvLX (with X being 0 or 1).

The motion vector predictor mvpLX is derived in the following ordered steps.

- The derivation process for motion vector predictor candidates from neighboring prediction unit partitions in subclause Error: Reference source not found is invoked with luma location ( xP, yP ), the width and the height of the prediction unit nPSW and nPSH, and refIdxLX (with X being 0 or 1, respectively) as inputs and the availability flags availableFlagLXN and the motion vectors mvLXN with N being replaced by A, B as the output.

- If both availableFlagLXA and availableFlagLXB are equal to 1 and mvLXA is not equal to mvLXB, availableFlagLXCol is set equal to 0, otherwise, the derivation process for temporal luma motion vector prediction in subclause Error: Reference source not found is invoked with luma location ( xP, yP ) , the width and the height of the prediction unit nPSW and nPSH, and refIdxLX (with X being 0 or 1, respectively) as the inputs and with the output being the availability flag availableFlagLXCol and the temporal motion vector predictor mvLXCol.

- The motion vector predictor candidate list, mvpListLX, is constructed as follows.

- mvLXA, if availableFlagLXA is equal to 1

- mvLXB, if availableFlagLXB is equal to 1

- mvLXCol, if availableFlagLXCol is equal to 1

- When motion vectors have the same value, the motion vectors are removed from the list except the motion vector which has the smallest order in the mvpListLX.

- When mvpListLX is empty, a zero motion vector is added as follows.

mvpListLX[ 0 ][0] = 0	(8-1)

mvpListLX[ 0 ][1] = 0	(8-2)

- The variable numMVPCandLX is set to the number of elements within the mvpListLX and maxNumMVPCand is set to 2.

- The motion vector predictor list is modified to contain exactly maxNumMVPCand motion vector predictor candidates as follows.

    - If numMVPCandLX is less than maxNumMVPCand, the following applies. the derivation process for zero motion vector predictor candidates specified in subclause  is invoked with mvpListLX and numMVPCandLX given as input and the output is assigned to mvpListLX and numMVPCandLX.

mvpListLX[ numMVPCandLX ][ 0 ] = 0	(8-330)

mvpListLX[ numMVPCandLX ][ 1 ] = 0	(8-431)

numMVPCandLX = numMVPCandLX + 1	(8-532)

    - Otherwise (numMVPCandLX is equal to or greater than maxNumMVPCand), all motion vector predictor candidates mvpListLX[ idx ] with idx greater than maxNumMVPCand − 1 are removed from the list.

- The motion vector of mvpListLX[ mvp_lX_flag[ xP, yP ] ] is assigned to mvpLX.

- 8.4.2.1.9	Derivation process for zero motion vector predictor candidates

Inputs of this process are

- a motion vector predictor list mvpListLX,

- the number of elements numMVPCandLX within mvpListLX.

Outputs of this process are

- the motion vector predictor list mvpListLX,

- the number of elements numMVPCandLX within mvpListLX.

When no motion vector in mvpListLX is equal to (0,0), the zero motion vector predictor candidate is added at the end of mvpListLX and numMVPCandLX is incremented by 1 as follows.

mvpListLX[ numMVPCandLX ][ 0 ] = 0	(8-6)

mvpListLX[ numMVPCandLX ][ 1 ] = 0	(8-7)

numMVPCandLX = numMVPCandLX + 1	(8-8)