| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 8th Meeting: San José, CA, USA, 1–10 February, 2012 | Document: JCTVC-H0240_r2 |
| --- | --- |

| Title: | Non-CE9: Throughput improvement for merge/skip mode |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Hui Yong Kim, *Kyung Yong Kim, *Sang Min Kim, *Gwang Hoon Park, Seunghyun Cho, Sung-Chang Lim, Jinho Lee, and Jin Soo Choi ETRI, 218 Gajeong-ro, Yuseong-gu, Daejeon, 305-700, Korea *Media Lab., Kyung Hee University 1, Seochun, Kiheung, Youngin, Gyonggi, 446-701, Korea | Tel: Email: | +82-42-860-5772 {sclim, hykim5, jinosoul, shcho, jschoi}@etri.re.kr *+82-31-201-3680 {kimky, jephiros, ghpark} @khu.ac.kr |
| Source: | ETRI and Kyung Hee University |  |  |

_____________________________

# Abstract

This contribution proposes a CU-based approach for merge candidate list construction, where all the PUs inside a CU shares a single MCL (Merge Candidate List). It is indicated that the proposed approach not only removes inter-PU dependency but also reduces the number of MCLs that should be constructed for merge/skip estimation to 14% of HM5, hence making the HM design much friendlier to high-throughput applications. Two restricted version of the proposal are also presented: Variant 1 only removes inter-PU dependency while Variant 2 only applies the proposal for 8x8 CUs. It is reported that the average coding losses are roughly 0.3% for the original proposal, 0.0% for the Variant 1, and 0.1% for the Variant 2, respectively.

# Introduction

In the current HM design, there is sequential dependency between PUs since the spatial position for TMVP reference index derivation could be overlapped with another PU as illustrated in Fig. 1. This sequential dependency between PUs prohibits parallel merge estimation PU0 and PU1, and hence makes it very difficult to meet high-throughput requirement from e.g., UHDTV applications.

Fig.1 Inter-PU dependency in TMVP reference index derivation

In the current HM design, every PU has its own MCL (Merge Candidates List). At encoder side, the number of MCLs that should be constructed for mode decision could reach to a very large number. For a 64x64 block, this number is 593 as calculated in the Table 1. The larger the number, there is the more chance of occurrence of different motion candidates increasing the memory contention possibility. Note that even when the MCLs could be constructed in parallel, the memory can be accessed only in a sequential manner. Thus for high-throughput encoder design, it is desirable to reduce the number of different motion candidates as much as possible.

Table 1. The number of MCLs that should be constructed for 64x64 mode decision

| CU Size | # MCL constructions for a 64x64 block |
| --- | --- |
| 64x64 | 13 |
| 32x32 | 13*4 = 52 |
| 16x16 | 13*4*4 = 208 |
| 8x8 | 5*4*4*4 = 320 |
| Sum | 593 |

# Proposal

In this contribution, we propose a CU-based approach for MCL (Merge Candidate List) construction, where all the PUs inside a CU shares a single MCL, which is the 2Nx2N PU’s. Fig. 2 illustrates this concept for SMP cases. The same applies also for AMP cases.

Fig.2 Proposed Solution: all PUs share a single MCL, which is the 2Nx2N PU.

Note that the proposed solution not only removes inter-PU dependency in TMVP reference index derivation but also significantly reduces the number of MCLs that should be constructed for mode decision. Table 2 shows that the number for a 64x64 block is reduced from 594 to 85, which is just 14% of HM5 case.

Table 2. The number of MCLs that should be constructed for 64x64 mode decision

| CU Size | # MCL constructions for a 64x64 block |  |  |
| --- | --- | --- | --- |
|  | HM5 | Proposed | Ratio to HM5 |
| 64x64 | 13 | 1 | 1 : 13 (7.7%) |
| 32x32 | 13*4 = 52 | 4 | 1 : 13 (7.7%) |
| 16x16 | 13*4*4 = 208 | 4*4 = 16 | 1 : 13 (7.7%) |
| 8x8 | 5*4*4*4 = 320 | 4*4*4 = 64 | 1 : 5 (20.0%) |
| Sum | 593 | 85 | 1 : 7 (14%) |

Fig. 3 compares the worst case number of different motion candidates for a 64x64 block at 8x8 CU level. For simple calculation some assumptions were made as written in Fig. 3. Note that the worst case number is reduced from 432 to 232, which is roughly the half of HM5 case.

Fig.3 Rough comparison of worst-case number of different motion candidates in a 64x64 block:
the proposed method reduces the worst case number roughly by half at 8x8 CU level.

Note also that the proposed solution also gives benefits to decoders as well as to encoders: Since MCL is constructed only once for a CU, the MCL construction cycles are always fixed to the minimum regardless the actual number of PUs in the CU. This improves both parallelism and simplicity of the HM design.

This contribution also presents two differently restricted versions of the proposed solution: Variant 1 only removes inter-PU dependency in TMVP reference index derivation by sharing a TMVP refIdx position of 2Nx2N PU with all the other PU partitions. Variant 2 applies the proposed solution only for 8x8 CUs, which is the worst case.

# Experimental results

The proposed method was implemented on HM5.0 software. Table 3, Table 4, and Table 5 show the summary results of the proposed solution, the Variant 1, and the Variant 2, respectively, compared to HM 5.0 anchor under common test conditions [2].

Table 3. Summary result of the proposed method

Table 4. Summary result of the Variant 1 (i.e., for TMVP refIdx only)

Table 5. Summary result of the Variant 2 (i.e., for 8x8 CUs only)

# Conclusions

We proposed the CU-based MCL construction approach for throughput improvement of merge/skip mode. The proposed solution removes inter-PU dependency in merge mode, reduces the required number of MCLs for merge estimation by about 86%, reduces the worst case number of different motion candidates roughly by half, and hence generally makes the HM design much friendlier to high-throughput implementation while introducing just 0.3% loss in average. We also provided two differently restricted version of the original solution to show the possibility of still improving the throughput features with less loss. We recommend adopting the proposed solution for the next version of HM.

# Reference

- H. Y. Kim et al., “CU-based Merge Candidate List Construction,” JCTVC-G416, Nov. 2011.

- F. Bossen, "Common HM test conditions and software reference configurations," JCTVC-G1200, Nov. 2011.

# Patent rights declaration(s)

ETRI and Kyung Hee University may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).