# Annex A: WD5(draft3) modification for the proposed variant 1

### Decoding process for prediction units in inter prediction mode

Inputs to this process are:

–	a luma location ( xC, yC ) specifying the top-left luma sample of the current coding unit relative to the top left luma sample of the current picture,

–	a luma location ( xB, yB ) specifying the top-left luma sample of the current prediction unit relative to the top left luma sample of the current coding unit,

–	a variable nCS specifying the size of the current coding unit,

–	a variable nPSW specifying the width of the current prediction unit,

–	a variable nPSH specifying the width of the current prediction unit,

- a variable PartIdx specifying the index of the current prediction unit within the current coding unit.

Outputs of this process are:

–	a (nCSL)x(nCSL) array predSamplesL of luma prediction samples, where nCSL is derived as specified below,

–	a (nCSC)x(nCSC) array predSamplesCb of chroma prediction samples for the component Cb, where nCSC is derived as specified below,

–	a (nCSC)x(nCSC) array predSamplesCr of chroma prediction samples for the component Cr, where nCSC is derived as specified below.

The variable nCSL is set equal to nCS and the variable nCSC is set equal to nCS >> 1. [Ed: (WJ) revisit for supporting other chroma formats]

The decoding process for prediction units in inter prediction mode consists of the following ordered steps:

- Derivation process for motion vector components and reference indices as specified in subclause 8.4.2.1.

Inputs to this process are

–	a luma location ( xC, yC ) of the top-left luma sample of the current coding unit relative to the top-left luma sample of the current picture,

–	a luma location ( xB, yB ) of the top-left luma sample of the current prediction unit relative to the top-left luma sample of the current coding unit,

–	a variable nCS specifying the size of the current coding unit,

–	variables specifying the width and the height of the prediction unit for luma, nPSW and nPSH,

  - a variable PartIdx specifying the index of the current prediction unit within the current coding unit.

Outputs of this process are

–	luma motion vectors mvL0 and mvL1, and chroma motion vectors mvCL0 and mvCL1,

–	reference indices refIdxL0 and refIdxL1,

–	prediction list utilization flags predFlagL0 and predFlagL1.

- Decoding process for inter sample prediction as specified in subclause Error: Reference source not found.

Inputs to this process are

–	a luma location ( xC, yC ) of the top-left luma sample of the current coding unit relative to the top-left luma sample of the current picture,

–	a luma location ( xB, yB ) of the top-left luma sample of the current prediction unit relative to the top-left luma sample of the current coding unit,

–	a variable nCS specifying the size of the current coding unit,

–	variables specifying the width and the height of the prediction unit for luma, nPSW and nPSH.

–	luma motion vectors mvL0 and mvL1, and chroma motion vectors mvCL0 and mvCL1,

–	reference indices refIdxL0 and refIdxL1,

–	prediction list utilization flags predFlagL0 and prefFlagL1.

Outputs of this process are

–	inter prediction samples (predSamples); which are a (nCSL)x(nCSL) array predSamplesL of prediction luma samples and two (nCSC)x(nCSC) arrays predSamplesCr, and predSamplesCr of prediction chroma samples, one for each of the chroma components Cb and Cr.

For use in derivation processes of variables invoked later in the decoding process, the following assignments are made:

MvL0[ xB, yB ] = mvL0		(8-1)
MvL1[ xB, yB ] = mvL1		(8-2)

RefIdxL0[ xB, yB ] = refIdxL0		(8-3)
RefIdxL1[ xB, yB ] = refIdxL1		(8-4)

PredFlagL0[ xB, yB ] = predFlagL0		(8-5)
PredFlagL1[ xB, yB ] = predFlagL1		(8-6)

#### Derivation process for motion vector components and reference indices

Input to this process are

- a luma location ( xC, yC ) of the top-left luma sample of the current coding unit relative to the top-left luma sample of the current picture,

- a luma location ( xB, yB ) of the top-left luma sample of the current prediction unit relative to the top-left luma sample of the current coding unit,

- a variable nCS specifying the size of the current coding unit,

- variables specifying the width and the height of the prediction unit for luma, nPSW and nPSH,

- a variable PartIdx specifying the index of the current prediction unit within the current coding unit.

Outputs of this process are

- luma motion vectors mvL0 and mvL1 and chroma motion vectors mvCL0 and mvCL1,

- reference indices refIdxL0 and refIdxL1,

- prediction list utilization flags predFlagL0 and predFlagL1.

Let ( xP, yP ) specify the top-left luma sample of the current prediction unit relative to the top-left luma sample of the current picture where xP = xC + xB and yP = yC + yB.

For the derivation of the variables mvL0 and mvL1, refIdxL0 and refIdxL1 as well as PredFlagL0 and PredFlagL1, the following applies.

- If PredMode is equal to MODE_SKIP, the derivation process for luma motion vectors for merge mode as specified in subclause 8.4.2.1.1 is invoked with the luma location ( xC, yC ), the luma location ( xP, yP ), variables nCS, nPSW, nPSH and the partition index PartIdx as inputs and the output being the luma motion vectors mvL0, mvL1, the reference indices refIdxL0, refIdxL1, and the prediction list utilization flags predFlagL0 and predFlagL1.

- Otherwise, if PredMode is equal to MODE_INTER and merge_flag[ xP ][ yP ] is equal to 1,, the derivation process for luma motion vectors for merge mode as specified in subclause 8.4.2.1.1 is invoked with the luma location ( xC, yC ), the luma location ( xP, yP ), variables nCS, nPSW and nPSH and the partition index PartIdx as inputs and the outputs being the luma motion vectors mvL0 and mvL1, the reference indices refIdxL0 and refIdxL1, the prediction utilization flags predFlagL0 and predFlagL1.

- Otherwise, for X being replaced by either 0 or 1 in the variables predFlagLX, mvLX, refIdxLX and in Pred_LX and in the syntax elements ref_idx_lX and mvd_lX, the following applies.

- The variables LcToLx, refIdxLX and predFlagLX are derived as follows.

- If inter_pred_flag[ xP ][ yP ] is equal to Pred_LC and PredLCToPredLx[ ref_idx_lc[ xP ][ yP ] ] is equal to Pred_LX,

refIdxLX = RefIdxLCToRefIdxLx[ ref_idx_lc[ xP ][ yP ] ]	(8-7)					predFlagLX = 1		(8-8)					mvd_lX[ xP ][ yP ][ 0 ] = mvd_lc[ xP ][ yP ][ 0 ]	(8-9)
					mvd_lX[ xP ][ yP ][ 1 ] = mvd_lc[ xP ][ yP ][ 1 ]	(8-10)
					mvp_idx_lX[ xP ][ yP ][ 0 ] = mvp_idx_lc[ xP ][ yP ]	(8-11)
					LcToLx = LX		(8-12)

- Otherwise, if inter_pred_flag[ xP ][ yP ] is equal to Pred_LX or Pred_BI,

refIdxLX = ref_idx_lX[ xP ][ yP ]		(8-13)
					predFlagLX = 1		(8-14)

- Otherwise, the variables refIdxLX and predFlagLX are specified by

refIdxLX = -1		(8-15)
					predFlagLX = 0		(8-16)

- The variable mvdLX is derived as follows.

mvdLX[ 0 ] = mvd_lX[ xP ][ yP ][ 0 ]		(8-17)
				mvdLX[ 1 ] = mvd_lX[ xP ][ yP ][ 1 ]		(8-18)

- When predFlagLX is equal to 1, the variable mvpLX is derived as follows.

- The derivation process for luma motion vector prediction in subclause Error: Reference source not found is invoked with the luma location ( xP, yP ), variables nPSW and nPSH and refIdxLX as the inputs and the output being mvpLX.

- When predFlagLX is equal to 1, the luma motion vector mvLX is derived as

mvLX[ 0 ] = mvpLX[ 0 ] + mvdLX[ 0 ]	(8-19)
				mvLX[ 1 ] = mvpLX[ 1 ] + mvdLX[ 1 ]	(8-20)

When ChromaArrayType is not equal to 0 and predFlagLX (with X being either 0 or 1) is equal to 1, the derivation process for chroma motion vectors in subclause Error: Reference source not found is invoked with mvLX and refIdxLX as inputs and the output being mvCLX.

##### Derivation process for luma motion vectors for merge mode

This process is only invoked when PredMode is equal to MODE_SKIP or PredMode is equal to MODE_ INTER and merge_flag [ xP ][ yP ] is equal to 1, where ( xP, yP ) specify the top-left luma sample of the current prediction unit relative to the top-left luma sample of the current picture.

Inputs of this process are

- a luma location ( xC, yC ) of the top-left luma sample of the current coding unit relative to the top-left luma sample of the current picture,

- a luma location ( xP, yP ) of the top-left luma sample of the current prediction unit relative to the top-left luma sample of the current picture,

- a variable nCS specifying the size of the current coding unit,

- variables specifying the width and the height of the prediction unit for luma, nPSW and nPSH,

- a variable PartIdx specifying the index of the current prediction unit within the current coding unit.

Outputs of this process are

–	the luma motion vectors mvL0 and mvL1,

–	the reference indices refIdxL0 and refIdxL1,

–	the prediction list utilization flags predFlagL0 and predFlagL1.

The motion vectors mvL0 and mvL1, the reference indices refIdxL0 and refIdxL1, and the prediction utilization flags predFlagL0 and predFlagL1 are derived as specified by the following ordered steps:

- The derivation process for merging candidates from neighboring prediction unit partitions in subclause Error: Reference source not found is invoked with luma location ( xP, yP ), the width and the height of the prediction unit nPSW and nPSH and the partition index PartIdx as inputs and the output is assigned to the availability flags availableFlagN, the reference indices refIdxL0N and refIdxL1N, the prediction list utilization flags predFlagL0N and predFlagL1N and the motion vectors mvL0N and mvL1N with N being replaced by A0, A1, B0, B1 or B2.

- The reference index for temporal merging candidate is derived as follows..

- If the prediction unit covering luma location ( xP − 1, yP + nPSH − 1 ) is available and PredMode is not MODE_INTRA, refIdxLX is set equal to refIdxLX[ xP − 1, yP + nPSH − 1 ].

- If the prediction unit covering luma location ( xC − 1, yC + nCS − 1 ) is available and PredMode is not MODE_INTRA, refIdxLX is set equal to refIdxLX[ xC − 1, yC + nCS − 1 ].

- Otherwise, refIdxLX is set equal to 0.

- The derivation process for temporal luma motion vector prediction in subclause Error: Reference source not found is invoked with luma location ( xP, yP ), refIdxLX as the inputs and with the output being the availability flag availableFlagLXCol and the temporal motion vector mvLXCol. The variables availableFlagCol and predFlagLXCol (with X being 0 or 1, respectively) are derived as specified below.

availableFlagCol = availableFlagL0Col || availableFlagL1Col	(8-21)

predFlagLXCol = availableFlagLXCol	(8-22)

- The merging candidate list, mergeCandList, is constructed as follows.

  - A1, if availableFlagA1 is equal to 1

  - B1, if availableFlagB1 is equal to 1

  - B0, if availableFlagB0 is equal to 1

  - A0, if availableFlagA0 is equal to 1

  - B2, if availableFlagB2 is equal to 1

  - Col, if availableFlagCol is equal to 1

- When merging candidates have the same motion vectors and the same reference indices, the merging candidates are removed from the list except the merging candidate which has the smallest order in the mergeCandList.

- The variable numMergeCand and numOrigMergeCand are set to the number of merging candidates in the mergeCandList.

- When slice_type is equal to B, the following applies.

  - The derivation process for combined bi-predictive merging candidates specified in subclause Error: Reference source not found is invoked with mergeCandList, the reference indices refIdxL0N and refIdxL1N, the prediction list utilization flags predFlagL0N and predFlagL1N, the motion vectors mvL0N and mvL1N of every candidate N being in mergeCandList, numMergeCand and numOrigMergeCand given as input and the output is assigned to mergeCandList, numMergeCand, the reference indices refIdxL0combCandk and refIdxL1combCandk, the prediction list utilization flags predFlagL0combCandk and predFlagL1combCandk and the motion vectors mvL0combCandk and mvL1combCandk of every new candidate combCandk being added in mergeCandList. The number of candidates being added numCombMergeCand is set equal to ( numMergeCand – numOrigMergeCand ). When numCombMergeCand is greater than 0, k ranges from 0 to numCombMergeCand − 1, inclusive.

  - The derivation process for non-scaled bi-predictive merging candidates specified in subclause Error: Reference source not found is invoked with mergeCandList, the reference indices refIdxL0N and refIdxL1N, the prediction list utilization flags predFlagL0N and predFlagL1N, the motion vectors mvL0N and mvL1N of every candidate N being in mergeCandList, numMergeCand and numOrigMergeCand given as input and the output is assigned to mergeCandList, numMergeCand, the reference indices refIdxL0nscaleCandl and refIdxL1nscaleCandl, the prediction list utilization flags predFlagL0nscaleCandl and predFlagL1nscaleCandl, the motion vectors mvL0nscaleCandl and mvL1nscaleCandl of every new candidate nscaleCandl being added in mergeCandList. The number of candidates being added numNscaleMergeCand is set equal to ( numMergeCand – numOrigMergeCand – numCombMergeCand ). When numNscaleMergeCand is greater than 0, l ranges from 0 to numNscaleMergeCand – 1, inclusive.

- The derivation process for zero motion vector merging candidates specified in subclause Error: Reference source not found is invoked with the mergeCandList, the reference indices refIdxL0N and refIdxL1N, the prediction list utilization flags predFlagL0N and predFlagL1N, the motion vectors mvL0N and mvL1N of every candidate N being in mergeCandList and the NumMergeCand as the inputs and the output is assigned to mergeCandList, numMergeCand, the reference indices refIdxL0zeroCandm and refIdxL1zeroCandm, the prediction list utilization flags predFlagL0zeroCandm and predFlagL1zeroCandm, the motion vectors mvL0zeroCandm and mvL1zeroCandm of every new candidate zeroCandm being added in mergeCandList. The number of candidates being added numZeroMergeCand is set equal to ( numMergeCand – numOrigMergeCand – numCombMergeCand – numNscaleMergeCand ). When numZeroMergeCand is greater than 0, m ranges from 0 to numZeroMergeCand – 1, inclusive.

- The following assignments are made with N being the candidate at position merge_idx[ xP][ yP ] in the merging candidate list mergeCandList ( N = mergeCandList[ merge_idx[ xP][ yP ] ] ) and X being replaced by 0 or 1:

mvLX[ 0 ] = mvLXN[ 0 ]	(8-23)

mvLX[ 1 ] = mvLXN[ 1 ]	(8-24)

refIdxLX = refIdxLXN	(8-25)

predFlagLX = predFlagLXN	(8-26)