| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 8th Meeting: San José, CA, USA, 1–10 February, 2012 | Document: JCTVC-H0242 |
| --- | --- |

| Title: | DC mode as a default intra mode |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Il-Koo Kim Tammy Lee JaeHyun Kim JeongHoon Park | Email: | ilkoo.kim@samsung.com tammy.lee@samsung.com jhgim@samsung.com jeonghoon@samsung.com |
| Source: | Samsung Electronics Co., Ltd. |  |  |

_____________________________

# Abstract

In this contribution, DC mode is proposed as default intra mode. For this purpose, complexity and hit number between DC and planar mode are analyzed. In HM5.0, default intra mode is assumed as planar mode in intra mode coding process. Previously, in HM4.0, DC mode was default intra mode. According to tests, ratio between number of operations for (DC prediction + DC prediction filtering) and that of planar prediction is approximately 13.3. Hit rate of planar mode becomes higher when default intra mode is set as planar mode rather than DC mode. In similar fashion, hit number of DC mode becomes higher when default intra mode is set as DC mode rather than planar mode. Combined with both results, it could be concluded that the complexity of default DC mode case is lower than that of default planar mode case since DC mode has higher number of hit with lower complexity than planar mode. Average coding efficiency changes from this modification for each configuration are 0.0% (AI-HE), 0.0% (AI-LC), 0.0% (RA-HE), 0.0% (RA-LC), 0.0% (LD-HE), 0.0% (LD-LC) and 0.1% (RA-HE-10bit). Coding efficiency gain for Class F is -0.1%. Encoding and decoding time are 100% and 99%, respectively. It is noted that default DC mode is more efficient than default planar mode, especially for class F since DC prediction is better than planar prediction to represent uniform region for computer-generated video, such as screen contents. Based on the results, it is proposed to use DC mode as default intra mode in HM6.0.

# Introduction

In HM5.0, if neighbouring PU of current PU is not available or is not intra PU or located in above LCU, intra mode of the PU is assumed as planar mode in intra mode coding process. Assumed mode is considered as “default” intra mode. Previously, in HM4.0, DC mode was default intra mode. In this contribution, complexity difference when DC or planar mode is set as default intra mode is analyzed, and based on the results, DC mode is re-suggested as default intra mode as a conclusion.

# Default intra mode: DC vs. planar

## Number of operation analysis

It is known that DC and planar mode is useful in plane area. Especially, planar mode is known to be effective in the area having gradation in it. Strong point of planar mode is also claimed in subjective quality improvement. However, complexity of planar mode is higher than DC mode. When an intra mode of PU is DC mode, DC prediction operation consists of performing prediction by copying the average value of neighbouring pixels and filtering the part of it. Strong point of DC mode is coming from its simplicity. Table 1 summarizes the number of operations needed for DC and planar mode for 32x32 prediction unit (PU). It was counted based on actual implementation in HM5.0 reference software [1]. Total number of operations needed for DC mode is the sum of numbers required to perform both DC prediction and DC prediction filtering. According to the table, ratio between number of operations for (DC prediction + DC prediction filtering) and that of planar prediction is approximately 13.3.

Table 1. Number of operations needed for DC mode and planar mode: 32x32 PU case

| 32x32 PU | multiplication | division | add or subtract | shift | sum |
| --- | --- | --- | --- | --- | --- |
| DC prediction | 0 | 1 | 66 | 0 | 67 |
| DC pred. filtering | 63 | 0 | 127 | 63 | 253 |
| Planar prediction | 0 | 0 | 3168 | 1088 | 4256 |

Figure 1 illustrates ratios between the number of operations needed to perform DC and planar prediction for each PU size. As you can see, the ratio increases as size of PU increases. The range is between 2.1 and 13.3.

Figure 1. Ratio between the number of operations needed to perform DC and planar prediction

## Hit ratio analysis

If the default intra mode is changed from DC mode to planar mode, or vice versa, number of utilization of planar and DC mode is also changed. In this section, number of hit of DC and planar mode are provided when the default intra mode is DC or planar mode, respectively. Table 2 summarizes number of hit of DC and planar mode when default intra mode is set as planar or DC. To get these statistics, HM5.0 [1] was modified and tested under common test conditions [2]. The results contain counts from Class F sequences.

Table 2. Number of hit of DC and planar mode when the default intra mode is DC or planar mode

|  | Default intra mode : planar |  |  |  |  |  | Default intra mode : DC |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | DC |  | planar |  | Ratio (DC/planar) |  | DC |  | planar |  | Ratio (DC/planar) |
| AI-HE | 106319002 |  | 108974895 |  | 0.98 |  | 125161867 |  | 96244063 |  | 1.30 |  |
| AI-LC | 146598332 |  | 120048850 |  | 1.22 |  | 165758944 |  | 106774352 |  | 1.55 |  |
| RA-HE | 7144617 |  | 7692199 |  | 0.93 |  | 8642367 |  | 6233130 |  | 1.39 |  |
| RA-LC | 8543907 |  | 8080440 |  | 1.06 |  | 10001637 |  | 6609510 |  | 1.51 |  |
| LD-HE | 2181866 |  | 2964875 |  | 0.74 |  | 2930284 |  | 2175855 |  | 1.35 |  |
| LD-LC | 3056830 |  | 3598524 |  | 0.85 |  | 3905565 |  | 2671018 |  | 1.46 |  |
| RA-HE-10 | 5764466 |  | 6472374 |  | 0.89 |  | 7129560 |  | 5160053 |  | 1.38 |  |

Hit number of planar mode becomes higher when default intra mode is set as planar mode rather than DC mode. In similar fashion, hit number of DC mode becomes higher when default intra mode is set as DC mode rather than planar mode. Combined with results in section 2.1, it could be concluded that the complexity of default DC mode case is lower than that of default planar mode case since DC mode has higher number of hit with lower complexity than planar mode.

# Experimental results

An experiment was conducted under common test condition [2]. HM5.0 reference software was used as base software [1]. Table 3 summarizes BD-Rate gain from this experiment. Average coding efficiency changes from this modification for each configuration are 0.0% (AI-HE), 0.0% (AI-LC), 0.0% (RA-HE), 0.0% (RA-LC), 0.0% (LD-HE), 0.0% (LD-LC) and 0.1% (RA-HE-10bit). Coding efficiency gain for Class F is -0.1%. Encoding and decoding time are 100% and 99%, respectively. It is noted that default DC mode is more efficient than default planar mode, especially for class F since DC prediction is better than planar prediction to represent uniform region for computer-generated video, such as screen contents.

Table 3. Result of setting DC mode as a default intra mode

# Conclusions

In this contribution, complexity difference between DC and planar mode was analyzed and experimental result was provided. Average coding efficiency changes from this modification for each configuration are 0.0% with 100% and 99% encoding and decoding time, respectively. Coding efficiency gain for Class F is -0.1%. It is noted that default DC mode is more efficient than default planar mode, especially for class F since DC prediction is better than planar prediction to represent uniform region for artificial video, such as screen contents. Based on the results, it is proposed to use DC mode as default intra mode in HM6.0.

# Suggested Working Draft (WD) modification

Modified texts are marked as green color based on JCTVC-G1103_d6.doc [3].

### Derivation process for luma intra prediction mode

Inputs to this process are:

–	a luma location ( xB, yB ) specifying the top-left luma sample of the current block relative to the top-left luma sample of the current picture,

–	a variable log2PUSize specifying the size of the current prediction unit,

–	variable arrays IntraPredMode (If available) that are previously (in decoding order) derived for adjacent coding units.

Output of this process is the variable IntraPredMode[ xB ][ yB ].

Table 8-4 specifies the value for the intra prediction mode and the associated names.

Table 8-4 – Specification of intra prediction mode and associated names

| Intra prediction mode | Associated names |
| --- | --- |
| 0 | Intra_Planar |
| 1 | Intra_Vertical |
| 2 | Intra_Horizontal |
| 3 | Intra_DC |
| Otherwise (4..34) | Intra_Angular |
| 35 | Intra_FromLuma (used only for chroma) |

Table 8-5 specifies the number of luma intra prediction modes intraPredModeNum depending on log2PUSize.

Table 8-5 – Specification of intraPredModeNum

| log2PUSize | intraPredModeNum |
| --- | --- |
| 2 | 18 |
| 3 | 35 |
| 4 | 35 |
| 5 | 35 |
| 6 | 35 |

Error: Reference source not found specifies the mapping table used for converting the number of intra prediction modes.

IntraPredMode[ xB ][ yB ] labelled 0, 1, 2, .., 34 represents directions of predictions as illustrated in Figure 8-2.

Figure 8-2 – Intra prediction mode directions (informative)

IntraPredMode[ xB ][ yB ] is derived as the following ordered steps. [Ed. (WJ): proponent suggests to move this part to the syntax since the other syntax elements utilize IntraPredMode. But it seems too complex to move all the following process to the syntax table. Maybe it’s better to move this part to the semantics section or simply avoid the use of IntraPredMode to parse the syntax item]

- The derivation process for neighbouring treeblocks specified in subclause XXX with ( xB,  yB ) given as input and the output is assigned to tbAddrA and tbAddrB specifying the treeblock addresses of treeblocks covering ( xBA,  yBA ) and ( xBB, yBB ) respectively where ( xBA,  yBA ) is set equal to ( xB-1,  yB ) and ( xBB,  yBB ) is set equal to ( xB,  yB-1 ).

- For N being either replaced A or B, the variables intraPredModeN are derived as follows.

- If the treeblock with address tbAddrN is not available, intraPredModeN is set equal to Intra_DC.

- Otherwise, if the coding unit covering ( xBN,  yBN ) is not coded as intra mode, intraPredModeN is set equal to Intra_DC,

- Otherwise, if yB-1 is smaller than YLCU, intraPredModeA is set equal to IntraPredMode[ xBA ][ yBA ] and intraPredModeB is set equal to Intra_DC.

- Otherwise, intraPredModeN is set equal to IntraPredMode[ xBN ][ yBN ], where IntraPredMode is the variable array assigned to the coding unit covering the luma location ( xBN, yBN ).

- For N being either replaced A or B, the variables candIntraPredModeN are derived as follows.

- If intraPredModeN is greater than or equal to intraPredModeNum, candIntraPredModeN is set equal to Intra_Planar.

- Otherwise, candIntraPredModeN is set equal to intraPredModeN

- If candIntraPredModeA is equal to candIntraPredModeB, the candIntraPredModeA is modified as follows:

- If candIntraPredModeA is not equal to Intra_Planar, candIntraPredModeA is set equal to Intra_Planar

- Otherwise, candIntraPredModeA is set equal to Intra_DC

- The candModeList[x] is derived as follows:

candModeList[0] = Min( candIntraPredModeA, candIntraPredModeB )	(8-1)
candModeList[1] = Max( candIntraPredModeA, candIntraPredModeB )	(8-2)

- IntraPredMode[ xB ][ yB ] is derived by applying the following procedure:

- If prev_intra_pred_flag[ xB ][ yB ] is true, the IntraPredMode[ xB ][ yB ] is set equal to  candModeList[ mpm_flag ][ xB ][ yB ]]

- Otherwise IntraPredMode[ xB ][ yB ] is derived by applying the following ordered steps:

  - IntraPredMode[ xB ][ yB ] = rem_intra_luma_pred_mode

  - When IntraPredMode[ xB ][ yB ] is equal or greater than candModeList[ 0 ], the value of IntraPredMode[ xB ][ yB ] is increased by one

  - When IntraPredMode[ xB ][ yB ] is equal or greater than candModeList[ 1 ], the value of IntraPredMode[ xB ][ yB ] is increased by one

# References

- https://hevc.hhi.fraunhofer.de/svn/svn_HEVCSoftware/tags/HM-5.0/

- F. Bossen, “Common test conditions and software reference configurations,” Document of Joint Collaborative Team on Video Coding, JCTVC-G1200, Nov. 2011.

- B. Bross, W.-J. Han, G. J. Sullivan, J.-R. Ohm and T. Wiegand, “High Efficiency Video Coding (HEVC) text specification Working Draft 5,” Document of Joint Collaborative Team on Video Coding, JCTVC-G1103, Nov. 2011.

# Patent rights declaration(s)

Samsung Electronics Co., Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).