| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 8th Meeting: San José, CA, USA, 1–10 February, 2012 | Document: JCTVC-H0243 |
| --- | --- |

| Title: | Simplification of spatial motion vector scaling process |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Il-Koo Kim JeongHoon Park | Email: | ilkoo.kim@samsung.com jeonghoon@samsung.com |
| Source: | Samsung Electronics Co., Ltd. |  |  |

_____________________________

# Abstract

In HM5.0, spatial scaling for above motion vector is allowed only when all PUs of left candidates is not available or intra coded. In this contribution, as a simplification, scaling for above candidate is completely removed to reduce complexity further. Average coding efficiency changes from this modification for each configuration are 0.0% (RA-HE), 0.1% (RA-LC), 0.0% (LD-HE), 0.0% (LD-LC) and 0.1% (RA-HE-10bit). Encoding and decoding time are within the range between 99% and 100%, approximately. As a conclusion, it is proposed to adopt this simplification in the forthcoming HM6.0.

# Introduction

Figure 1. Candidate positions for spatial motion vector candidate

In motion vector prediction of HM5.0, 2 types of motion vector candidates are considered: spatial motion vector candidate and temporal motion vector candidate. In this contribution, spatial motion vector candidate is our concern. In derivation process of spatial motion vector candidate, 2 motion vector candidates are derived based on motion vectors of each PU located in 5 different positions as depicted in Figure 1. In the process of derivation, 1 motion vector candidate is selected utilizing PUs in the left side (A0, A1) of current PU and 1 motion vector candidate is derived utilizing PUs in the above side (B0, B1, B2) of current PU. Figure 2 shows flow diagram of the derivation process.

Figure 2. Derivation process of spatial motion vector candidate (temporal motion vector candidate is omitted for simplicity of presentation)

More specifically, the order of derivation for left side of current PU is set as A0 à A1 à scaled A0 à scaled A1. The order of derivation for above side of current PU is set as B0 à B1 à B2 à scaled B0 à scaled B1 à scaled B2. Scaled An and scaled Bn mean scaled motion vector of PU located in position An or Bn, n is being 0 or 1 for An, and being 0 or 1 or 2 for Bn. For each step, motion vector candidate is checked. If no motion vector is available, then proceed to next step. Otherwise, corresponding motion vector is put into motion vector candidate list. Scaled An and Bn are considered when POC is different between reference picture of neighbouring PU and that of current PU regardless of reference picture list. In general, spatial scaling is not allowed for above motion vector. As a special case, only when all PUs of left candidates is not available or intra coded, scaling for above motion vector is allowed.

# Simplification of spatial motion vector scaling process

As described in above section, spatial scaling for above candidates is allowed only when PUs located in position A0 and A1 are not available or intra coded. However, despite the condition is not occurred frequently, it should be checked for every spatial motion vector candidate derivation. In this contribution, regardless of the condition, it is proposed not to perform spatial scaling process for above motion vector candidate derivation. In other words, spatial scaling process is completely removed from the derivation process of above motion vector candidate and only applied to left motion vector candidates. Figure 3 illustrates the proposed derivation process of spatial motion vector candidate.

Figure 3. Simplified derivation process of spatial motion vector candidate

# Experimental results

An experiment was conducted under common test condition [2]. HM5.0 reference software was used as base software [1]. Table 1 summarizes BD-Rate gain from this experiment. Average coding efficiency changes from this modification for each configuration are 0.0% (RA-HE), 0.1% (RA-LC), 0.0% (LD-HE), 0.0% (LD-LC) and 0.1% (RA-HE-10bit). Encoding and decoding time are within the range between 99% and 100%, approximately.

Table 1. Summarized result of simplification of spatial motion vector scaling process

# Conclusions

In this contribution, regardless of the condition, it has been proposed not to perform spatial scaling process for above motion vector candidate derivation. In other words, spatial scaling process is completely removed from the derivation process of above motion vector candidate. An experiment was conducted under common test condition. Average coding efficiency changes from this modification for each configuration are 0.0% (RA-HE), 0.1% (RA-LC), 0.0% (LD-HE), 0.0% (LD-LC) and 0.1% (RA-HE-10bit). Encoding and decoding time are within the range between 99% and 100%, approximately. As a conclusion, it is proposed to adopt this simplification in the forthcoming HM6.0.

# Suggested Working Draft (WD) modification

Modified texts are marked as green color based on JCTVC-G1103_d6.doc [3].

##### Derivation process for motion vector predictor candidates

Inputs to this process are

- a luma location ( xP, yP ) specifying the top-left luma sample of the current prediction unit relative to the top-left sample of the current picture,

- variables specifying the width and the height of the prediction unit for luma, nPSW and nPSH,

- the reference index of the current prediction unit partition refIdxLX (with X being 0 or 1).

Outputs of this process are (with N being replaced by A, or B)

- the motion vectors mvLXN of the neighbouring prediction units,

- the availability flags availableFlagLXN of the neighbouring prediction units.

Figure 8-4 – Spatial motion vector neighbours

The variable isScaledFlagLX with X being 0 or 1 is set equal to 0.

The motion vector mvLXA and the availability flag availableFlagLXA are derived in the following ordered steps:

- Let a set of two sample locations be (xAk, yAk), with k = 0, 1, specifies sample locations with xAk = xP – 1, yA0 = yP + nPSH and yA1 = yA0 - MinPuSize. The set of sample locations ( xAk, yAk ) represent the sample locations immediately to the left side of the left partition boundary and it’s extended line.

- Let the availability flag availableFlagLXA be initially set equal to 0 and the both components of mvLXA are set equal to 0.

- If one of the following conditions is true, the variable isScaledFlagLX is set equal to 1.

- the prediction unit covering luma location ( xA0, yA0 ) is available and PredMode is not MODE_INTRA.

- the prediction unit covering luma location ( xA1, yA1 ) is available and PredMode is not MODE_INTRA.

- For ( xAk, yAk ) from ( xA0, yA0 ) to ( xA1, yA1 ) where yA1 = yA0 - MinPuSize, the following applies repeatedly until availableFlagLXA is equal to 1:

- If the prediction unit covering luma location ( xAk, yAk ) is available, PredMode is not MODE_INTRA, predFlagLX[ xAk ][ yAk ] is equal to 1 and the reference index refIdxLX[ xAk ][ yAk ] is equal to the reference index of the current prediction unit refIdxLX, availableFlagLXA is set equal to 1 and the motion vector mvLXA is set equal to the motion vector mvLX[ xAk ][ yAk ], refIdxA is set equal to refIdxLX[ xAk ][ yAk ] and ListA is set equal to LX.

- Otherwise if the prediction unit covering luma location ( xAk, yAk ) is available, PredMode is not MODE_INTRA, predFlagLY[ xAk ][ yAk ] (with Y = !X) is equal to 1 and PicOrderCnt( RefPicListLY( refIdxLY[ xAk ][ yAk ] ) ) is equal to PicOrderCnt( RefPicListLX( refIdxLX ) ), availableFlagLXA is set equal to 1, the motion vector mvLXA is set equal to the motion vector mvLY[ xAk ][ yAk ], refIdxA is set equal to refIdxLY[ xAk ][ yAk ] ,  ListA is set equal to LY and mvLXA is set equal to mvLXA.

- When availableFlagLXA is equal to 0, for ( xAk, yAk ) from ( xA0, yA0 ) to ( xA1, yA1 ) where yA1 = yA0 - MinPuSize, the following applies repeatedly until availableFlagLXA is equal to 1:

- If the prediction unit covering luma location ( xAk, yAk ) is available, PredMode is not MODE_INTRA, predFlagLX[ xAk ][ yAk ] is equal to 1, availableFlagLXA is set equal to 1, the motion vector mvLXA is set equal to the motion vector mvLX[ xAk ][ yAk ], refIdxA is set equal to refIdxLX[ xAk ][ yAk ], ListA is set equal to LX.

- Otherwise if the prediction unit covering luma location ( xAk, yAk ) is available, PredMode is not MODE_INTRA, predFlagLY[ xAk ][ yAk ] (with Y = !X) is equal to 1, availableFlagLXA is set equal to 1, the motion vector mvLXA is set equal to the motion vector mvLY[ xAk ][ yAk ], refIdxA is set equal to refIdxLY[ xAk ][ yAk ], ListA is set equal to LY.

- When availableFlagLXA is equal to 1, mvLXA is derived as specified below.

tx = ( 16384 + ( Abs( td ) >> 1 ) ) / td	(8-1)

DistScaleFactor = Clip3( −4096, 4095, ( tb * tx + 32 ) >> 6 )	(8-2)

mvLXA = Sign( DistScaleFactor * mvLXA ) * 			
		( (Abs( DistScaleFactor * mvLXA ) + 127 ) >> 8 )	(8-3)

where td and tb are derived as

td = Clip3( -128, 127, PicOrderCntVal – PicOrderCnt( RefPicListListA( refIdxA ) ) )	(8-4)

tb = Clip3( -128, 127, PicOrderCntVal – PicOrderCnt( RefPicListLX( refIdxLX ) ) )	(8-5)

The motion vector mvLXB and the availability flag availableFlagLXB are derived in the following ordered steps:

- Let a set of three sample location (xBk, yBk), with k = 0,1,2, specifies sample locations with xB0 = xP + nPSW, xB1 = xB0 MinPuSize , xB2 = xP - MinPuSize and yBk = yP – 1. The set of sample locations ( xBk, yBk ) represent the sample locations immediately to the upper side of the above partition boundary and it’s extended line. [Ed.: (WJ) MinPuSize should be defined somewhere]

- If yP-1 is smaller than YLCU, the following applies.

xB0 = (xB0>>Log2MinCUSize)<<Log2MinCUSize) + ((xB0>>Log2MinCUSize)&1)*(MinPuSize*2-1)	(8-6)
xB1 = (xB1>>Log2MinCUSize)<<Log2MinCUSize) + ((xB1>>Log2MinCUSize)&1)*(MinPuSize*2-1)	(8-7)
xB2 = (xB2>>Log2MinCUSize)<<Log2MinCUSize) + ((xB2>>Log2MinCUSize)&1)*(MinPuSize*2-1)	(8-8)

where MinPuSize = 1 << ( Log2MinCUSize – 1)

- Let the availability flag availableFlagLXB be initially set equal to 0 and the both components of mvLXB are set equal to 0.

- For ( xBk, yBk ) from ( xB0, yB0 ) to ( xB2, yB2 ) where xB0 = xP +nPSW, xB1 = xB0 - MinPuSize , and xB2 =  xP - MinPuSize, the following applies repeatedly until availableFlagLXB is equal to 1:

- If the prediction unit covering luma location ( xBk, yBk ) is available, PredMode is not MODE_INTRA, predFlagLX[ xBk ][ yBk ] is equal to 1, and the reference index refIdxLX[ xBk ][ yBk ] is equal to the reference index of the current prediction unit refIdxLX, availableFlagLXB is set equal to 1 and the motion vector mvLXB is set equal to the motion vector mvLX[ xBk ][ yBk ], refIdxB is set equal to refIdxLX[ xBk ][ yBk ] and ListB is set equal to LX.

- Otherwise if the prediction unit covering luma location ( xBk, yBk ) is available, PredMode is not MODE_INTRA, predFlagLY[ xBk ][ yBk ] (with Y = !X) is equal to 1 and PicOrderCnt( RefPicListLY( refIdxLY[ xBk ][ yBk ] ) ) is equal to PicOrderCnt( RefPicListLX( refIdxLX ) ), availableFlagLXB is set equal to 1, the motion vector mvLXB is set equal to the motion vector mvLY[ xBk ][ yBk ],  refIdxB is set equal to refIdxLY[ xBk ][ yBk ],  and ListB is set equal to LY.

- When isScaledFlagLX is equal to 0, availableFlagLXB is set equal to 0 and for ( xBk, yBk ) from ( xB0, yB0 ) to ( xB2, yB2 ) where xB0 = xP +nPSW, xB1 = xB0 - MinPuSize , and xB2 =  xP - MinPuSize, the following applies repeatedly until availableFlagLXB is equal to 1:

- If the prediction unit covering luma location ( xBk, yBk ) is available, PredMode is not MODE_INTRA, predFlagLX[ xBk ][ yBk ] is equal to 1, availableFlagLXB is set equal to 1, the motion vector mvLXB is set equal to the motion vector mvLX[ xBk ][ yBk ], refIdxB is set equal to refIdxLX[ xBk ][ yBk ], ListB is set equal to LX.

- Otherwise if the prediction unit covering luma location ( xBk, yBk ) is available, PredMode is not MODE_INTRA, predFlagLY[ xBk ][ yBk ] (with Y = !X) is equal to 1, availableFlagLXB is set equal to 1, the motion vector mvLXB is set equal to the motion vector mvLY[ xBk ][ yBk ], refIdxB is set equal to refIdxLY[ xBk ][ yBk ], ListB is set equal to LY.

- When availableFlagLXB is equal to 1 and PicOrderCnt( RefPicListListB( refIdxB ) ) is not equal to PicOrderCnt( RefPicListLX( refIdxLX ) ), mvLXB is derived as specified below.

tx = ( 16384 + ( Abs( td ) >> 1 ) ) / td	(8-9)

DistScaleFactor = Clip3( −4096, 4095, ( tb * tx + 32 ) >> 6 )	(8-10)

mvLXB = Sign( DistScaleFactor * mvLXA ) * 			
		( (Abs( DistScaleFactor * mvLXA ) + 127 ) >> 8 )	(8-11)

where td and tb are derived as

td = Clip3( -128, 127, PicOrderCntVal – PicOrderCnt( RefPicListListB( refIdxB ) ) )	(8-12)

tb = Clip3( -128, 127, PicOrderCntVal – PicOrderCnt( RefPicListLX( refIdxLX ) ) )	(8-13)

# References

- https://hevc.hhi.fraunhofer.de/svn/svn_HEVCSoftware/tags/HM-5.0/

- F. Bossen, “Common test conditions and software reference configurations,” Document of Joint Collaborative Team on Video Coding, JCTVC-G1200, Nov. 2011.

- B. Bross, W.-J. Han, G. J. Sullivan, J.-R. Ohm and T. Wiegand, “High Efficiency Video Coding (HEVC) text specification Working Draft 5,” Document of Joint Collaborative Team on Video Coding, JCTVC-G1103, Nov. 2011.

# Patent rights declaration(s)

Samsung Electronics Co., Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).