| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 8th Meeting: San José, CA, USA, 1–10 February, 2012 | Document: JCTVC-H0244 |
| --- | --- |

| Title: | Intra-mode bypass parallelism (IMBP) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | C. Rosewarne 1 Thomas Holt Drive, North Ryde, NSW 2113, Australia. | Tel: Email: | +61 2 9805 2844 chris.rosewarne@cisra.canon.com.au |
|  | M. Maeda 30-2, Shimomaruko 3-chome, Ohta-ku, Tokyo 146-8501, Japan. | Tel: Email: | +81-3-3758-2111 maeda.mitsuru@canon.co.jp |
| Source: | Canon Information Systems Research Australia (CiSRA), Canon Inc. |  |  |

_____________________________

# Abstract

This contribution presents a method of encoding CU data for intra mode. The method tries to improve CU decoding performance by grouping data into chunks to achieve higher processing performance and to allow parallel processing with other CU data. Simulation results reportedly show no coding efficiency loss. This modification was implemented on top of HM-5.0 and resulted in degradations of 0.0% for IA_HE, 0.0% for RA_HE 0.0% for LB_HE 0.0% for LP_HE configurations in the Luma channel (relative to HM-5.0).

# Introduction

In the HM-5.0 intra mode CU data is generally encoded as a number of arithmetically-coded and bypass-coded bins. It is known that bypass-coded bins can be processed with higher throughput than arithmetically coded bins. It is also known that bypass bin processing performance can be improved for the case of multiple consecutive bins.

This contribution proposes a way to encode intra mode CU data to group bypass-coded bins so that it will be possible to

- Process the grouped bypass bins on encoder and decoder side as a single chunk

- Decode the grouped bypass bins in parallel to arithmetically coded bins

# Description of the proposed change

## Current implementation (HM-5.0)

The HM-5.0 intra mode CU data in relevance to this proposal can be described as consisting of the following components:

- Arithmetically coded quad-tree structural data: split flags, etc

- Arithmetically coded luma intra prediction data: prev_intra_luma_pred_flag

- Bypass coded luma intra prediction data: mpm_idx, rem_intra_luma_pred_mode

- Other CU data

Figure 2.1 shows the structure of the encoded stream for an intra-mode LCU with several CUs in it.

Figure 2.1

The data in the blue blocks on Figure 2.1 encoded with bypass bins. The rest of the data uses either arithmetical or bypass coding.

## Proposed stream organization

It is proposed that for the intra mode LCUs the bypass coded luma prediction data is grouped over all CUs of an LCU. This is depicted on Figure 2.2:

Figure 2.2

In the proposed method for the intra mode LCUs the structural data for the entire LCU is encoded first together with the prev_intra_luma_pred_flag binary flag (green blocks on Figure 2.2). Next the luma intra prediction data (mpm_idx and rem_intra_luma_pred_mode) is encoded (blue blocks on Figure 2.2). This data uses bypass coding only thus it is stated that it can be decoded very quickly when grouped as proposed. It is also stated that the decoding of the other data (yellow blocks on Figure 2.2) can be started before the processing of bypass coded luma prediction data has finished thus achieving parallelization of decoding.

# Experiment Results

## Bitstream statistics

According to the statistics gathered over four exemplary streams encoded in the AI_HE mode, QP=37, using base HM-5.0 software and IMBP:

- Approximately 25% of all encoded bins in a stream are EP-coded bins;

- Approximately 40% of all EP-coded bins (or about 10,5% of all stream bins) can be optimized with IMBP;

- IMBP can group relevant EP-bins into chunks reducing the total number of EP-chunks in the stream by 48%;

- After IMBP grouping the average length of relevant EP-chunks increases from 2,5 bins per chunk to 151,6 bin per chunk;

Figure 3.1

Figure 3.1 is built based on the data from imbp_stats.xls. The figure shows the total count of contiguous chunks of EP-coded bins for four exemplary streams (AI-HE, QP=37) measured for HM-5.0 anchor stream and IMBP-optimized stream. The red bar indicates the intra mode (IM) chunks which are the subject for IMBP optimization. The blue bar indicates all other chunks of EP-coded bins. From the presented statistics it follows that the IMBP optimizaion reduces the total number of EP-chunks in a stream by approximatey 50%.

## Standard simulations

Results presented in Table 3.1 below refer to the accompanying spreadsheet for results with greater precision. Execution took place on a Linux cluster using GCC 3.4.6.

Table 3.1. Simulation results for IMBP

|  | All Intra HE |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Class A | 0.0% | -0.1% | 0.0% |
| Class B | 0.0% | 0.0% | 0.0% |
| Class C | 0.0% | 0.0% | 0.0% |
| Class D | 0.0% | 0.0% | 0.0% |
| Class E | 0.0% | -0.1% | 0.0% |
| Class F | 0.0% | 0.0% | -0.1% |
| Overall | 0.0% | 0.0% | 0.0% |
|  | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 102% |  |  |
| Dec Time[%] | 99% |  |  |
|  | Random Access HE |  |  |
|  | Y | U | V |
| Class A | 0.0% | -0.2% | -0.2% |
| Class B | 0.0% | 0.0% | -0.1% |
| Class C | 0.0% | -0.1% | 0.0% |
| Class D | 0.0% | -0.1% | 0.1% |
| Class E |  |  |  |
| Class F | 0.0% | 0.0% | -0.1% |
| Overall | 0.0% | -0.1% | 0.0% |
|  | 0.0% | -0.1% | 0.0% |
| Enc Time[%] | 101% |  |  |
| Dec Time[%] | 100% |  |  |
|  | Low delay B HE |  |  |
|  | Y | U | V |
| Class A |  |  |  |
| Class B | 0.0% | 0.0% | 0.0% |
| Class C | 0.0% | 0.0% | 0.0% |
| Class D | 0.0% | 0.0% | 0.0% |
| Class E | 0.0% | 0.0% | 0.0% |
| Class F | 0.0% | 0.0% | 0.0% |
| Overall | 0.0% | 0.0% | 0.0% |
|  | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 101% |  |  |
| Dec Time[%] | 100% |  |  |
|  | Low delay P HE |  |  |
|  | Y | U | V |
| Class A |  |  |  |
| Class B | 0.0% | 0.0% | 0.0% |
| Class C | 0.0% | 0.0% | 0.0% |
| Class D | 0.0% | 0.0% | 0.0% |
| Class E | 0.0% | 0.0% | 0.0% |
| Class F | 0.0% | 0.0% | 0.0% |
| Overall | 0.0% | 0.0% | 0.0% |
|  | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 101% |  |  |
| Dec Time[%] | 100% |  |  |

# Working Draft changes

## Proposed Working Draft Changes

Proposed working draft additions are marked green.

### 7.3.5 Coding tree syntax

| coding_tree( x0, y0, log2CUSize, cuDepth ) { | Descriptor |
| --- | --- |
| if (slice_type == I && SliceGranularity == 0) { |  |
| if (cuDepth == 0) { |  |
| coding_tree_split_and_mpm_flags(x0, y0, log2CUSize) |  |
| coding_tree_luma_intra_mode(x0, y0, log2CUSize) |  |
| } |  |
| } else { |  |
| if( x0 + ( 1 << log2CUSize ) <= PicWidthInSamplesL && y0 + ( 1 << log2CUSize ) <= PicHeightInSamplesL && cuAddress( x0, y0 ) >= SliceAddress && log2CUSize > Log2MinCUSize ) { |  |
| split_coding_unit_flag[ x0 ][ y0 ] | ae(v) |
| } |  |
| } |  |
| if( adaptive_loop_filter_flag && alf_cu_control_flag ) { |  |
| if( cuDepth <= alf_cu_control_max_depth ) |  |
| if( cuDepth == alf_cu_control_max_depth ¦¦ |  |
| split_coding_unit_flag[ x0 ][ y0 ] == 0 ) |  |
| AlfCuFlagIdx++ |  |
| } |  |
| if( cu_qp_delta_enabled_flag && log2CUSize >= log2MinCUDQPSize ) |  |
| IsCuQpDeltaCoded = 0 |  |
| if( split_coding_unit_flag[ x0 ][ y0 ] ) { |  |
| x1 = x0 + ( ( 1 << log2CUSize ) >> 1 ) |  |
| y1 = y0 + ( ( 1 << log2CUSize ) >> 1 ) |  |
| if( cuAddress( x1, y0 ) > SliceAddress ) |  |
| moreDataFlag = coding_tree( x0, y0, log2CUSize – 1, cuDepth + 1 ) |  |
| if( cuAddress( x0, y1 ) > SliceAddress && moreDataFlag && x1 < PicWidthInSamplesL ) |  |
| moreDataFlag = coding_tree( x1, y0, log2CUSize − 1, cuDepth + 1 ) |  |
| if( cuAddress( x1, y1 ) > SliceAddress && moreDataFlag && y1 < PicHeightInSamplesL ) |  |
| moreDataFlag = coding_tree( x0, y1, log2CUSize − 1, cuDepth + 1 ) |  |
| if( moreDataFlag && x1 < PicWidthInSamplesL && y1 < PicHeightInSamplesL ) |  |
| moreDataFlag = coding_tree( x1, y1, log2CUSize − 1, cuDepth + 1 ) |  |
| } else { |  |
| if(adaptive_loop_filter_flag && alf_cu_control_flag ) |  |
| AlfCuFlag[ x0 ][ y0 ] = alf_cu_flag[ AlfCuFlagIdx ] |  |
| coding_unit( x0, y0, log2CUSize ) |  |
| if( (slice_type != I ¦¦ SliceGranularity != 0) && granularity_block_boundary( x0, y0, log2CUSize ) ) { |  |
| end_of_slice_flag | ae(v) |
| moreDataFlag = !end_of_slice_flag |  |
| } else |  |
| moreDataFlag = 1 |  |
| } |  |
| return moreDataFlag |  |
| } |  |

#### 7.3.5.1 Coding tree syntax for split and MPM flags

| coding_tree_split_and_mpm_flags( x0, y0, log2CUSize ) { | Descriptor |
| --- | --- |
| if( x0 + ( 1 << log2CUSize ) <= PicWidthInSamplesL && y0 + ( 1 << log2CUSize ) <= PicHeightInSamplesL && cuAddress( x0, y0 ) >= SliceAddress && log2CUSize > Log2MinCUSize ) { |  |
| split_coding_unit_flag[ x0 ][ y0 ] | ae(v) |
| } |  |
| if( split_coding_unit_flag[ x0 ][ y0 ] ) { |  |
| x1 = x0 + ( ( 1 << log2CUSize ) >> 1 ) |  |
| y1 = y0 + ( ( 1 << log2CUSize ) >> 1 ) |  |
| if( cuAddress( x1, y0 ) > SliceAddress ) |  |
| coding_tree_split_and_mpm_flags ( x0, y0, log2CUSize – 1 ) |  |
| if( cuAddress( x0, y1 ) > SliceAddress && x1 < PicWidthInSamplesL ) |  |
| coding_tree_split_and_mpm_flags ( x1, y0, log2CUSize − 1 ) |  |
| if( cuAddress( x1, y1 ) > SliceAddress && y1 < PicHeightInSamplesL ) |  |
| coding_tree_split_and_mpm_flags ( x0, y1, log2CUSize − 1 ) |  |
| if( x1 < PicWidthInSamplesL && y1 < PicHeightInSamplesL ) |  |
| coding_tree_split_and_mpm_flags ( x1, y1, log2CUSize − 1 ) |  |
| } else { |  |
| coding_unit_part_mode_and_mpm_flag ( x0, y0, log2CUSize ) |  |
| } |  |
| } |  |

7.3.5.2 Coding tree syntax for luma intra mode

| coding_tree_luma_intra_mode( x0, y0, log2CUSize ) { | Descriptor |
| --- | --- |
| if( split_coding_unit_flag[ x0 ][ y0 ] ) { |  |
| x1 = x0 + ( ( 1 << log2CUSize ) >> 1 ) |  |
| y1 = y0 + ( ( 1 << log2CUSize ) >> 1 ) |  |
| if( cuAddress( x1, y0 ) > SliceAddress ) |  |
| coding_tree_luma_intra_mode( x0, y0, log2CUSize – 1 ) |  |
| if( cuAddress( x0, y1 ) > SliceAddress && x1 < PicWidthInSamplesL ) |  |
| coding_tree_luma_intra_mode ( x1, y0, log2CUSize − 1 ) |  |
| if( cuAddress( x1, y1 ) > SliceAddress && y1 < PicHeightInSamplesL ) |  |
| coding_tree_luma_intra_mode ( x0, y1, log2CUSize − 1 ) |  |
| if( x1 < PicWidthInSamplesL && y1 < PicHeightInSamplesL ) |  |
| coding_tree_luma_intra_mode ( x1, y1, log2CUSize − 1 ) |  |
| } else { |  |
| coding_unit_luma_intra_mode( x0, y0, log2CUSize ) |  |
| } |  |
| } |  |

### 7.3.6 Coding unit syntax

| coding_unit( x0, y0, log2CUSize ) { | Descriptor |
| --- | --- |
| if( slice_type != I ) |  |
| skip_flag[ x0 ][ y0 ] | ae(v) |
| if( skip_flag[ x0 ][ y0 ] ) |  |
| prediction_unit( x0, y0 , log2CUSize ) |  |
| else if ( slice_type != I ¦¦ log2CUSize == Log2MinCUSize) { |  |
| if (slice_type != I ¦¦ SliceGranularity != 0) { |  |
| pred_type | ae(v) |
| } |  |
| x1 = x0 + ( ( 1 << log2CUSize ) >> 1 ) |  |
| y1 = y0 + ( ( 1 << log2CUSize ) >> 1 ) |  |
| x2 = x1 − ( ( 1 << log2CUSize ) >> 2 ) |  |
| y2 = y1 − ( ( 1 << log2CUSize ) >> 2 ) |  |
| x3 = x1 + ( ( 1 << log2CUSize ) >> 2 ) |  |
| y3 = y1 + ( ( 1 << log2CUSize ) >> 2 ) |  |
| if( PartMode == PART_2Nx2N ) { |  |
| prediction_unit( x0, y0 , log2CUSize ) |  |
| } else if( PartMode == PART_2NxN ) { |  |
| prediction_unit( x0, y0 , log2CUSize ) |  |
| prediction_unit( x0, y1 , log2CUSize ) |  |
| } else if( PartMode == PART_Nx2N ) { |  |
| prediction_unit( x0, y0 , log2CUSize ) |  |
| prediction_unit( x1, y0 , log2CUSize ) |  |
| } else if( PartMode == PART_2NxnU ) { |  |
| prediction_unit( x0, y0 , log2CUSize ) |  |
| prediction_unit( x0, y2 , log2CUSize ) |  |
| } else if( PartMode == PART_2NxnD ) { |  |
| prediction_unit( x0, y0 , log2CUSize ) |  |
| prediction_unit( x0, y3 , log2CUSize ) |  |
| } else if( PartMode == PART_nLx2N ) { |  |
| prediction_unit( x0, y0 , log2CUSize ) |  |
| prediction_unit( x2, y0 , log2CUSize ) |  |
| } else if( PartMode == PART_nRx2N ) { |  |
| prediction_unit( x0, y0 , log2CUSize ) |  |
| prediction_unit( x3, y0 , log2CUSize ) |  |
| } else { /* PART_NxN */ |  |
| prediction_unit( x0, y0 , log2CUSize ) |  |
| prediction_unit( x1, y0 , log2CUSize ) |  |
| prediction_unit( x0, y1 , log2CUSize ) |  |
| prediction_unit( x1, y1 , log2CUSize ) |  |
| } |  |
| if( !pcm_flag ) { |  |
| transform_tree( x0, y0, log2CUSize, log2CUSize, 0, 0 ) |  |
| transform_coeff( x0, y0, log2CUSize, log2CUSize, 0, 0 ) |  |
| transform_coeff( x0, y0, log2CUSize, log2CUSize, 0, 1 ) |  |
| transform_coeff( x0, y0, log2CUSize, log2CUSize, 0, 2 ) |  |
| } |  |
| } |  |
| } |  |

#### 7.3.6.1 Coding unit syntax for partitioning mode and MPM flag

| coding_unit_part_mode_and_mpm_flag ( x0, y0, log2CUSize ) { | Descriptor |
| --- | --- |
| if (log2CUSize == Log2MinCUSize) { |  |
| pred_type | ae(v) |
| } |  |
| x1 = x0 + ( ( 1 << log2CUSize ) >> 1 ) |  |
| y1 = y0 + ( ( 1 << log2CUSize ) >> 1 ) |  |
| x2 = x1 − ( ( 1 << log2CUSize ) >> 2 ) |  |
| y2 = y1 − ( ( 1 << log2CUSize ) >> 2 ) |  |
| x3 = x1 + ( ( 1 << log2CUSize ) >> 2 ) |  |
| y3 = y1 + ( ( 1 << log2CUSize ) >> 2 ) |  |
| if( PartMode == PART_2Nx2N ) { |  |
| prediction_unit_mpm_flag( x0, y0 , log2CUSize ) |  |
| } else { /* PART_NxN */ |  |
| prediction_unit_mpm_flag( x0, y0 , log2CUSize ) |  |
| prediction_unit_mpm_flag( x1, y0 , log2CUSize ) |  |
| prediction_unit_mpm_flag( x0, y1 , log2CUSize ) |  |
| prediction_unit_mpm_flag( x1, y1 , log2CUSize ) |  |
| } |  |
| } |  |

#### 7.3.6.2 Coding unit syntax for luma intra mode

| coding_unit_luma_intra_mode( x0, y0, log2CUSize ) { | Descriptor |
| --- | --- |
| x1 = x0 + ( ( 1 << log2CUSize ) >> 1 ) |  |
| y1 = y0 + ( ( 1 << log2CUSize ) >> 1 ) |  |
| x2 = x1 − ( ( 1 << log2CUSize ) >> 2 ) |  |
| y2 = y1 − ( ( 1 << log2CUSize ) >> 2 ) |  |
| x3 = x1 + ( ( 1 << log2CUSize ) >> 2 ) |  |
| y3 = y1 + ( ( 1 << log2CUSize ) >> 2 ) |  |
| if( PartMode == PART_2Nx2N ) { |  |
| prediction_unit_luma_intra_mode( x0, y0 , log2CUSize ) |  |
| } else { /* PART_NxN */ |  |
| prediction_unit_luma_intra_mode( x0, y0 , log2CUSize ) |  |
| prediction_unit_luma_intra_mode( x1, y0 , log2CUSize ) |  |
| prediction_unit_luma_intra_mode( x0, y1 , log2CUSize ) |  |
| prediction_unit_luma_intra_mode( x1, y1 , log2CUSize ) |  |
| } |  |
| } |  |

### 7.3.7 Prediction unit syntax

| prediction_unit( x0, y0, log2CUSize ) { | Descriptor |
| --- | --- |
| if( skip_flag[ x0 ][ y0 ] ) { |  |
| if( MaxNumMergeCand > 1 ) |  |
| merge_idx[ x0 ][ y0 ] | ae(v) |
| } else if( PredMode = = MODE_INTRA ) { |  |
| if (SliceGranularity == 0) { |  |
| if( !pcm_flag ) { |  |
| intra_chroma_pred_mode[ x0 ][ y0 ] | ae(v) |
| SignaledAsChromaDC = ( chroma_pred_from_luma_enabled_flag ? intra_chroma_pred_mode[ x0 ][ y0 ] == 3 : intra_chroma_pred_mode[ x0 ][ y0 ] == 2 ) |  |
| } |  |
| } else { |  |
| if( PartMode == PART_2Nx2N && log2CUSize >= Log2MinIPCMCUSize ) |  |
| pcm_flag | ae(v) |
| if( pcm_flag ) { |  |
| while ( !byte_aligned( ) ) |  |
| pcm_alignment_zero_bit | u(v) |
| for( i = 0; i < 1 << ( log2CUSize << 1 ); i++ ) |  |
| pcm_sample_luma[ i ] | u(v) |
| for( i = 0; i < ( 1 << ( log2CUSize << 1 ) ) >> 1; i++ ) |  |
| pcm_sample_chroma[ i ] | u(v) |
| } else { |  |
| prev_intra_luma_pred_flag[ x0 ][ y0 ] | ae(v) |
| if( prev_intra_luma_pred_flag[ x0 ][ y0 ] ) |  |
| mpm_idx[ x0 ][ y0 ] | ae(v) |
| else |  |
| rem_intra_luma_pred_mode[ x0 ][ y0 ] | ae(v) |
| intra_chroma_pred_mode[ x0 ][ y0 ] | ae(v) |
| SignaledAsChromaDC = ( chroma_pred_from_luma_enabled_flag ? intra_chroma_pred_mode[ x0 ][ y0 ] == 3 : intra_chroma_pred_mode[ x0 ][ y0 ] == 2 ) |  |
| } |  |
| } |  |
| } else { /* MODE_INTER */ |  |
| merge_flag[ x0 ][ y0 ] | ae(v) |
| if( merge_flag[ x0 ][ y0 ] ) { |  |
| if( MaxNumMergeCand > 1 ) |  |
| merge_idx[ x0 ][ y0 ] | ae(v) |
| } else { |  |
| if( slice_type = = B ) |  |
| inter_pred_flag[ x0 ][ y0 ] | ae(v) |
| if( inter_pred_flag[ x0 ][ y0 ] = = Pred_LC ) { |  |
| if( num_ref_idx_lc_active_minus1 > 0 ) |  |
| ref_idx_lc[ x0 ][ y0 ] | ae(v) |
| mvd_coding(mvd_lc[ x0 ][ y0 ][ 0 ], mvd_lc[ x0 ][ y0 ][ 1 ]) |  |
| mvp_idx_lc[ x0 ][ y0 ] | ae(v) |
| } |  |
| else { /* Pred_L0 or Pred_BI */ |  |
| if( num_ref_idx_l0_active_minus1 > 0 ) |  |
| ref_idx_l0[ x0 ][ y0 ] | ae(v) |
| mvd_coding(mvd_l0[ x0 ][ y0 ][ 0 ], mvd_l0[ x0 ][ y0 ][ 1 ]) |  |
| mvp_idx_l0[ x0 ][ y0 ] | ae(v) |
| } |  |
| if( inter_pred_flag[ x0 ][ y0 ] = = Pred_BI ) { |  |
| if( num_ref_idx_l1_active_minus1 > 0 ) |  |
| ref_idx_l1[ x0 ][ y0 ] | ae(v) |
| mvd_coding(mvd_l1[ x0 ][ y0 ][ 0 ], mvd_l1[ x0 ][ y0 ][ 1 ]) |  |
| mvp_idx_l1[ x0 ][ y0 ] | ae(v) |
| } |  |
| } |  |
| } |  |
| } |  |

#### 7.3.7.2 Prediction unit syntax for MPM flag

| prediction_unit_mpm_flag( x0, y0, log2CUSize ) { | Descriptor |
| --- | --- |
| if( PredMode = = MODE_INTRA ) { |  |
| if( PartMode == PART_2Nx2N && log2CUSize >= Log2MinIPCMCUSize ) |  |
| pcm_flag | ae(v) |
| if( pcm_flag ) { |  |
| while ( !byte_aligned( ) ) |  |
| pcm_alignment_zero_bit | u(v) |
| for( i = 0; i < 1 << ( log2CUSize << 1 ); i++ ) |  |
| pcm_sample_luma[ i ] | u(v) |
| for( i = 0; i < ( 1 << ( log2CUSize << 1 ) ) >> 1; i++ ) |  |
| pcm_sample_chroma[ i ] | u(v) |
| } else { |  |
| prev_intra_luma_pred_flag[ x0 ][ y0 ] | ae(v) |
| } |  |
| } |  |
| } |  |

#### 7.3.7.3 Prediction unit syntax for luma intra mode

| prediction_unit_luma_intra_mode( x0, y0, log2CUSize ) { | Descriptor |
| --- | --- |
| if( PredMode = = MODE_INTRA ) { |  |
| if( !pcm_flag[ x0 ][ y0 ] ) { |  |
| if( prev_intra_luma_pred_flag[ x0 ][ y0 ] ) |  |
| mpm_idx[ x0 ][ y0 ] | ae(v) |
| else |  |
| rem_intra_luma_pred_mode[ x0 ][ y0 ] | ae(v) |
| } |  |
| } |  |

## Categories

The above proposed changes to the draft represent a way to decode stream particularly suitable for expression in sequential algorithmic form. However for hardware implementations it might be beneficial to know data relations and dependencies. The tables below represent the relevant parts of the original WD5 syntax description sections along with categorization of data relevant to IMBP. From the categorization point the change introduced by IMBP is the grouping of all data belonging to the same category over a single LCU in the order of appearance of individual syntax elements.

The data in the category 1 in the tables below corresponds to the structural and other arithmetically encoded LCU data depicted as green blocks on Figure 2.2. The data in category 2 corresponds to the grouped bypass-coded intra-prediction mode data, the blue blocks on Figure 2.2. Finally the data in category 3 corresponds to the residual coefficient and other bypass and arithmetically coded data, yellow blocks on Figure 2.2

### 7.3.5 Coding tree syntax

| coding_tree( x0, y0, log2CUSize, cuDepth ) { | Category | Descriptor |
| --- | --- | --- |
| if( x0 + ( 1 << log2CUSize ) <= PicWidthInSamplesL && y0 + ( 1 << log2CUSize ) <= PicHeightInSamplesL && cuAddress( x0, y0 ) >= SliceAddress && log2CUSize > Log2MinCUSize ) { |  |  |
| split_coding_unit_flag[ x0 ][ y0 ] | 1 | ae(v) |
| } |  |  |
| if( adaptive_loop_filter_flag && alf_cu_control_flag ) { |  |  |
| if( cuDepth <= alf_cu_control_max_depth ) |  |  |
| if( cuDepth == alf_cu_control_max_depth ¦¦ |  |  |
| split_coding_unit_flag[ x0 ][ y0 ] == 0 ) |  |  |
| AlfCuFlagIdx++ |  |  |
| } |  |  |
| if( cu_qp_delta_enabled_flag && log2CUSize >= log2MinCUDQPSize ) |  |  |
| IsCuQpDeltaCoded = 0 |  |  |
| if( split_coding_unit_flag[ x0 ][ y0 ] ) { |  |  |
| x1 = x0 + ( ( 1 << log2CUSize ) >> 1 ) |  |  |
| y1 = y0 + ( ( 1 << log2CUSize ) >> 1 ) |  |  |
| if( cuAddress( x1, y0 ) > SliceAddress ) |  |  |
| moreDataFlag = coding_tree( x0, y0, log2CUSize – 1, cuDepth + 1 ) |  |  |
| if( cuAddress( x0, y1 ) > SliceAddress && moreDataFlag && x1 < PicWidthInSamplesL ) |  |  |
| moreDataFlag = coding_tree( x1, y0, log2CUSize − 1, cuDepth + 1 ) |  |  |
| if( cuAddress( x1, y1 ) > SliceAddress && moreDataFlag && y1 < PicHeightInSamplesL ) |  |  |
| moreDataFlag = coding_tree( x0, y1, log2CUSize − 1, cuDepth + 1 ) |  |  |
| if( moreDataFlag && x1 < PicWidthInSamplesL && y1 < PicHeightInSamplesL ) |  |  |
| moreDataFlag = coding_tree( x1, y1, log2CUSize − 1, cuDepth + 1 ) |  |  |
| } else { |  |  |
| if(adaptive_loop_filter_flag && alf_cu_control_flag ) |  |  |
| AlfCuFlag[ x0 ][ y0 ] = alf_cu_flag[ AlfCuFlagIdx ] |  |  |
| coding_unit( x0, y0, log2CUSize ) |  |  |
| if( granularity_block_boundary( x0, y0, log2CUSize ) ) { |  |  |
| end_of_slice_flag | 3 | ae(v) |
| moreDataFlag = !end_of_slice_flag |  |  |
| } else |  |  |
| moreDataFlag = 1 |  |  |
| } |  |  |
| return moreDataFlag |  |  |
| } |  |  |

### 7.3.6 Coding unit syntax

| coding_unit( x0, y0, log2CUSize ) { | Category | Descriptor |
| --- | --- | --- |
| if( slice_type != I ) |  |  |
| skip_flag[ x0 ][ y0 ] | n/a | ae(v) |
| if( skip_flag[ x0 ][ y0 ] ) |  |  |
| prediction_unit( x0, y0 , log2CUSize ) |  |  |
| else if( slice_type != I ¦ ¦ log2CUSize = = Log2MinCUSize ) { |  |  |
| pred_type | 1 | ae(v) |
| x1 = x0 + ( ( 1 << log2CUSize ) >> 1 ) |  |  |
| y1 = y0 + ( ( 1 << log2CUSize ) >> 1 ) |  |  |
| x2 = x1 − ( ( 1 << log2CUSize ) >> 2 ) |  |  |
| y2 = y1 − ( ( 1 << log2CUSize ) >> 2 ) |  |  |
| x3 = x1 + ( ( 1 << log2CUSize ) >> 2 ) |  |  |
| y3 = y1 + ( ( 1 << log2CUSize ) >> 2 ) |  |  |
| if( PartMode == PART_2Nx2N ) { |  |  |
| prediction_unit( x0, y0 , log2CUSize ) |  |  |
| } else if( PartMode == PART_2NxN ) { |  |  |
| prediction_unit( x0, y0 , log2CUSize ) |  |  |
| prediction_unit( x0, y1 , log2CUSize ) |  |  |
| } else if( PartMode == PART_Nx2N ) { |  |  |
| prediction_unit( x0, y0 , log2CUSize ) |  |  |
| prediction_unit( x1, y0 , log2CUSize ) |  |  |
| } else if( PartMode == PART_2NxnU ) { |  |  |
| prediction_unit( x0, y0 , log2CUSize ) |  |  |
| prediction_unit( x0, y2 , log2CUSize ) |  |  |
| } else if( PartMode == PART_2NxnD ) { |  |  |
| prediction_unit( x0, y0 , log2CUSize ) |  |  |
| prediction_unit( x0, y3 , log2CUSize ) |  |  |
| } else if( PartMode == PART_nLx2N ) { |  |  |
| prediction_unit( x0, y0 , log2CUSize ) |  |  |
| prediction_unit( x2, y0 , log2CUSize ) |  |  |
| } else if( PartMode == PART_nRx2N ) { |  |  |
| prediction_unit( x0, y0 , log2CUSize ) |  |  |
| prediction_unit( x3, y0 , log2CUSize ) |  |  |
| } else { /* PART_NxN */ |  |  |
| prediction_unit( x0, y0 , log2CUSize ) |  |  |
| prediction_unit( x1, y0 , log2CUSize ) |  |  |
| prediction_unit( x0, y1 , log2CUSize ) |  |  |
| prediction_unit( x1, y1 , log2CUSize ) |  |  |
| } |  |  |
| if( !pcm_flag ) { |  |  |
| transform_tree( x0, y0, log2CUSize, log2CUSize, 0, 0 ) |  |  |
| transform_coeff( x0, y0, log2CUSize, log2CUSize, 0, 0 ) |  |  |
| transform_coeff( x0, y0, log2CUSize, log2CUSize, 0, 1 ) |  |  |
| transform_coeff( x0, y0, log2CUSize, log2CUSize, 0, 2 ) |  |  |
| } |  |  |
| } |  |  |
| } |  |  |

### 7.3.7 Prediction unit syntax

| prediction_unit( x0, y0, log2CUSize ) { | Category | Descriptor |
| --- | --- | --- |
| if( skip_flag[ x0 ][ y0 ] ) { |  |  |
| if( MaxNumMergeCand > 1 ) |  |  |
| merge_idx[ x0 ][ y0 ] | n/a | ae(v) |
| } else if( PredMode = = MODE_INTRA ) { |  |  |
| if( PartMode == PART_2Nx2N && log2CUSize >= Log2MinIPCMCUSize ) |  |  |
| pcm_flag | n/a | ae(v) |
| if( pcm_flag ) { |  |  |
| while ( !byte_aligned( ) ) |  |  |
| pcm_alignment_zero_bit | n/a | u(v) |
| for( i = 0; i < 1 << ( log2CUSize << 1 ); i++ ) |  |  |
| pcm_sample_luma[ i ] | n/a | u(v) |
| for( i = 0; i < ( 1 << ( log2CUSize << 1 ) ) >> 1; i++ ) |  |  |
| pcm_sample_chroma[ i ] | n/a | u(v) |
| } else { |  |  |
| prev_intra_luma_pred_flag[ x0 ][ y0 ] | 1 (2 if EP) | ae(v) |
| if( prev_intra_luma_pred_flag[ x0 ][ y0 ] ) |  |  |
| mpm_idx[ x0 ][ y0 ] | 2 | ae(v) |
| else |  |  |
| rem_intra_luma_pred_mode[ x0 ][ y0 ] | 2 | ae(v) |
| intra_chroma_pred_mode[ x0 ][ y0 ] | 3 | ae(v) |
| SignaledAsChromaDC = ( chroma_pred_from_luma_enabled_flag ? intra_chroma_pred_mode[ x0 ][ y0 ] == 3 : intra_chroma_pred_mode[ x0 ][ y0 ] == 2 ) |  |  |
| } |  |  |
| } else { /* MODE_INTER */ |  |  |
| merge_flag[ x0 ][ y0 ] | n/a | ae(v) |
| if( merge_flag[ x0 ][ y0 ] ) { |  |  |
| if( MaxNumMergeCand > 1 ) |  |  |
| merge_idx[ x0 ][ y0 ] | n/a | ae(v) |
| } else { |  |  |
| if( slice_type = = B ) |  |  |
| inter_pred_flag[ x0 ][ y0 ] | n/a | ae(v) |
| if( inter_pred_flag[ x0 ][ y0 ] = = Pred_LC ) { |  |  |
| if( num_ref_idx_lc_active_minus1 > 0 ) |  |  |
| ref_idx_lc[ x0 ][ y0 ] | n/a | ae(v) |
| mvd_coding(mvd_lc[ x0 ][ y0 ][ 0 ], mvd_lc[ x0 ][ y0 ][ 1 ]) |  |  |
| mvp_idx_lc[ x0 ][ y0 ] | n/a | ae(v) |
| } |  |  |
| else { /* Pred_L0 or Pred_BI */ |  |  |
| if( num_ref_idx_l0_active_minus1 > 0 ) |  |  |
| ref_idx_l0[ x0 ][ y0 ] | n/a | ae(v) |
| mvd_coding(mvd_l0[ x0 ][ y0 ][ 0 ], mvd_l0[ x0 ][ y0 ][ 1 ]) |  |  |
| mvp_idx_l0[ x0 ][ y0 ] | n/a | ae(v) |
| } |  |  |
| if( inter_pred_flag[ x0 ][ y0 ] = = Pred_BI ) { |  |  |
| if( num_ref_idx_l1_active_minus1 > 0 ) |  |  |
| ref_idx_l1[ x0 ][ y0 ] | n/a | ae(v) |
| mvd_coding(mvd_l1[ x0 ][ y0 ][ 0 ], mvd_l1[ x0 ][ y0 ][ 1 ]) |  |  |
| mvp_idx_l1[ x0 ][ y0 ] | n/a | ae(v) |
| } |  |  |
| } |  |  |
| } |  |  |
| } |  |  |

# Conclusion

Presented is an approach to change the intra mode CU coding syntax. The authors state that the presented approach improves the performance and allows parallel processing of the bypass-coded intra prediction mode bins in intra mode CUs. Results were presented using HM-5.0.

# References

[1] B. Bross, et al., “WD5: Working Draft 5 of High-Efficiency Video Coding” JCT-VC document JCTVC-G1103, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21–30 November, 2011.

[2] F. Bossen, “Common HM test conditions and software reference configurations” JCT-VC document JCTVC-G1200, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21–30 November, 2011.

# Patent rights declaration(s)

CiSRA may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Canon Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).