| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 8th Meeting: San José, CA, USA, 1–10 February, 2012 | Document: JCTVC-H0245 |
| --- | --- |

| Title: | CE11: cross-verification of Motorola's proposal (H0511) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Xiaozhen Zheng | Tel: Email: | zhengxiaozhen@huawei.com |
| Source: | HiSilicon Technologies |  |  |

_____________________________

# Abstract

This contribution document reports the cross verification results of proposal H0245. The source code was implemented based on HM-5.0. We compiled, inspected, and ran the code of these cases with Intra-HE, Intra-LC, RA-HE, LB-HE, RA-LC, LB-LC, and RA-HE10 configurations. The cross-check has been completed successfully and the results match those provided by proponents of JCTVC-H0245.

# Test description and crosscheck results

This cross-check document reports the cross-check results of CE11 subtest 4. The test case was crosschecked by setting the macro “MOT_NSQT_SCAN” to 1, and macro “MOT_SIGMAP” to 1. At H0511, 16x4 and 4x16 transform blocks’ coefficient scanning is changed from subblock scanning + 4x4 diagonal scanning to 16x4 / 4x16 diagonal scanning.

Meanwhile, the context derivation of significance map for non-square transform block is also changed at H0511. The derivation method provided at H0511 updates the context based on 4 or 5 neighbor coefficients,. The tables 1 shows the results of the 16x4 / 4x16 diagonal scanning + first significance map context derivation method (marked as “Case 1”).

The encoder and decoder reconstructed YUV files for two cases are matched. The experimental results of the rate-distortion are illustrated in Error: Reference source not found~ 2. These results match the BD-rate results provided by proponents of JCTVC- H0245.

Table 1 Results of H0511 (Case 1)

|  | Random Access HE |  |  | Random Access LC |  |  | Random Access HE-10 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A (8bit) | 0.0% | 0.0% | -0.1% | #VALUE! | #VALUE! | #VALUE! | 0.2% | 0.2% | 1.3% |
| Class B | 0.1% | 0.4% | 0.5% | #VALUE! | #VALUE! | #VALUE! | 0.1% | 0.4% | 0.4% |
| Class C | 0.0% | 0.0% | 0.2% | #VALUE! | #VALUE! | #VALUE! |  |  |  |
| Class D | 0.1% | -0.1% | 0.1% | #VALUE! | #VALUE! | #VALUE! |  |  |  |
| Class E |  |  |  |  |  |  |  |  |  |
| Overall | 0.0% | 0.1% | 0.2% | #VALUE! | #VALUE! | #VALUE! | 0.1% | 0.3% | 0.8% |
|  | 0.0% | 0.1% | 0.2% | #VALUE! | #VALUE! | #VALUE! | 0.1% | 0.4% | 0.8% |
| Class F | -0.1% | 0.1% | 0.1% | #VALUE! | #VALUE! | #VALUE! |  |  |  |
| Enc Time[%] | 100% |  |  | #NUM! |  |  | 100% |  |  |
| Dec Time[%] | 101% |  |  | #NUM! |  |  | 101% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  | Low delay B HE-10 |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |  |  |  |
| Class B | 0.0% | 0.6% | 0.5% | #VALUE! | #VALUE! | #VALUE! |  |  |  |
| Class C | 0.1% | 0.2% | 0.2% | #VALUE! | #VALUE! | #VALUE! |  |  |  |
| Class D | 0.0% | -0.2% | 0.2% | #VALUE! | #VALUE! | #VALUE! |  |  |  |
| Class E | 0.1% | 0.0% | -0.4% | #VALUE! | #VALUE! | #VALUE! |  |  |  |
| Overall | 0.0% | 0.2% | 0.2% | #VALUE! | #VALUE! | #VALUE! |  |  |  |
|  | 0.0% | 0.3% | 0.2% | #VALUE! | #VALUE! | #VALUE! |  |  |  |
| Class F | 0.0% | 0.0% | -0.2% | #VALUE! | #VALUE! | #VALUE! |  |  |  |
| Enc Time[%] | 100% |  |  | #NUM! |  |  |  |  |  |
| Dec Time[%] | 99% |  |  | #NUM! |  |  |  |  |  |