| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 8th Meeting: San José, CA, USA, 1–10 February, 2012 | Document: JCTVC-H0246 |
| --- | --- |

| Title: | CE5: Cross-verification of Ghent University's proposal on transform skipping (H0169) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Xiaozhen Zheng | Tel: Email: | zhengxiaozhen@huawei.com |
| Source: | HiSilicon Technologies |  |  |

_____________________________

# Abstract

This contribution document reports the cross verification results of proposal H0169. The source code was implemented based on HM-5.0. We compiled, inspected, and ran the code of these cases with RA-HE, LB-HE, RA-LC, LB-LC, and RA-HE10 configurations. The cross-check has been completed successfully and the results match those provided by proponents of JCTVC-H0246.

# Test description and crosscheck results

This cross-check document report the cross-check results of the proposal H0246. H0246 is based on CE5’s basis source code that was released by BBC. Transform skipping is performed depending on TU sizes. H0246 provides three test cases that are summarized below:

Case1: transform skipping is used at TU whose width and height are smaller than 4 (set macro “TSM_SIZE” to 4);

Case2: transform skipping is used at TU whose width and height are smaller than 8(set macro “TSM_SIZE” to 8);

Case3: transform skipping is used at TU whose width and height are smaller than 16(set macro “TSM_SIZE” to 16)

For other sizes than indicated, only the 2D transform is performed on residual coding. The cross-check results of LB-HE, LB-LC at Case2, and results of Case3 are reported at this contribution

Table 1 Results of TSM_SIZE = 8 (Case 2)

|  | Low delay B HE |  |  | Low delay B LC |  |  | Low delay B HE-10 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |  |  |  |
| Class B | -0.1% | 0.2% | 0.3% | -0.4% | 0.4% | 1.0% |  |  |  |
| Class C | -0.5% | -0.4% | -0.3% | -1.1% | -0.1% | 0.5% |  |  |  |
| Class D | -0.9% | 0.1% | 0.2% | -1.8% | 2.2% | 2.0% |  |  |  |
| Class E | 0.0% | 0.4% | 0.4% | -0.2% | 0.7% | 1.0% |  |  |  |
| Overall | -0.4% | 0.1% | 0.2% | -0.9% | 0.8% | 1.1% |  |  |  |
|  | -0.4% | 0.0% | 0.1% | -0.9% | 0.8% | 1.1% |  |  |  |
| Class F | -4.0% | -2.7% | -2.5% | -5.1% | -2.8% | -3.0% |  |  |  |
| Enc Time[%] | 111% |  |  | 108% |  |  |  |  |  |
| Dec Time[%] | 100% |  |  | 102% |  |  |  |  |  |

Table 2 Results of TSM_SIZE = 16 (Case 3)

|  | Random Access HE |  |  | Random Access LC |  |  | Random Access HE-10 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A (8bit) | 0.0% | -0.2% | -0.3% | 0.0% | -0.1% | 0.0% | 0.1% | 0.0% | -0.1% |
| Class B | -0.2% | 0.0% | 0.1% | -0.5% | 0.3% | 0.5% | -0.1% | 0.0% | 0.1% |
| Class C | -0.4% | -0.3% | -0.2% | -0.9% | -0.1% | 0.2% |  |  |  |
| Class D | -0.8% | -0.1% | -0.2% | -1.4% | 0.3% | 0.2% |  |  |  |
| Class E |  |  |  |  |  |  |  |  |  |
| Overall | -0.4% | -0.1% | -0.1% | -0.8% | 0.1% | 0.3% | 0.0% | 0.0% | 0.0% |
|  | -0.4% | -0.1% | -0.1% | -0.8% | 0.1% | 0.2% | 0.0% | 0.0% | 0.0% |
| Class F | -1.9% | -1.0% | -1.0% | -2.6% | -1.4% | -1.5% |  |  |  |
| Enc Time[%] | 121% |  |  | 116% |  |  | 121% |  |  |
| Dec Time[%] | 99% |  |  | 103% |  |  | 100% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  | Low delay B HE-10 |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |  |  |  |
| Class B | -0.2% | 0.4% | 0.5% | -0.8% | 0.0% | 0.8% |  |  |  |
| Class C | -0.7% | -0.3% | -0.2% | -1.4% | 0.1% | 0.5% |  |  |  |
| Class D | -1.2% | 0.4% | 0.3% | -2.0% | 1.6% | 1.5% |  |  |  |
| Class E | -0.1% | 0.3% | -0.1% | -0.9% | 1.3% | 1.9% |  |  |  |
| Overall | -0.5% | 0.2% | 0.2% | -1.3% | 0.7% | 1.1% |  |  |  |
|  | -0.5% | 0.2% | 0.1% | -1.2% | 0.7% | 1.0% |  |  |  |
| Class F | -4.3% | -2.9% | -2.9% | -5.7% | -4.0% | -3.5% |  |  |  |
| Enc Time[%] | 118% |  |  | 112% |  |  |  |  |  |
| Dec Time[%] | 100% |  |  | 99% |  |  |  |  |  |

After the inspection of software provided by the proponents, it is confirmed that the modification of source code is match with H0246's description.