# Annex A: WD5(draft3) modification for the proposed method

Yellow marked: JCTVC-H0090

Green marked: JCTVC-H0092

Cyan marked: JCTVC-H0247

## Decoding process for coding units coded in inter prediction mode

Inputs to this process are:

–	a luma location ( xC, yC ) specifying the top-left luma sample of the current coding unit relative to the top left luma sample of the current picture,

–	a variable log2CUSize specifying the size of the current coding unit.

Output of this process is a modified reconstructed picture before deblocking filtering.

The variable nCSL is set equal to 1 << log2CUSize and the variable nCSC is set equal to ( 1 << log2CUSize ) >> 1.

Decoding process for coding units coded in inter prediction mode consists of following ordered steps:

- The inter prediction process as specified in subclause 8.4.1 is invoked with the luma location ( xC, yC ), coding unit size log2CUSize, inter prediction mode PredMode, and prediction partition mode PartMode as the inputs and the outputs are 3 arrays predSamplesL, predSamplesCb, predSamplesCr.

- The decoding process for the residual signal of coding units coded in inter prediction mode specified in subclause Error: Reference source not found is invoked with the luma location ( xC, yC ), the size of the current coding unit log2CUSize as inputs and the outputs are 3 arrays resSamplesL, resSamplesCb, resSamplesCr.

- The residual signal accumulation process as specified in subclause XXXX is invoked with arrays predSamplesL, predSamplesCb, predSamplesCr, and the arrays resSamplesL, resSamplesCb, resSamplesCr as the inputs and the outputs are 3 arrays recSamplesL, recSamplesCb, recSamplesCr. [Ed: (WJ) it may be split per each color component. Revisit after writing residual signal accumulation process]

- The picture reconstruction process for a component before deblocking filtering as specified in subclause XXXX is invoked with the luma location ( xB, yB ), and 3 arrays recSamplesL, recSamplesCb, recSamplesCr as the inputs and the output is a modified reconstructed picture before deblocking filtering. [Ed: (WJ) it may be split per each color component. Revisit after writing picture reconstruction process]

### Inter prediction process

This process is invoked when decoding coding unit whose PredMode is not equal to MODE_INTRA.

Inputs to this process are:

–	a luma location ( xC, yC ) specifying the top-left luma sample of the current coding unit relative to the top left luma sample of the current picture,

–	a variable log2CUSize specifying the size of the current coding unit,

–	a variable PredMode specifying prediction mode of current coding unit,

–	a variable PartMode specifying prediction partition mode of current coding unit.

Outputs of this process are:

–	a (nCSL)x(nCSL) array predSamplesL of luma prediction samples, where nCSL is derived as specified below,

–	a (nCSC)x(nCSC) array predSamplesCb of chroma prediction samples for the component Cb, where nCSC is derived as specified below,

–	a (nCSC)x(nCSC) array predSamplesCr of chroma prediction samples for the component Cr, where nCSC is derived as specified below.

The variable nCSL is set equal to 1 << log2CUSize and the variable nCSC is set equal to ( 1 << log2CUSize ) >> 1. [Ed: (WJ) revisit for supporting other chroma formats]

The variable nCS1L is set equal to nCSL >> 1.

Depending on PartMode, the following applies: [Ed: (WJ) is PUType better?]

–	If PartMode is equal to PART_2Nx2N, the following ordered steps apply:

- The decoding process for prediction units in inter prediction mode as specified in subclause 8.4.2 is invoked with the luma location ( xC, yC ), the luma location ( xB, yB ) set equal to ( 0, 0 ), the size of the coding unit nCSL, the width of the luma prediction samples nPSW set equal to nCSL, the height of the luma prediction samples nPSH set equal to nCSL and a partition index PartIdx set equal to 0 as inputs, and the outputs are a (nCSL)x(nCSL) array predSamplesL and two (nCSC)x(nCSC) arrays predSamplesCb and predSamplesCr.

–	Otherwise, if PartMode is equal to PART_2NxN, the following ordered steps apply:

- The decoding process for prediction units in inter prediction mode as specified in subclause 8.4.2 is invoked with the luma location ( xC, yC ), the luma location ( xB, yB ) set equal to ( 0, 0 ), the size of the coding unit nCSL, the width of the luma prediction samples nPSW set equal to nCSL, the height of the luma prediction samples nPSH set equal to nCS1L and a partition index PartIdx set equal to 0 as inputs, and the outputs are a (nCSL)x(nCSL) array predSamplesL and two (nCSC)x(nCSC) arrays predSamplesCb and predSamplesCr.

- The decoding process for prediction units in inter prediction mode as specified in subclause 8.4.2  is invoked with the luma location ( xC, yC ), the luma location ( xB, yB ) set equal to ( 0, nCS1L ), the size of the coding unit nCSL, the width of the luma prediction samples nPSW set equal to nCSL, the height of the luma prediction samples nPSH set equal to nCS1L and a partition index PartIdx set equal to 1 as inputs, and the outputs are a (nCSL)x(nCSL) array predSamplesL and two (nCSC)x(nCSC) arrays predSamplesCb and predSamplesCr.

Otherwise, if PartMode is equal to PART_Nx2N, the following ordered steps apply:

- The decoding process for prediction units in inter prediction mode as specified in subclause 8.4.2  is invoked with the luma location ( xC, yC ), the luma location ( xB, yB ) set equal to ( 0, 0 ), the size of the coding unit nCSL, the width of the luma prediction samples nPSW set equal to nCS1L, the height of the luma prediction samples nPSH set equal to nCSL and a partition index PartIdx set equal to 0 as inputs, and the outputs are a (nCSL)x(nCSL) array predSamplesL and two (nCSC)x(nCSC) arrays predSamplesCb and predSamplesCr.

- The decoding process for prediction units in inter prediction mode as specified in subclause 8.4.2  is invoked with the luma location ( xC, yC ), the luma location ( xB, yB ) set equal to ( nCS1L, 0 ), the size of the coding unit nCSL, the width of the luma prediction samples nPSW set equal to nCS1L, the height of the luma prediction samples nPSH set equal to nCSL and a partition index PartIdx set equal to 1 as inputs, and the outputs are a (nCSL)x(nCSL) array predSamplesL and two (nCSC)x(nCSC) arrays predSamplesCb and predSamplesCr.

Otherwise, if PartMode is equal to PART_NxN, the following ordered steps apply:

- The decoding process for prediction units in inter prediction mode as specified in subclause 8.4.2  is invoked with the luma location ( xC, yC ), the luma location ( xB, yB ) set equal to ( 0, 0 ), the size of the coding unit nCSL, the width of the luma prediction samples nPSW set equal to nCS1L, the height of the luma prediction samples nPSH set equal to nCS1L and a partition index PartIdx set equal to 0 as inputs, and the outputs are a (nCSL)x(nCSL) array predSamplesL and two (nCSC)x(nCSC) arrays predSamplesCb and predSamplesCr.

- The decoding process for prediction units in inter prediction mode as specified in subclause 8.4.2  is invoked with the luma location ( xC, yC ), the luma location ( xB, yB ) set equal to ( nCS1L, 0 ), the size of the coding unit nCSL, the width of the luma prediction samples nPSW set equal to nCS1L, the height of the luma prediction samples nPSH set equal to nCS1L and a partition index PartIdx set equal to 1 as inputs, and the outputs are a (nCSL)x(nCSL) array predSamplesL and two (nCSC)x(nCSC) arrays predSamplesCb and predSamplesCr.

- The decoding process for prediction units in inter prediction mode as specified in subclause 8.4.2  is invoked with the luma location ( xC, yC ), the luma location ( xB, yB ) set equal to ( 0, nCS1L ), the size of the coding unit nCSL, the width of the luma prediction samples nPSW set equal to nCS1L, the height of the luma prediction samples nPSH set equal to nCS1L and a partition index PartIdx set equal to 2 as inputs, and the outputs are a (nCSL)x(nCSL) array predSamplesL and two (nCSC)x(nCSC) arrays predSamplesCb and predSamplesCr.

- The decoding process for inter prediction units as specified in subclause 8.4.2  is invoked with the luma location ( xC, yC ), the luma location ( xB, yB ) set equal to ( nCS1L, nCS1L ), the size of the coding unit nCSL, the width of the luma prediction samples nPSW set equal to nCS1L, the height of the luma prediction samples nPSH set equal to nCS1L and a partition index PartIdx set equal to 3 as inputs, and the outputs are a (nCSL)x(nCSL) array predSamplesL and two (nCSC)x(nCSC) arrays predSamplesCb and predSamplesCr.

### Decoding process for prediction units in inter prediction mode

Inputs to this process are:

–	a luma location ( xC, yC ) specifying the top-left luma sample of the current coding unit relative to the top left luma sample of the current picture,

–	a luma location ( xB, yB ) specifying the top-left luma sample of the current prediction unit relative to the top left luma sample of the current coding unit,

–	a variable nCS specifying the size of the current coding unit,

–	a variable nPSW specifying the width of the current prediction unit,

–	a variable nPSH specifying the width of the current prediction unit,

- a variable PartIdx specifying the index of the current prediction unit within the current coding unit.

Outputs of this process are:

–	a (nCSL)x(nCSL) array predSamplesL of luma prediction samples, where nCSL is derived as specified below,

–	a (nCSC)x(nCSC) array predSamplesCb of chroma prediction samples for the component Cb, where nCSC is derived as specified below,

–	a (nCSC)x(nCSC) array predSamplesCr of chroma prediction samples for the component Cr, where nCSC is derived as specified below.

The variable nCSL is set equal to nCS and the variable nCSC is set equal to nCS >> 1. [Ed: (WJ) revisit for supporting other chroma formats]

The decoding process for prediction units in inter prediction mode consists of the following ordered steps:

- Derivation process for motion vector components and reference indices as specified in subclause 8.4.2.1.

Inputs to this process are

–	a luma location ( xC, yC ) of the top-left luma sample of the current coding unit relative to the top-left luma sample of the current picture,

–	a luma location ( xB, yB ) of the top-left luma sample of the current prediction unit relative to the top-left luma sample of the current coding unit,

–	a variable nCS specifying the size of the current coding unit,

–	variables specifying the width and the height of the prediction unit for luma, nPSW and nPSH,

  - a variable PartIdx specifying the index of the current prediction unit within the current coding unit.

Outputs of this process are

–	luma motion vectors mvL0 and mvL1, and chroma motion vectors mvCL0 and mvCL1,

–	reference indices refIdxL0 and refIdxL1,

–	prediction list utilization flags predFlagL0 and predFlagL1.

- Decoding process for inter sample prediction as specified in subclause Error: Reference source not found.

Inputs to this process are

–	a luma location ( xC, yC ) of the top-left luma sample of the current coding unit relative to the top-left luma sample of the current picture,

–	a luma location ( xB, yB ) of the top-left luma sample of the current prediction unit relative to the top-left luma sample of the current coding unit,

–	a variable nCS specifying the size of the current coding unit,

–	variables specifying the width and the height of the prediction unit for luma, nPSW and nPSH.

–	luma motion vectors mvL0 and mvL1, and chroma motion vectors mvCL0 and mvCL1,

–	reference indices refIdxL0 and refIdxL1,

–	prediction list utilization flags predFlagL0 and prefFlagL1.

Outputs of this process are

–	inter prediction samples (predSamples); which are a (nCSL)x(nCSL) array predSamplesL of prediction luma samples and two (nCSC)x(nCSC) arrays predSamplesCr, and predSamplesCr of prediction chroma samples, one for each of the chroma components Cb and Cr.

For use in derivation processes of variables invoked later in the decoding process, the following assignments are made:

MvL0[ xB, yB ] = mvL0		(8-1)
MvL1[ xB, yB ] = mvL1		(8-2)

RefIdxL0[ xB, yB ] = refIdxL0		(8-3)
RefIdxL1[ xB, yB ] = refIdxL1		(8-4)

PredFlagL0[ xB, yB ] = predFlagL0		(8-5)
PredFlagL1[ xB, yB ] = predFlagL1		(8-6)

#### Derivation process for motion vector components and reference indices

Input to this process are

- a luma location ( xC, yC ) of the top-left luma sample of the current coding unit relative to the top-left luma sample of the current picture,

- a luma location ( xB, yB ) of the top-left luma sample of the current prediction unit relative to the top-left luma sample of the current coding unit,

- a variable nCS specifying the size of the current coding unit,

- variables specifying the width and the height of the prediction unit for luma, nPSW and nPSH,

- a variable PartIdx specifying the index of the current prediction unit within the current coding unit.

Outputs of this process are

- luma motion vectors mvL0 and mvL1 and chroma motion vectors mvCL0 and mvCL1,

- reference indices refIdxL0 and refIdxL1,

- prediction list utilization flags predFlagL0 and predFlagL1.

Let ( xP, yP ) specify the top-left luma sample of the current prediction unit relative to the top-left luma sample of the current picture where xP = xC + xB and yP = yC + yB.

For the derivation of the variables mvL0 and mvL1, refIdxL0 and refIdxL1 as well as PredFlagL0 and PredFlagL1, the following applies.

- If PredMode is equal to MODE_SKIP, the derivation process for luma motion vectors for merge mode as specified in subclause  is invoked with the luma location ( xC, yC ), the luma location ( xP, yP ), variables nCS, nPSW, nPSH and the partition index PartIdx as inputs and the output being the luma motion vectors mvL0, mvL1, the reference indices refIdxL0, refIdxL1, and the prediction list utilization flags predFlagL0 and predFlagL1.

- Otherwise, if PredMode is equal to MODE_INTER and merge_flag[ xP ][ yP ] is equal to 1,, the derivation process for luma motion vectors for merge mode as specified in subclause  is invoked with the luma location ( xC, yC ), the luma location ( xP, yP ), variables nCS, nPSW and nPSH and the partition index PartIdx as inputs and the outputs being the luma motion vectors mvL0 and mvL1, the reference indices refIdxL0 and refIdxL1, the prediction utilization flags predFlagL0 and predFlagL1.

- Otherwise, for X being replaced by either 0 or 1 in the variables predFlagLX, mvLX, refIdxLX and in Pred_LX and in the syntax elements ref_idx_lX and mvd_lX, the following applies.

- The variables LcToLx, refIdxLX and predFlagLX are derived as follows.

- If inter_pred_flag[ xP ][ yP ] is equal to Pred_LC and PredLCToPredLx[ ref_idx_lc[ xP ][ yP ] ] is equal to Pred_LX,

refIdxLX = RefIdxLCToRefIdxLx[ ref_idx_lc[ xP ][ yP ] ]	(8-7)					predFlagLX = 1		(8-8)					mvd_lX[ xP ][ yP ][ 0 ] = mvd_lc[ xP ][ yP ][ 0 ]	(8-9)
					mvd_lX[ xP ][ yP ][ 1 ] = mvd_lc[ xP ][ yP ][ 1 ]	(8-10)
					mvp_idx_lX[ xP ][ yP ][ 0 ] = mvp_idx_lc[ xP ][ yP ]	(8-11)
					LcToLx = LX		(8-12)

- Otherwise, if inter_pred_flag[ xP ][ yP ] is equal to Pred_LX or Pred_BI,

refIdxLX = ref_idx_lX[ xP ][ yP ]		(8-13)
					predFlagLX = 1		(8-14)

- Otherwise, the variables refIdxLX and predFlagLX are specified by

refIdxLX = -1		(8-15)
					predFlagLX = 0		(8-16)

- The variable mvdLX is derived as follows.

mvdLX[ 0 ] = mvd_lX[ xP ][ yP ][ 0 ]		(8-17)
				mvdLX[ 1 ] = mvd_lX[ xP ][ yP ][ 1 ]		(8-18)

- When predFlagLX is equal to 1, the variable mvpLX is derived as follows.

- The derivation process for luma motion vector prediction in subclause Error: Reference source not found is invoked with the luma location ( xP, yP ), variables nPSW and nPSH and refIdxLX as the inputs and the output being mvpLX.

- When predFlagLX is equal to 1, the luma motion vector mvLX is derived as

mvLX[ 0 ] = mvpLX[ 0 ] + mvdLX[ 0 ]	(8-19)
				mvLX[ 1 ] = mvpLX[ 1 ] + mvdLX[ 1 ]	(8-20)

When ChromaArrayType is not equal to 0 and predFlagLX (with X being either 0 or 1) is equal to 1, the derivation process for chroma motion vectors in subclause Error: Reference source not found is invoked with mvLX and refIdxLX as inputs and the output being mvCLX.

##### 8.4.2.1.1 Derivation process for luma motion vectors for merge mode

This process is only invoked when PredMode is equal to MODE_SKIP or PredMode is equal to MODE_ INTER and merge_flag [ xP ][ yP ] is equal to 1, where ( xP, yP ) specify the top-left luma sample of the current prediction unit relative to the top-left luma sample of the current picture.

Inputs of this process are

- a luma location ( xC, yC ) of the top-left luma sample of the current coding unit relative to the top-left luma sample of the current picture,

- a luma location ( xP, yP ) of the top-left luma sample of the current prediction unit relative to the top-left luma sample of the current picture,

- a variable nCS specifying the size of the current coding unit,

- variables specifying the width and the height of the prediction unit for luma, nPSW and nPSH,

- a variable PartIdx specifying the index of the current prediction unit within the current coding unit.

Outputs of this process are

–	the luma motion vectors mvL0 and mvL1,

–	the reference indices refIdxL0 and refIdxL1,

–	the prediction list utilization flags predFlagL0 and predFlagL1.

The motion vectors mvL0 and mvL1, the reference indices refIdxL0 and refIdxL1, and the prediction utilization flags predFlagL0 and predFlagL1 are derived as specified by the following ordered steps:

- The derivation process for merging candidates from neighboring prediction unit partitions in subclause  is invoked with the luma location ( xC, yC ), the size of the current coding unit nCS, the luma location ( xP, yP ), the width and the height of the prediction unit nPSW and nPSH and the partition index PartIdx as inputs and the output is assigned to the parallel derivation flag parFlagN, the availability flags availableFlagN, the reference indices refIdxL0N and refIdxL1N, the prediction list utilization flags predFlagL0N and predFlagL1N and the motion vectors mvL0N and mvL1N with N being replaced by A0, A1, B0, B1 or B2.

- The reference index for temporal merging candidate is derived as follows.

- If the prediction unit covering luma location ( xC − 1, yC + nCS − 1 ) is available and PredMode is not MODE_INTRA and parFlagA1 is equal to 0, refIdxLX is set equal to refIdxLX[ xC − 1, yC + nCS − 1 ].

- Otherwise, refIdxLX is set equal to 0.

- The derivation process for temporal luma motion vector prediction in subclause Error: Reference source not found is invoked with luma location ( xP, yP ), refIdxLX as the inputs and with the output being the availability flag availableFlagLXCol and the temporal motion vector mvLXCol. The variables availableFlagCol and predFlagLXCol (with X being 0 or 1, respectively) are derived as specified below.

availableFlagCol = availableFlagL0Col || availableFlagL1Col                      (8-21)

predFlagLXCol = availableFlagLXCol                                        (8-22)

- The merging candidate list, mergeCandList, is constructed as follows.

  - The merging candidates with availableFlagN equal to 1 and parFlagN equal to 0 are added to mergeCandList in the order of A1, B1, B0, A0, and B2.

  - Col is added to mergeCandList if availableFlagCol is equal to 1.

  - For the merging candidates with parFlagN equal to 1, the derivation process for merging candidates from neighboring prediction unit partitions of MER in subclause  is invoked with luma location ( xP, yP ), the width and the height of the prediction unit nPSW and nPSH as inputs and the output is assigned to the availability flags availableFlagN, the reference indices refIdxL0N and refIdxL1N, the prediction list utilization flags predFlagL0N and predFlagL1N and the motion vectors mvL0N and mvL1N with N being replaced by A0, A1, B0, B1 or B2.

  - The merging candidates with availableFlagN equal to 1 are added to mergeCandList in the order of A1, B1, B0, A0 and B2.

- When merging candidates have the same motion vectors and the same reference indices, the merging candidates are removed from the list except the merging candidate which has the smallest order in the mergeCandList.

…

##### 8.4.2.1.2 Derivation process for spatial merging candidates

Inputs to this process are

- a luma location ( xC, yC ) of the top-left luma sample of the current coding unit relative to the top-left luma sample of the current picture,

- a luma location ( xP, yP ) specifying the top-left luma sample of the current prediction unit relative to the top-left sample of the current picture,

- a variable nCS specifying the size of the current coding unit,

- variables specifying the width and the height of the prediction unit for luma, nPSW and nPSH,

- a variable PartIdx specifying the index of the current prediction unit within the current coding unit.

Outputs of this process are (with N being replaced by A0, A1, B0, B1 or B2 and with X being replaced by 0 or 1)

- the parallel derivation flags parFlagN of the neighboring prediction units.

- the availability flags availableFlagN of the neighbouring prediction units,

- the reference indices refIdxLXN of the neighbouring prediction units,

- the prediction list utilization flags predFlagLXN of the neighbouring prediction units,

- the motion vectors mvLXN of the neighbouring prediction units.

When nPSW or nPSH is not greater than 8, xP is set equal to xC and nPSW and nPSH are set equal to nCS.

For the derivation of availableFlagN, with N being A0, A1, B0, B1 or B2 and ( xN, yN ) being ( xP – 1,  yP + nPSH ), ( xP − 1,  yP + nPSH − 1 ), ( xP + nPSW,  yP – 1 ), ( xP + nPSW − 1,  yP – 1 ) or ( xP – 1,  yP – 1 ), and (xN’, yN’) being ( xP – 4,  yP + nPSH ), ( xP − 4,  yP + nPSH − 4 ), ( xP + nPSW,  yP – 4 ), ( xP + nPSW − 4,  yP – 4 ) or ( xP – 4,  yP – 4 ), the following applies.

–	The variable parLevel is set equal to log2_parallel_merge_level_minus2 + 2.

–	parFlag is set equal to 1 if all of the following conditions are satisfied. Otherwise, parFlag is set equal to 0.

    - xN >> parLevel == xN’ >> parLevel

    - yN >> parLevel == yN’ >> parLevel

–	If one of the following conditions is true, the availableFlagN is set equal to 0, both components mvLXN are set equal to 0, refIdxLXN and predFlagLX[ xN, yN ] of the prediction unit covering luma location ( xN, yN ) are assigned respectively to mvLXN, refIdxLXN and predFlagLXN.

  - parFlagN is equal to 1.

  - N is equal to B2 and availableFlagA0 + availableFlagA1 + availableFlagB0 + availableFlagB1 is equal to 4.

  - The prediction unit covering luma location ( xN, yN ) is not available or PredMode is MODE_INTRA.

  - PartMode of the current prediction unit is PART_2NxN or PART_2NxnU or PART_2NxnD and PartIdx is equal to 1 and N is equal to B1 and nCS is greater than 8.

  - PartMode of the current prediction unit is PART_Nx2N or PART_nLx2N or PART_nRx2N and PartIdx is equal to 1 and N is equal to A1 and nCS is greater than 8.

–	Otherwise, availableFlagN is set equal to 1 and the variables mvLX[ xN, yN ], refIdxLX[ xN, yN ] and predFlagLX[ xN, yN ] of the prediction unit covering luma location ( xN, yN ) are assigned respectively to mvLXN, refIdxLXN and predFlagLXN.

##### 8.4.2.1.3 Derivation process for spatial merging candidates of MER

Inputs to this process are

- a luma location ( xP, yP ) specifying the top-left luma sample of the current prediction unit relative to the top-left sample of the current picture,

- variables specifying the width and the height of the prediction unit for luma, nPSW and nPSH,

Outputs of this process are (with N being replaced by A0, A1, B0, B1 or B2 and with X being replaced by 0 or 1)

- the availability flags availableFlagN of the neighbouring prediction units,

- the reference indices refIdxLXN of the neighbouring prediction units,

- the prediction list utilization flags predFlagLXN of the neighbouring prediction units,

- the motion vectors mvLXN of the neighbouring prediction units.

For the derivation of availableFlagN, with N being A0, A1, B0, B1 or B2 and ( xN, yN ) being ( xP – 1,  yP + nPSH ), ( xP − 1,  yP + nPSH − 1 ), ( xP + nPSW,  yP – 1 ), ( xP + nPSW − 1,  yP – 1 ) or ( xP – 1,  yP – 1 ), the following applies.

–	The variable parLevel is set equal to log2_parallel_merge_level_minus2 + 2.

–	If N is B2, the variable aboveFlag is set equal to 1 if xN % ( 1 << parLevel ) > yN % ( 1 << parLevel ). Otherwise, aboveFlag is set equal to 0.

–	The luma location ( xN’, yN’ ) is derived as follows.

  - If N is A0 or A1 or N is B2 and aboveFlag is set equal to 0, the following applies.

    - xN’ = ( xN >> parLevel ) << parLevel

    - yN’ = yN

  - Otherwise, the following applies.

    - xN’ = xN

    - yN’ = ( yN >> parLevel ) << parLevel

–	If the prediction unit covering luma location ( xN’, yN’ ) is not available or PredMode is MODE_INTRA, the availableFlagN is set equal to 0, both components mvLXN are set equal to 0, refIdxLXN and predFlagLX[ xN’, yN’ ] of the prediction unit covering luma location ( xN’, yN’ ) are assigned respectively to mvLXN, refIdxLXN and predFlagLXN.

–	Otherwise, availableFlagN is set equal to 1 and the variables mvLX[ xN’, yN’ ], refIdxLX[ xN’, yN’ ] and predFlagLX[ xN’, yN’ ] of the prediction unit covering luma location ( xN’, yN’ ) are assigned respectively to mvLXN, refIdxLXN and predFlagLXN