| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 8th Meeting: San José, CA, USA, 1–10 February, 2012 | Document: JCTVC-H0247 |
| --- | --- |

| Title: | AHG10: Unified design on parallel merge/skip with reduced candidates |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Hui Yong Kim (ETRI), *Kyung Yong Kim (KHU), ***Yongjoon Jeon, Byeongmoon Jeon (LG) **Minhua Zhou (Texas Instruments Inc.), ****Xing Wen, Oscar C. Au (HKUST) ETRI, 218 Gajeong-ro, Yuseong-gu, Daejeon, 305-700, Korea *Media Lab., Kyung Hee University 1, Seochun, Kiheung, Youngin, Gyonggi, 446-701, Korea **LG, #221 Yangjae-dong, Seoucho-gu, Seoul 137-130, Korea ***Texas Instruments Inc., USA ****Department of Electronic and Computer Engineering HKUST, Clear Water Bay, Kowloon, Hong Kong | Tel: Email: | +82-42-860-5069 hykim5@etri.re.kr *+82-31-201-3680 kimky@khu.ac.kr **+82-10-9043-9419 yongjoon.jeon@lge.com, bm.jeon@lge.com ***+1-214-480-3816 zhou@ti.com ****+852-66261845 wxxab@ust.hk, eeau@ust.hk |
| Source: | ETRI, KHU (Kyung Hee University), LG, TI (Texas Instruments Inc.), and HKUST (Hong Kong University of Science and Technology). |  |  |

_____________________________

# Abstract

This contribution reports the results for a unified design of JCTVC-H092, JCTVC-H0090, and JCTVC-H0240 for high-throughput merge/skip estimation. It is indicated that the unified solution provides the following features that are essential for high-throughput encoder design: 1) removal of inter-PU dependency by default, 2) further removal of inter-CU dependency in a configurable manner, and 3) reduction of the worst-case # of MCL constructions roughly by half. It is reported that the BDR penalty introduced by the unified design is roughly 0.2%, 0.4%, 1.1%, and 2.1% for PME (Parallel Merge Estimation) region sizes of 8x8, 16x16, 32x32, and 64x64, respectively.

# Introduction

This contribution aims at a unified design for parallel merge/skip estimation (PME) which essentially is a harmonized design of the following features:

- TMVP refIdx is derived independent of PUs of a same CU, removing the inter-PU dependency by default. (JCTVC-H0092)

- Each 8x8 CU is limited to have a single MCL at all parallel merge/skip levels, reducing the worst-case number of MCLs roughly by half. (JCTVC-H0240 CU-level MCL, but only limited to 8x8 CUs.)

- The MCL (Merge Candidate List) construction and merge/skip motion estimation processes are decoupled from the regular motion estimation at CU-group level, removing both the inter-PU dependency and the inter-CU dependency within a parallel merge estimation (PME) region. (JCTVC-H0082 and JCTVC-H0090)

- The second and third features are configurable by a high-level parameter as follows:

| parallel merge level | description |
| --- | --- |
| log2_parallel_merge_level_minus2 = 0 | 1) Within each CU, all TMVPs share the refIdx of 2Nx2N PU. |
| log2_parallel_merge_level_minus2 = 1 | 1) Within each CU, all TMVPs share the refIdx of 2Nx2N PU. 2) Within a 8x8 CU, all PUs share the MCL of 2Nx2N PU. 3) Parallel merge/skip is enabled for every 8x8 region. |
| log2_parallel_merge_level_minus2 = 2 | 1) Within each CU, all TMVPs share the refIdx of 2Nx2N PU. If the refIdx position is inside PME region, use ‘0’ instead. 2) Within a 8x8 CU, all PUs share the MCL of 2Nx2N PU. 3) Parallel merge/skip is enabled for every 16x16 region. |
| log2_parallel_merge_level_minus2 = 3 | 1) Within each CU, all TMVPs share the refIdx of 2Nx2N PU. If the refIdx position is inside PME region, use ‘0’ instead. 2) Within a 8x8 CU, all PUs share the MCL of 2Nx2N PU. 3) Parallel merge/skip is enabled for every 32x32 region. |
| log2_parallel_merge_level_minus2 = 4 | 1) Within each CU, all TMVPs share the refIdx of 2Nx2N PU. If the refIdx position is inside PME region, use ‘0’ instead. 2) Within a 8x8 CU, all PUs share the MCL of 2Nx2N PU. 3) Parallel merge/skip is enabled for every 64x64 region. |

With this unified solution, we essentially can achieve the following design goals:

- decouples the MCL construction and merge/skip motion estimation from the regular motion estimation at any CU-group levels and CU levels (this decoupling basically enables a HEVC encoder to fully leverage HEVC quality potential at a same motion estimation throughput of AVC)

- reduces the worst case number of MCLs by limiting each 8x8 CU to have a single MCL at all parallel merge/skip levels

- and is configurable to provide coding efficiency and throughput trade-offs on the encoder side.

# Algorithm description

In the current merge/skip list derivation process, the following rules are applied:

- If the current CU size is 8x8, the MCL of 2Nx2N PU is used for all the other PUs.

- When deriving spatial MVPs, if a neighbouring PU and the current PU belong to a same PME region, the MVP candidate is treated as unavailable.

- When deriving the reference index for TMVP, if the left neighbouring PU and the current PU belong to a same MER, the reference index for TMVP is set to 0. Otherwise, the TMVP reference index for 2Nx2N PU is used for the TMVP reference index for the current PU.

- After the TMVP reference index derivation process (before the first pruning stage), the MVPs made unavailable by the second rule are replaced by the MVPs located outside the PME region. The closest one to each original MVP position is used as shown in Fig. 1. If the PU is available and not intra-coded, the MVP is added to the list.

PME (Parallel Merge Estimation) region

Fig 1. The positions of the MVPs fetched from outside the PME region

# Experimental results

The proposed method was implemented on HM5.0 software. Table 3, Table 4, and Table 5 show the summary results of the proposed solution, the Variant 1, and the Variant 2, respectively, compared to HM 5.0 anchor under common test conditions [5].

Table 1. Summary result for log2_parallel_merge_level_minus2 = 0 
(The results were copied from JCTVC-H0092.)

Table 2. Summary result for log2_parallel_merge_level_minus2 = 1 (8x8 PME region)

Table 3. Summary result for log2_parallel_merge_level_minus2 = 2 (16x16 PME region)

Table 4. Summary result for log2_parallel_merge_level_minus2 = 3 (32x32 PME region)

Table 5. Summary result for log2_parallel_merge_level_minus2 = 4 (64x64 PME region)

# Conclusions

We proposed a unified design of JCTVC-H092, JCTVC-H0090, and JCTVC-H0240 for high-throughput merge/skip estimation. The unified solution provides 1) removal of inter-PU dependency by default, 2) further removal of inter-CU dependency in a configurable manner, and 3) reduction of the worst-case # of MCL constructions roughly by half. The BDR penalty introduced by the unified design is roughly 0.4% and 1.1% for 16x16 and 32x32 PME (Parallel Merge Estimation) region sizes, which are typically required pipeline block sizes for high-throughput applications such as real-time HD and UHD video encoding.

# Reference

- M. Zhou, “AHG10: Configurable and CU-group level parallel merge/skip,” JCTVC-H0082, Feb. 2012.

- Y. Jeon, et al., “AHG10: Unified design on parallel merge/skip,” JCTVC-H0090, Feb. 2012.

- Y. Jeon, et al., “Non-CE9: Removing PU dependency in TMVP reference index derivation,” JCTVC-H0092, Feb. 2012.

- H. Y. Kim, et al., “Non-CE9: Throughput improvement for merge/skip mode,” JCTVC-H0240, Feb.

- F. Bossen, "Common HM test conditions and software reference configurations," JCTVC-G1200, Nov. 2011.

# Patent rights declaration(s)

ETRI and Kyung Hee University may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

LG Electronics may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Texas Instruments, Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Hong Kong University of Science and Technology may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).