| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 8th Meeting: San José, CA, USA, 1–10 February, 2012 | Document: JCTVC-H0248 |
| --- | --- |

| Title: | Clean-up of redundant CABAC memorization process |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yi-Jen Chiu 2200 Mission College Blvd. Santa Clara CA USA 95054 | Tel: Email: | 408-765-9494 yi-jen.chiu@intel.com |
|  | Wenhao Zhang 8F Raycom Infotech Park A, No. 2 KeXueYuan South Rd., Haidian District, Beijing, China, 100190 | Tel: Email: | 86-10-82171657 wenhao.zhang@intel.com |
|  | Lidong Xu 8F Raycom Infotech Park A, No. 2 KeXueYuan South Rd., Haidian District, Beijing, China, 100190 | Tel: Email: | 86-10-82171425 lidong.xu@intel.com |
|  | Yu Han 8F Raycom Infotech Park A, No. 2 KeXueYuan South Rd., Haidian District, Beijing, China, 100190 | Tel: Email: | 86-10-82171388 yu.han@intel.com |
| Source: | Intel Corp. |  |  |

_____________________________

# Abstract

The document proposes a clean-up of the CABAC description in HM5.1 reference software and in Working Draft 5 to remove the redundant invocation of CABAC memorization process.

# Problem Statement

As described in Working Draft 5, the CABAC memorization process will be invoked in two scenarios:

Scenario 1: When starting the parsing of a coding tree as specified in subclause Error: Reference source not found and entropy_coding_synchro is not equal to 0, the following applies.

…

- Otherwise, if (tile_boundary_independence_flag is equal to 1 and ( tbAddr − firstTbInTileAddr ) % tileWidthInLCUs is equal to entropy_coding_synchro + 1) or (tile_boundary_independence_flag is equal to 0 and tbAddr % picWidthInLCUs is equal to entropy_coding_synchro + 1), the memorization process of the CABAC parsing process is invoked as specified in subclause Error: Reference source not found.

Scenario 2: When starting the parsing of a coding tree as specified in subclause Error: Reference source not found and tile_boundary_independence_idc is equal to 0, the following applies.

…

- Otherwise, if x0 is equal to x0Tile and y0 is equal to y0Tile+(1<< log2MaxCUSize) the memorization process of the CABAC parsing process is invoked as specified in subclause ‎‎Error: Reference source not found.

In scenario 1, memorization process stores all the current CABAC models, which will be loaded by the synchronization process at the starting of next LCU row. The invocation of memorization process is dependent of the value of tile_boundary_independence_flag. Two examples of invocation are shown in Figure 1 and Figure 2. When the current LCU row is at the bottom row of an independent tile and/or a frame, the stored CABAC models are not required to be loaded. As demonstrated in Figures 1 and 2, the memorization processes invoked after the LCU with marker “R” is redundant.

entropy_coding_synchro+1 LCUs

Tile boundary

After coding the blue LCU, memorization process is invoked.

R

R

R

R

Figure 1 Example of memorization process in scenario 1, when entropy_coding_synchro is not equal to 0 and tile_boundary_independence_flag is equal to 1

R

entropy_coding_synchro+1 LCUs

Tile boundary

After coding the blue LCU, memorization process is invoked.

Figure 2 Example of memorization process in scenario 1, when entropy_coding_synchro is not equal to 0 and tile_boundary_independence_flag is equal to 0

In scenario 2, memorization process is invoked to the purpose of low latency CABAC initialization for dependent tiles. All the CABAC models of top-right LCU of a tile are stored, which will be loaded by the synchronization process at the starting of right neighbouring tile. Figure 3 shows an example of scenario 2. When the current LCU is at the right edge of a frame, the stored CABAC models are not required to be loaded. As demonstrated in Figures 3, the memorization processes invoked after the LCU with marker “R” is also redundant.

Tile boundary

After coding the blue LCU, memorization process is invoked.

R

R

Figure 3 Example of memorization process in scenario 2, when tile_boundary_independence_flag is equal to 0

Working Draft 5 and HM5.1 reference software have this redundant memorization processes.

# Proposed solution

It is proposed to change the description in Working Draft by adding conditions to avoid redundant invoking of memorization process. The modification for the Working Draft text is as follows,

Scenario 1: When starting the parsing of a coding tree as specified in subclause Error: Reference source not found and entropy_coding_synchro is not equal to 0, the following applies.

…

- Otherwise, if (tile_boundary_independence_flag is equal to 1 and, ( tbAddr − firstTbInTileAddr ) % tileWidthInLCUs is equal to entropy_coding_synchro + 1 and ( tbAddr − firstTbInTileAddr ) / tileWidthInLCUs is not equal to tileHeightInLCUs – 1) or (tile_boundary_independence_flag is equal to 0 and, tbAddr % picWidthInLCUs is equal to entropy_coding_synchro + 1 and tbAddr / picWidthInLCUs is not equal to picHeightInLCUs – 1), the memorization process of the CABAC parsing process is invoked as specified in subclause Error: Reference source not found.

Scenario 2: When starting the parsing of a coding tree as specified in subclause Error: Reference source not found and tile_boundary_independence_idc is equal to 0, the following applies.

…

- Otherwise, if x0 is equal to x0Tile and, y0 is equal to y0Tile+(1<< log2MaxCUSize) and tbAddr % picWidthInLCUs is not equal to picWidthInLCUs – 1, the memorization process of the CABAC parsing process is invoked as specified in subclause ‎‎Error: Reference source not found.

The version of clean-up software on HM5.1 is provided as the attachment.

# Patent rights declaration(s)

Intel Corp. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).