| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 8th Meeting: San José, CA, USA, 1–10 February, 2012 | Document: JCTVC-H0248 |
| --- | --- |

| Title: | Clean-up of redundant CABAC memorization process |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yi-Jen Chiu 2200 Mission College Blvd. Santa Clara CA USA 95054 | Tel: Email: | 408-765-9494 yi-jen.chiu@intel.com |
|  | Wenhao Zhang 8F Raycom Infotech Park A, No. 2 KeXueYuan South Rd., Haidian District, Beijing, China, 100190 | Tel: Email: | 86-10-82171657 wenhao.zhang@intel.com |
|  | Lidong Xu 8F Raycom Infotech Park A, No. 2 KeXueYuan South Rd., Haidian District, Beijing, China, 100190 | Tel: Email: | 86-10-82171425 lidong.xu@intel.com |
|  | Yu Han 8F Raycom Infotech Park A, No. 2 KeXueYuan South Rd., Haidian District, Beijing, China, 100190 | Tel: Email: | 86-10-82171388 yu.han@intel.com |
| Source: | Intel Corp. |  |  |

_____________________________

# Abstract

The document proposes a clean-up of the CABAC description in HM5.0 reference software and in Working Draft 5.0 to remove the redundant invocation of CABAC memorization process.

# Problem Statement

Working Draft 5.0 has the following description for CABAC memorization process for the condition that the syntax entropy_coding_synchro is not equal to 0,

- Otherwise, if (tile_boundary_independence_flag is equal to 1 and ( tbAddr − firstTbInTileAddr ) % tileWidthInLCUs is equal to entropy_coding_synchro + 1) or (tile_boundary_independence_flag is equal to 0 and tbAddr % picWidthInLCUs is equal to entropy_coding_synchro + 1), the memorization process of the CABAC parsing process is invoked as specified in subclause Error: Reference source not found.

The invocation of memorization process is dependent of the value of tile_boundary_independence_flag. Two examples of invocation in Working Draft 5.0 are shown in Figure 1 and Figure 2. Memorization process stores all the current CABAC models, which will be loaded by the synchronization process at the starting of next LCU row. When the current LCU row is at the bottom row of an independent tile and/or a slice, the stored CABAC models is not required to be loaded. As demonstrated in Figures 1 and 2, the memorization processes invoked before the LCU with marker “R” is redundant.Working Draft 5 and HM5.0 reference software have this redundant memorization processes

R

entropy_coding_synchro+1 LCUs

Tile boundary

Before coding the blue LCU, memorization process is invoked.

R

R

R

Figure 1 Example of CABAC memorization process, when tile_boundary_independence_flag = 1

R

entropy_coding_synchro+1 LCUs

Tile boundary

Before coding the blue LCU, memorization process is invoked.

Figure 2 Example of CABAC memorization process, when tile_boundary_independence_flag = 0

# Proposed solution

It is proposed to change the description in Working Draft by adding a condition to avoid redundant invoking of memorization process. The modification for the Working Draft text is as follows,

- Otherwise, if (tile_boundary_independence_flag is equal to 1, ( tbAddr − firstTbInTileAddr ) / tileWidthInLCUs is equal to tileHeightInLCUs – 1 and ( tbAddr − firstTbInTileAddr ) % tileWidthInLCUs is equal to entropy_coding_synchro + 1) or (tile_boundary_independence_flag is equal to 0 , tbAddr / picWidthInLCUs is equal to picHeightInLCUs – 1 and tbAddr % picWidthInLCUs is equal to entropy_coding_synchro + 1), the memorization process of the CABAC parsing process is invoked as specified in subclause Error: Reference source not found.

The version of clean-up software on HM5.0 is provided as the attachment.

# Patent rights declaration(s)

Intel Corp. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).