| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 8th Meeting: San José, CA, USA, 1–10 February, 2012 | Document: JCTVC-H0250 |
| --- | --- |

| Title: | Non CE9: Removal of non-scaled merge candidate |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Oguz Bici Jani Lainema Kemal Ugur | Email: | ext-oguz.bici@nokia.com jani.lainema@nokia.com kemal.ugur@nokia.com |
| Source: | Nokia |  |  |

_____________________________

# Abstract

Currently in HM5, the merge mode initially generates spatial and temporal candidates and after removing redundant candidates, additional candidates are generated to achieve coding gain and a desired number of candidates. One of the tools to generate additional candidates is the ‘non-scaled’ candidate which is obtained for B slices by inverting sign of motion vector. In this proposal, it is claimed that the coding gain associated with the tool is negligible and it is proposed to completely remove this tool. The reported compression efficiency impact in terms of BD rate is 0.0%, 0.1%, 0.1%, 0.0% and 0.0% for RA_HE, RA_LC, RA_HE_10, LB_HE and LB_LC, respectively.

# Introduction

Currently in HM5, the merge mode initially generates spatial and temporal candidates and after removing redundant candidates, additional candidates are generated to achieve coding gain and a desired number of candidates. One of the tools to generate additional candidates is the ‘non-scaled’ candidate which is obtained for B slices by inverting sign of motion vector.

# Proposal

In this contribution, as a simplification, it is proposed to completely remove the ‘non-scaled merge candidate’ as the coding gain associated with the tool is very limited as reported in the next section.

# Results

HM5.0 software is used for the implementation of the proposal and the results are compared against HM5.0 as the anchor under the common test conditions and reference configurations specified in [1]. The results of the compression efficiency tests are presented in Table 1. Note that encoding times may include some variations due to heterogeneous resources and workloads of the nodes in the simulation cluster.

Table 1

|  | Random Access HE |  |  | Random Access LC |  |  | Random Access HE-10 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A (8bit) | 0.0% | 0.1% | 0.0% | 0.0% | 0.1% | 0.1% | 0.1% | 0.1% | 0.2% |
| Class B | 0.0% | 0.0% | 0.1% | 0.1% | 0.1% | 0.1% | 0.1% | 0.1% | 0.1% |
| Class C | 0.0% | 0.1% | 0.1% | 0.1% | 0.1% | 0.1% |  |  |  |
| Class D | 0.1% | -0.1% | 0.0% | 0.1% | 0.1% | 0.2% |  |  |  |
| Class E |  |  |  |  |  |  |  |  |  |
| Overall | 0.0% | 0.0% | 0.1% | 0.1% | 0.1% | 0.1% | 0.1% | 0.1% | 0.1% |
|  | 0.0% | 0.0% | 0.1% | 0.1% | 0.1% | 0.1% | 0.1% | 0.1% | 0.1% |
| Class F | 0.0% | 0.0% | 0.0% | 0.1% | 0.1% | 0.1% |  |  |  |
| Enc Time[%] | 99% |  |  | 99% |  |  | 99% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  | 100% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  | Low delay B HE-10 |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |  |  |  |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| Class C | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| Class D | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| Class E | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| Overall | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
|  | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| Class F | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| Enc Time[%] | 100% |  |  | 101% |  |  |  |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |  |  |  |

# Conclusions

This contribution has presented results of completely removing the ‘non-scaled merge candidate’ in merge of HM5. The reported compression efficiency impact in terms of BD rate is 0.0%, 0.1%, 0.1%, 0.0% and 0.0% for RA_HE, RA_LC, RA_HE_10, LB_HE and LB_LC, respectively. As the resultant coding gain loss is very limited, it is recommended to adopt the proposed changes in the next HM and WD.

# WD Changes

The WD changes consist of only removing text. Two sections, 8.4.2.1.1 and 8.4.2.1.4 from JCTVC-G1103_d5, are affected. The removed portions of the text are highlighted with yellow.

##### 8.4.2.1.1 Derivation process for luma motion vectors for merge mode

This process is only invoked when PredMode is equal to MODE_SKIP or PredMode is equal to MODE_ INTER and merge_flag [ xP ][ yP ] is equal to 1, where ( xP, yP ) specify the top-left luma sample of the current prediction unit relative to the top-left luma sample of the current picture.

Inputs of this process are

- a luma location ( xP, yP ) of the top-left luma sample of the current prediction unit relative to the top-left luma sample of the current picture,

- variables specifying the width and the height of the prediction unit for luma, nPSW and nPSH,

- a variable PartIdx specifying the index of the current prediction unit within the current coding unit.

Outputs of this process are

–	the luma motion vectors mvL0 and mvL1,

–	the reference indices refIdxL0 and refIdxL1,

–	the prediction list utilization flags predFlagL0 and predFlagL1.

The motion vectors mvL0 and mvL1, the reference indices refIdxL0 and refIdxL1, and the prediction utilization flags predFlagL0 and predFlagL1 are derived as specified by the following ordered steps:

- The derivation process for merging candidates from neighboring prediction unit partitions in subclause Error: Reference source not found is invoked with luma location ( xP, yP ), the width and the height of the prediction unit nPSW and nPSH and the partition index PartIdx as inputs and the output is assigned to the availability flags availableFlagN, the reference indices refIdxL0N and refIdxL1N, the prediction list utilization flags predFlagL0N and predFlagL1N and the motion vectors mvL0N and mvL1N with N being replaced by A0, A1, B0, B1 or B2.

- The reference index for temporal merging candidate is derived as follows..

- If the prediction unit covering luma location ( xP − 1, yP + nPSH − 1 ) is available and PredMode is not MODE_INTRA, refIdxLX is set equal to refIdxLX[ xP − 1, yP + nPSH − 1 ].

- Otherwise, refIdxLX is set equal to 0.

- The derivation process for temporal luma motion vector prediction in subclause Error: Reference source not found is invoked with luma location ( xP, yP ), refIdxLX as the inputs and with the output being the availability flag availableFlagLXCol and the temporal motion vector mvLXCol. The variables availableFlagCol and predFlagLXCol (with X being 0 or 1, respectively) are derived as specified below.

availableFlagCol = availableFlagL0Col || availableFlagL1Col	(8-1)

predFlagLXCol = availableFlagLXCol	(8-2)

- The merging candidate list, mergeCandList, is constructed as follows.

  - A1, if availableFlagA1 is equal to 1

  - B1, if availableFlagB1 is equal to 1

  - B0, if availableFlagB0 is equal to 1

  - A0, if availableFlagA0 is equal to 1

  - B2, if availableFlagB2 is equal to 1

  - Col, if availableFlagCol is equal to 1

- When merging candidates have the same motion vectors and the same reference indices, the merging candidates are removed from the list except the merging candidate which has the smallest order in the mergeCandList.

- The variable numMergeCand and numOrigMergeCand are set to the number of merging candidates in the mergeCandList.

- When slice_type is equal to B, the following applies.

  - The derivation process for combined bi-predictive merging candidates specified in subclause Error: Reference source not found is invoked with mergeCandList, the reference indices refIdxL0N and refIdxL1N, the prediction list utilization flags predFlagL0N and predFlagL1N, the motion vectors mvL0N and mvL1N of every candidate N being in mergeCandList, numMergeCand and numOrigMergeCand given as input and the output is assigned to mergeCandList, numMergeCand, the reference indices refIdxL0combCandk and refIdxL1combCandk, the prediction list utilization flags predFlagL0combCandk and predFlagL1combCandk and the motion vectors mvL0combCandk and mvL1combCandk of every new candidate combCandk being added in mergeCandList. The number of candidates being added numCombMergeCand is set equal to ( numMergeCand – numOrigMergeCand ). When numCombMergeCand is greater than 0, k ranges from 0 to numCombMergeCand − 1, inclusive.

  - The derivation process for non-scaled bi-predictive merging candidates specified in subclause Error: Reference source not found is invoked with mergeCandList, the reference indices refIdxL0N and refIdxL1N, the prediction list utilization flags predFlagL0N and predFlagL1N, the motion vectors mvL0N and mvL1N of every candidate N being in mergeCandList, numMergeCand and numOrigMergeCand given as input and the output is assigned to mergeCandList, numMergeCand, the reference indices refIdxL0nscaleCandl and refIdxL1nscaleCandl, the prediction list utilization flags predFlagL0nscaleCandl and predFlagL1nscaleCandl, the motion vectors mvL0nscaleCandl and mvL1nscaleCandl of every new candidate nscaleCandl being added in mergeCandList. The number of candidates being added numNscaleMergeCand is set equal to ( numMergeCand – numOrigMergeCand – numCombMergeCand ). When numNscaleMergeCand is greater than 0, l ranges from 0 to numNscaleMergeCand – 1, inclusive.

- The derivation process for zero motion vector merging candidates specified in subclause Error: Reference source not found is invoked with the mergeCandList, the reference indices refIdxL0N and refIdxL1N, the prediction list utilization flags predFlagL0N and predFlagL1N, the motion vectors mvL0N and mvL1N of every candidate N being in mergeCandList and the NumMergeCand as the inputs and the output is assigned to mergeCandList, numMergeCand, the reference indices refIdxL0zeroCandm and refIdxL1zeroCandm, the prediction list utilization flags predFlagL0zeroCandm and predFlagL1zeroCandm, the motion vectors mvL0zeroCandm and mvL1zeroCandm of every new candidate zeroCandm being added in mergeCandList. The number of candidates being added numZeroMergeCand is set equal to ( numMergeCand – numOrigMergeCand – numCombMergeCand – numNscaleMergeCand ). When numZeroMergeCand is greater than 0, m ranges from 0 to numZeroMergeCand – 1, inclusive.

- The following assignments are made with N being the candidate at position merge_idx[ xP][ yP ] in the merging candidate list mergeCandList ( N = mergeCandList[ merge_idx[ xP][ yP ] ] ) and X being replaced by 0 or 1:

mvLX[ 0 ] = mvLXN[ 0 ]	(8-3)

mvLX[ 1 ] = mvLXN[ 1 ]	(8-4)

refIdxLX = refIdxLXN	(8-5)

predFlagLX = predFlagLXN

##### 8.4.2.1.4 Derivation process for non-scaled bi-predictive merging candidates

Inputs of this process are

- a merging candidate list mergeCandList,

- reference indices refIdxL0N and refIdxL1N of every candidate N being in mergeCandList,

- prediction list utilization flags predFlagL0N and predFlagL1N of every candidate N being in mergeCandList,

- motion vectors mvL0N and mvL1N of every candidate N being in mergeCandList,

- the number of elements numMergeCand within mergeCandList,

- the number of elements numOrigMergeCand within the mergeCandList after the spatial and temporal merge candidate derivation process,

Outputs of this process are

- the merging candidate list mergeCandList,

- the number of elements numMergeCand within mergeCandList.

- reference indices refIdxL0nscaleCandl and refIdxL10nscaleCandl of every new candidate nscaleCandl being added in mergeCandList during the invokation of this process,

- prediction list utilization flags predFlagL0nscaleCandl and predFlagL10nscaleCandl of every new candidate nscaleCandl being added in mergeCandList during the invokation of this process,

- motion vectors mvL0nscaleCandl and mvL10nscaleCandl of every new candidate nscaleCandl being added in mergeCandList during the invokation of this process,

The function RefPicOrderCnt( refidx, LX ) specifies the value of PicOrderCnt of the picture that is the reference picture RefPicListX[ refidx ] of the current picture with X being 0 or 1. PicOrderCnt of the reference picture shall be maintained until the picture is marked as “non-existing”. The variable PicOrderCnt specifies the PicOrderCnt of the current picture.

[Ed. (BB): PicOrderCnt(.) needs to be defined. Currently the Decoding process for picture order count is not described]

When numOrigMergeCand is greater than 0 and numMergeCand is less than MaxNumMergeCand, the variable numInputMergeCand is set to numMergeCand, the variables origIdx and nscaleCnt are set to 0, the variable nscaleStop is set to false and the following steps are repeated until nscaleStop is equal to true.

- The following assignments are made with origCand being the candidate at position origIdx in the merging candidate list mergeCandList ( origCand = mergeCandList[ origIdx ] ).

- The following is applied one time with X being 0 and Y being 1. When nscaleCnt is less than 1 and numMergeCand is less than MaxNumMergeCand, it is applied a second time with X being 1 and Y being 0.

    - When all of the following conditions are true,

      - refIdxLXorigCand >= 0

      - RefPicOrderCnt( refIdxLXorigCand, LX ) != RefPicOrderCnt( refIdxLXorigCand, LY )

      - abs( PicOrderCnt − RefPicOrderCnt( refIdxLXorigCand, LX ) ) = = 
abs( PicOrderCnt − RefPicOrderCnt( refIdxLXorigCand, LY ) )

      - refIdxLXorigCand < num_ref_idx_lY_active_minus1 + 1

the following applies.

      - The candidate nscaleCandl with l equal to ( numMergeCand − numInputMergeCand )is added at the end of mergeCandList ( mergeCandList[ numMergeCand ] = nscaleCandl ) and the reference indices, the prediction list utilization flags and the motion vectors of nscaleCandl are dervied as follows and numMergeCand is incremented by 1.

refIdxL0nscaleCandl = refIdxLXorigCand		(8-6)

refIdxL1nscaleCandl = refIdxLXorigCand		(8-7)

predFlagL0nscaleCandl = 1		(8-8)

predFlagL1nscaleCandl = 1		(8-9)

mvL0nscaleCandl[ 0 ] = mvLXorigCand[ 0 ] 		(8-10)

mvL0nscaleCandl[ 1 ] = mvLXorigCand[ 1 ] 		(8-11)

mvL1nscaleCandl[ 0 ] = − mvLXorigCand[ 0 ] 		(8-12)

mvL1nscaleCandl[ 1 ] = − mvLXorigCand[ 1 ] 		(8-13)

numMergeCand = numMergeCand + 1	(8-14)

      - The variable nscaleCnt is incremented by 1.

- The variable origIdx is incremented by 1.

When origIdx is equal to numOrigMergeCand or numMergeCand is equal to MaxNumMergeCand or nscaleCnt is equal to 1, nscaleStop is set to true.

# References

[1] F. Bossen, "Common test conditions and software reference configurations," JCTVC-G1200, Nov 2011

# Patent rights declaration(s)

Nokia may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).