| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 8th Meeting: San José, CA, USA, 1–10 February, 2012 | Document: JCTVC-H0251 |
| --- | --- |

| Title: | Context reduction for merge index coding |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Oguz Bici Jani Lainema Kemal Ugur | Email: | ext-oguz.bici@nokia.com jani.lainema@nokia.com kemal.ugur@nokia.com |
| Source: | Nokia |  |  |

_____________________________

# Abstract

In this contribution, it is proposed to use single context for merge index coding by CABAC. Merge index consists of four bins and HM5 assigns one context for each bin, totalling 4 contexts. In the proposed method, one context is used for the first bin and remaining three bins are coded in by-pass mode. The reported compression efficiency impact in terms of BD rate is 0.0% for all the RA_HE, RA_LC, RA_HE_10, LB_HE and LB_LC configurations.

# Introduction and Proposal

Merge index consists of four bins and HM5 assigns one context for each bin, totalling 4 contexts.  In the proposed method, one context is used for the first bin and remaining three bins are coded in by-pass mode. The idea is illustrated in Figure 1.

Figure 1 Illustration of the proposal

# Results

HM5.0 software is used for the implementation of the proposal and the results are compared against HM5.0 as the anchor under the common test conditions and reference configurations specified in [1]. The results of the compression efficiency tests are presented in Table 1. Note that encoding times may include some variations due to heterogeneous resources and workloads of the nodes in the simulation cluster.

Table 1

|  | Random Access HE |  |  | Random Access LC |  |  | Random Access HE-10 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A (8bit) | 0.0% | 0.1% | -0.1% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% |
| Class B | 0.0% | 0.0% | 0.0% | 0.1% | 0.1% | 0.0% | 0.1% | 0.0% | 0.0% |
| Class C | 0.0% | 0.1% | 0.1% | 0.1% | 0.0% | 0.1% |  |  |  |
| Class D | 0.0% | -0.2% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| Class E |  |  |  |  |  |  |  |  |  |
| Overall | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class F | 0.0% | 0.0% | 0.0% | 0.1% | 0.1% | 0.1% |  |  |  |
| Enc Time[%] | 99% |  |  | 101% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  | 100% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  | Low delay B HE-10 |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |  |  |  |
| Class B | 0.0% | 0.0% | 0.1% | 0.0% | -0.1% | 0.2% |  |  |  |
| Class C | 0.0% | -0.1% | 0.0% | 0.0% | 0.2% | -0.1% |  |  |  |
| Class D | 0.0% | -0.1% | 0.2% | -0.1% | 0.2% | 0.2% |  |  |  |
| Class E | 0.0% | -0.3% | -0.2% | 0.0% | -0.5% | 0.3% |  |  |  |
| Overall | 0.0% | -0.1% | 0.0% | 0.0% | 0.0% | 0.1% |  |  |  |
|  | 0.0% | -0.1% | 0.0% | 0.0% | 0.0% | 0.1% |  |  |  |
| Class F | -0.1% | -0.1% | -0.4% | -0.1% | -0.3% | -0.4% |  |  |  |
| Enc Time[%] | 100% |  |  | 102% |  |  |  |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |  |  |  |

# Conclusions

This contribution has presented results of reducing number of merge index coding contexts from 4 to 1. The reported compression efficiency impact in terms of BD rate is 0.0% for all the RA_HE, RA_LC, RA_HE_10, LB_HE and LB_LC configurations. As there is no impact on the coding efficiency, it is recommended to adopt the proposed changes in the next HM and WD.

# WD Changes

Table 9-1 – Association of ctxIdx and syntax elements for each slice type in the initialisation process

|  | Syntax element | ctxIdxTable | Slice Type |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | I | P | B |
| prediction_unit() | prev_intra_luma_pred_flag | Error: Reference source not found | 0 | 1 | 2 |
|  | merge_flag | Error: Reference source not found |  | 0 | 1 |
|  | merge_idx | Error: Reference source not found |  | 0..3 | 1 4..7 |
|  | inter_pred_flag | Error: Reference source not found |  |  | 0..3 |
|  | ref_idx_lc, ref_idx_l0, ref_idx_l1 | Error: Reference source not found |  | 0..2 | 3..5 |
|  | abs_mvd_greater0_flag | Error: Reference source not found |  | 0 | 1 |
|  | abs_mvd_greater1_flag | Error: Reference source not found |  | 2 | 3 |
|  | mvp_lc_flag, mvp_l0_flag, mvp_l1_flag | Error: Reference source not found |  | 0 | 1 |

Table 9-2 – Values of variable initValue for merge_idx ctxIdx

| Initialisation variable | merge_idx ctxIdx |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| initValue | 100 | 116 86 | 102 | 133 | 116 | 87 | 119 | 103 |

| Table 9-3 – Assignment of ctxIdxInc to binIdx for all ctxIdxTable and ctxIdxOffset values |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Syntax element | ctxIdxTable, ctxIdxOffset |  | binIdx |  |  |  |  |
|  |  |  | 0 | 1 | 2 | 3 | >=4 |
| merge_idx | Error: Reference source not found | 0 | 0 0,1,2,3 (subclause ) | na 0,1,2,3 (subclause ) | na 0,2,3 (subclause ) | na 3 | na |
|  |  | 4 | 0 0,1,2,3 (subclause ) | na 0,1,2,3 (subclause ) | na 0,2,3 (subclause ) | na 3 | na |

###### 9.2.3.1.1.2 Derivation process of ctxIdxInc for the syntax element merge_index

Input to this process is binIdx.

Output of this process is ctxIdxInc.

The variable ctxIdxInc is derived as follows.

- If binIdx is equal to 0, the following applies.

- If availableLeft is equal to 1and availableAbove is equal to 1, ctxIdxInc is set equal to 0.

- Otherwise if availableLeft is equal to 1 or availableAbove is equal to 1, ctxIdxInc is set equal to 2.

- Otherwise (availableLeft is equal to 0 and availableAbove is equal to 0), ctxIdxInc is set equal to 3.

- Otherwise if binIdx is equal to 1 and NumMergeCand is greater than 2, the following applies.

- If availableAbove is equal to 1, ctxIdxInc is set equal to 2.

- Otherwise (availableAbove is equal to 0), ctxIdxInc is set equal to 3.

- Otherwise if binIdx is equal to 2 and NumMergeCand is greater than 3, ctxIdxInc is set equal to 3.

- Otherwise, ctxIdxInc is set equal to 0.

When ctxIdxInc is not equal to 0, ctxIdxInc is derived as follows.

ctxIdxInc  =  ctxIdxInc − availableCollocated		(9-1)

[Ed. (BB): The flags availableLeft, availableAbove, availableCollocated need to be defined or a simpler context derivation should be used.]

# References

[1] F. Bossen, "Common test conditions and software reference configurations," JCTVC-G1200, Nov 2011

# Patent rights declaration(s)

Nokia may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).