| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 8th Meeting: San José, CA, USA, 1–10 February, 2012 | Document: JCTVC-H0255_r3 |
| --- | --- |

| Title: | On partial updating of APS parameters |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Akira Minezawa Kazuo Sugimoto Shun-ichi Sekiguchi Information Technology R&D Center Mitsubishi Electric Corporation 5-1-1, Ofuna, Kamakura, JAPAN | Tel: Email: | +81-467-41-2463 Minezawa.Akira@ds.MitsubishiElectric.co.jp |
| Source: | Mitsubishi Electric Corporation |  |  |

_____________________________

# Abstract

In this contribution, two partial updating schemes of adaptation parameter set (APS) parameters are proposed. The first proposed scheme permits to have multiple aps_ids in a slice header and APS parameters associated with a slice are partially overridden by the referred multiple APSs. The second method has an aps_id and partial APS updating information in a slice header, and APS parameters indicated by the referred aps_id are overridden by the partial APS updating information. An aps_id overriding scheme is also proposed.

# Introduction

In JCTVC-G1026[1], a multiple APSs referring approach is proposed to save bits from coding repeated APS parameters. In this contribution, we propose two different overriding schemes of APS parameters in order to efficiently apply more flexible combination of APS parameters for each slice.

# Proposed scheme

## Method 1

This method proposes a modification scheme of JCTVC-G1026 in order to be able to partially override APS parameters for each slice.

This method allows a slice to refer to multiple APSs. The decoder parses multiple referred APSs to get APS parameters, and then the final APS parameters for the current slice is obtained by sequentially overriding previously referred APS parameters as illustrated in Figure 1. Therefore, a slice in Picture 1 activates combined ALF parameters to override ALF(0) by some existing filters in ALF(1) as depicted in  Figure 2.

Figure 1. Example of slice layer multiple APSs referring

Figure 2. Example of overriding ALF parameters

The summery of proposed syntax and semantics modifications are listed in following Table 1.

Table 1  Modifications Summary

| APS | Adding new elements signaling whether each adaptive filter set are presented and whether each quantization matrix are presented in this APS; |
| --- | --- |
| Slice Header | Adding new syntax elements signaling multiple APSs’ IDs and permitting to partially override APS parameters. |

The modified scaling list data syntax based on Working Draft 5 [2] is presented in Table 2, where the added syntax elements are highlighted in yellow bars.

Table 2  Modified scaling list data syntax

| scaling_list_param( ) { | Descriptor |
| --- | --- |
| scaling_list_present_flag | u(1) |
| if( scaling_list_present_flag ) |  |
| for( SizeID = 0; SizeID < 4; SizeID++ ) |  |
| for( MatrixID = 0; MatrixID < (SizeID = = 3) ? 2 : 6; MatrixID++ ) { |  |
| scaling_list_sub_present_flag[SizeID][MatrixID] | u(1) |
| if(scaling_list_sub_present_flag[SizeID][MatrixID]) |  |
| { |  |
| scaling_list_pred_mode_flag | u(1) |
| if( !scaling_list_pred_mode_flag ) |  |
| scaling_list_pred_matrix_id_delta | ue(v) |
| else |  |
| scaling_list( ScalingList[ SizeID ][ MatrixID ][ 0 ], ( 1 << ( 4 + ( sizeID << 1) ) ) ) |  |
| } |  |
| } |  |
| } |  |

The modified adaptive loop filter parameter syntax is presented in Table 3, where the added syntax elements are highlighted in yellow bars.

Table 3  Modified adaptive loop filter parameter syntax

| alf_param() { | Descriptor |
| --- | --- |
| alf_region_adaptation_flag | u(1) |
| alf_length_luma_minus_5_div2 | ue(v) |
| alf_no_filters_minus1 | ue(v) |
| if (alf_no_filters_minus1 == 1) |  |
| alf_start_second_filter | ue(v) |
| else if (alf_no_filters_minus1 > 1) { |  |
| for (i=1; i< (alf_region_adaptation_flag ? 16 : 15); i++) |  |
| alf_filter_pattern[i] | u(1) |
| alf_filter_present[i] | u(1) |
| } |  |
| if (AlfNumFilters > 1) |  |
| alf_pred_method | u(1) |
| for (i=0; i< AlfNumFilters; i++ ) |  |
| alf_nb_pred_luma[i] | u(1) |
| for (i=0; i< AlfNumFilters; i++) |  |
| for (j=0; j< AlfCodedLengthLuma; j++) |  |
| alf_coeff_luma[i][j] | ge(v) |
| alf_chroma_idc | ue(v) |
| if ( alf_chroma_idc ) { |  |
| alf_length_chroma_minus_5_div2 | ue(v) |
| for( i = 0; i< AlfCodedLengthChroma; i++ ) |  |
| alf_coeff_chroma[i] | se(v) |
| } |  |
| } |  |

The modified slice header is presented in Table 4, where the added syntax elements are highlighted in yellow bars.

Table 4  Modified slice header syntax

| slice_header( ) { |  | Descriptor |
| --- | --- | --- |
| first_slice_in_pic_flag |  | u(1) |
| if( first_slice_in_pic_flag = = 0 ) |  |  |
| slice_address |  | u(v) |
| slice_type |  | ue(v) |
| entropy_slice_flag |  | u(1) |
| if( !entropy_slice_flag ) { |  |  |
| … |  |  |
| if( scaling_list_enable_flag ¦ ¦ deblocking_filter_in_APS_enabled_flag ¦ ¦ sample_adaptive_offset_enabled_flag ¦ ¦ adaptive_loop_filter_enabled_flag ) { |  |  |
| if( sample_adaptive_offset_enabled_flag ) |  |  |
| slice_sample_adaptive_offset_flag |  | u(1) |
| if( adaptive_loop_filter_enabled_flag) |  |  |
| slice_adaptive_loop_filter_flag |  | u(1) |
| number_of_valid_aps | u(v) |  |
| if (number_of_valid_aps != 0 ) { |  |  |
| for (i = 0; i < number_of_valid_aps; i++) { |  |  |
| aps_id[i] | ue(v) |  |
| } |  |  |
| } |  |  |
| } |  |  |
| … |  |  |
| } |  |  |
| … |  |  |
| } |  |  |

The semantics of the new element in scaling list data syntax is presented below.

scaling_list_sub_present_flag[SizeID][MatrixID] equal to 1 specifies that the Quantization matrix parameters where an index of transform block size is equal to SizeID and an matrix type index is equal to MatrixID exist in this APS, equal to 0 specifies that the Quantization matrix parameters where an index of transform block size is equal to SizeID and an matrix type index is equal to MatrixID do not exist in this APS.

The semantics of the new element in adaptive loop filter parameter syntax is presented below.

alf_filter_present[ i ] specifies the filter index array corresponding to i-th variance index of luma samples, when alf_no_filters_minus1 is greater than 1. alf_filter_present[ i ] equal to 1 indicates the existence of filter coefficient i-th variance index of luma samples. The number of filter sets AlfNumFilters is derived as follows:

- If alf_no_filters_minus1 is less than 2,

AlfNumFilters = alf_no_filters_minus1 + 1		(7-16)

- Otherwise (alf_no_filters_minus1 is greater than 2)

AlfNumFilters = i alf_filter_pattern[ i ] with i = 0..15	(7-16)

The semantics of the new elements in slice header syntax are presented as below.

number_of_valid_aps specifies the number of valid APSs in decoding this slice. If number_of_valid_aps does not exist, its value is inferred to be 0. If number_of_valid_aps is equal to 0, it is inferred as no active APSs.

aps_id[i] specifies the IDs of i-th referred APSs in decoding this slice, where i shall be in the range of [0, number_of_valid_aps-1], inclusively. aps_id[0] indicates the ID of the base APS and the others indicate the IDs of the APSs overriding the base APS,

## Method 2

This method allows inserting APS updating parameters in a slice header in order to partially override APS parameters. APS parameters indicated by a referred aps_id in a slice header are overridden by the updating parameters for using the current slice decoding process.

The summery of proposed syntax and semantics modifications are listed as follows.

Table 5  Modifications Summary

| APS | Adding new elements signaling whether each adaptive filter set are presented and whether each quantization matrix are presented in this APS; |
| --- | --- |
| Slice Header | Adding new syntax elements signaling APS updating parameters and permitting to partially override APS parameters. |

The modified scaling list data syntax is presented in Table 6, where the added syntax elements are highlighted in yellow bars.

Table 6  Modified scaling list data syntax

| scaling_list_param( ) { | Descriptor |
| --- | --- |
| scaling_list_present_flag | u(1) |
| if( scaling_list_present_flag ) |  |
| for( SizeID = 0; SizeID < 4; SizeID++ ) |  |
| for( MatrixID = 0; MatrixID < (SizeID = = 3) ? 2 : 6; MatrixID++ ) { |  |
| scaling_list_sub_present_flag[SizeID][MatrixID] | u(1) |
| if(scaling_list_sub_present_flag[SizeID][MatrixID]) |  |
| { |  |
| scaling_list_pred_mode_flag | u(1) |
| if( !scaling_list_pred_mode_flag ) |  |
| scaling_list_pred_matrix_id_delta | ue(v) |
| else |  |
| scaling_list( ScalingList[ SizeID ][ MatrixID ][ 0 ], ( 1 << ( 4 + ( sizeID << 1) ) ) ) |  |
| } |  |
| } |  |
| } |  |

The modified adaptive loop filter parameter syntax is presented in Table 7, where the added syntax elements are highlighted in yellow bars.

Table 7  Modified adaptive loop filter parameter syntax

| alf_param() { | Descriptor |
| --- | --- |
| alf_region_adaptation_flag | u(1) |
| alf_length_luma_minus_5_div2 | ue(v) |
| alf_no_filters_minus1 | ue(v) |
| if (alf_no_filters_minus1 == 1) |  |
| alf_start_second_filter | ue(v) |
| else if (alf_no_filters_minus1 > 1) { |  |
| for (i=1; i< (alf_region_adaptation_flag ? 16 : 15); i++) |  |
| alf_filter_pattern[i] | u(1) |
| alf_filter_present[i] | u(1) |
| } |  |
| if (AlfNumFilters > 1) |  |
| alf_pred_method | u(1) |
| for (i=0; i< AlfNumFilters; i++ ) |  |
| alf_nb_pred_luma[i] | u(1) |
| for (i=0; i< AlfNumFilters; i++) |  |
| for (j=0; j< AlfCodedLengthLuma; j++) |  |
| alf_coeff_luma[i][j] | ge(v) |
| alf_chroma_idc | ue(v) |
| if ( alf_chroma_idc ) { |  |
| alf_length_chroma_minus_5_div2 | ue(v) |
| for( i = 0; i< AlfCodedLengthChroma; i++ ) |  |
| alf_coeff_chroma[i] | se(v) |
| } |  |
| } |  |

The modified slice header is presented in Table 8, where the added syntax elements are highlighted in yellow bars.

Table 8  Modified slice header syntax

| slice_header( ) { | Descriptor |
| --- | --- |
| first_slice_in_pic_flag | u(1) |
| if( first_slice_in_pic_flag = = 0 ) |  |
| slice_address | u(v) |
| slice_type | ue(v) |
| entropy_slice_flag | u(1) |
| if( !entropy_slice_flag ) { |  |
| … |  |
| if( scaling_list_enable_flag ¦ ¦ deblocking_filter_in_APS_enabled_flag ¦ ¦ sample_adaptive_offset_enabled_flag ¦ ¦ adaptive_loop_filter_enabled_flag ) { |  |
| if( sample_adaptive_offset_enabled_flag ) |  |
| slice_sample_adaptive_offset_flag | u(1) |
| if( adaptive_loop_filter_enabled_flag) |  |
| slice_adaptive_loop_filter_flag | u(1) |
| aps_id | ue(v) |
| aps_updating_parameter_enable_flag | u(1) |
| if(aps_updating_parameter_enable_flag) { |  |
| updating_scaling_list_data_present_flag | u(1) |
| updating_sample_adaptive_offset_flag | u(1) |
| updating_adaptive_loop_filter_flag | u(1) |
| if(updating_scaling_list_data_present_flag ) |  |
| scaling_list_param( ) |  |
| if(updating_sample_adaptive_offset_flag ) |  |
| sao_param( ) |  |
| if(updating_adaptive_loop_filter_flag ) |  |
| alf_param( ) |  |
| } |  |
| } |  |
| … |  |
| } |  |
| … |  |
| } |  |

The semantics of the new element in scaling list data syntax is presented below.

scaling_list_sub_present_flag[SizeID][MatrixID] equal to 1 specifies that the Quantization matrix parameters where an index of transform block size is equal to SizeID and an matrix type index is equal to MatrixID exist in this APS, equal to 0 specifies that the Quantization matrix parameters where an index of transform block size is equal to SizeID and an matrix type index is equal to MatrixID do not exist in this APS.

The semantics of the new element in adaptive loop filter parameter syntax is presented below.

alf_filter_present[ i ] specifies the filter index array corresponding to i-th variance index of luma samples, when alf_no_filters_minus1 is greater than 1. alf_filter_present[ i ] equal to 1 indicates the existence of filter coefficient i-th variance index of luma samples. The number of filter sets AlfNumFilters is derived as follows:

- If alf_no_filters_minus1 is less than 2,

AlfNumFilters = alf_no_filters_minus1 + 1		(7-16)

- Otherwise (alf_no_filters_minus1 is greater than 2)

AlfNumFilters = i alf_filter_pattern[ i ] with i = 0..15	(7-16)

The semantics of the new elements in slice header syntax are presented as below.

aps_updating_parameter_enable_flag equal to 1 specifies that the APS updating parameters exist in this slice, equal to 0 specifies that the APS updating parameters do not exist in this slice.

updating_scaling_list_data_present_flag equal to 1 specifies that the updating scaling list parameters exist in this slice, equal to 0 specifies that updating scaling list parameters do not exist in this slice.

updating_sample_adaptive_offset_flag equal to 1 specifies that the sample adaptive offset parameters exist in this slice, equal to 0 specifies that the sample adaptive offset parameters do not exist in this slice.

updating_adaptive_loop_filter_flag equal to 1 specifies that the adaptive loop filter parameters exist in this slice, equal to 0 specifies that the adaptive loop filter parameters do not exist in this slice.

# Reused aps_id scheme

We also propose to permit to reuse aps_id for saving the number of aps_id in a bitstream as shown in  Figure 3.

Figure 3. Example of reusing aps_id

# Conclusions

This contribution proposes two partial updating schemes of adaptation parameter set (APS) parameters. The first proposed scheme permits to have multiple aps_ids in a slice header and APS parameter associated with a slice is partially overridden by referring multiple aps_ids. The second scheme has an aps_id and partial APS updating information in a slice header, and APS parameters indicated by the referred aps_id are overridden by the partial APS updating information. An aps_id overriding scheme is also proposed. We propose these schemes to be adopted to HM-6.

# References

- Ming LI, “Using Multiple APSs for Quantization Matrix Parameters Signaling”, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-G1026, 7th Meeting: Geneva, CH, November, 2011.

- Benjamin Bross, “WD5: Working Draft 5 of High-Efficiency Video Coding”, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-G1103, 7th Meeting: Geneva, CH, November, 2011.

# Patent rights declaration(s)

Mitsubishi Electric Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).