| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 8th Meeting: San José, CA, USA, 1–10 February, 2012 | Document: JCTVC-H0257 |
| --- | --- |

| Title: | Cyclic POC |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Junpei Koyama, Kimihiko Kazui, Satoshi Shimada and Akira Nakagawa 4-1-1 Kamikodanaka, Nakahara-ku 211-8588 Kawasaki JAPAN | Tel: Email: | +81-44-754-2639 koyama.junpei@jp.fujitsu.com kazui.kimihiko@jp.fujitsu.com sshimada@jp.fujitsu.com anaka@jp.fujitsu.com |
| Source: | FUJITSU LABORATORIES LTD. |  |  |

_____________________________

# Abstract

This contribution proposes to change the definition of picture order count (POC) to a cyclic pattern. The range of POC value remains 32-bit and no syntax change is required.

# Problem Statement

## Current POC definition

In WD5/HM5[1], the derivation process of a POC value is same as that of “pic_order_cnt_type == 0” case in AVC.

The derivation process of “pic_order_cnt_type == 0” has the following features.

- The least significant bit (LSB) of a POC value of each picture is sent by syntax pic_order_cnt_lsb in a slice header. Its size is (log2_max_pic_order_cnt_lsb_minus_4 + 4) bit.

- A Decoder derives the POC value of each picture from three values. Syntax element pic_order_cnt_lsb, the LSB of the POC value of a previously decoded picture (prevPicOrderCntLsb) and the most significant bit (MSB) the a POC value of the previously decoded picture (prevPicOrderCntMsb). When a decoding process starts from a recovery point rather than from an IDR picture, the POC values of a picture at encoding side and that at decoding side may be different.

- The range of a POC value is[−231, 231 – 1].

- All pictures in one sequence shall not have the same POC value.

- The range of POC difference between a current picture and its reference pictures shall be [−215 , 215 – 1].

Especially, the fourth feature implies that an IDR interval is restricted to lower than 231. It is led that an encoder needs to insert an IDR picture before reaching the maximum POC value (i.e. 231-1).

## Problems

The AVC standard, from which the POC derivation process of current HEVC draft is derived, is widely used not only for consumer equipments such as cam-coders, but also for remote surveillance cameras used in public infrastructure systems such as for live monitoring of a river. These remote surveillance cameras are typically required low delay video transmission and 24/365 operation.

In case that an input source format is progressive and video frequency is 60 Hz, the POC value reaches its limitation 231−1 in about 1.13 year. The fourth feature described above forces video encoders to insert an IDR picture before reaching to the limit value.

This insertion event of an IDR picture causes some problems for manufacturers and users of remote surveillance cameras.

- Burden in encoder design, difficulty in validation

  - The IDR insertion arises only once a year. Implementing a function of an IDR picture insertion in the middle of the sequence is a burden because it happens quite rare. Moreover, such rareness makes the validation of the function difficult.

- Degradation in video quality

  - The gradual decoding refreshment (GDR) scheme is generally used for low delay video transmission. When GDR is used, all pictures except for an IDR picture at the beginning of a sequence are encoded as P-slices (or B-slices) in order to average the amount of bits of each picture. This enables to make the size of a bitstream buffer small and achieves low delay operation. However, when an IDR picture has to be inserted in the middle of a sequence, the amount of bits of the inserted IDR picture must be lowered to that of other non-IDR pictures. This enforces the encoder to selects relatively higher QP value and causes sudden degradation of video quality at the IDR picture and following pictures.

# Proposal

We propose to modify the range of POC value (PicOrderCntVal) and its derivation process so that the POC value becomes cyclic and the abovementioned problem is removed.

- The range of the POC value is changed from [-231, 231-1] (corresponds to signed 32-bit integer) to [0, 232-1] (corresponds to unsigned 32-bit integer).

  - Carry bit is discarded for addition and subtraction for the POC value reconstruction process.

    - unsigned int uiA, uiB, uiC, uiD;

    - Addition: uiC = uiA + uiB;

    - Subtraction: uiD = uiA - uiB;

  - When calculating the difference value of two POC values, the result of subtraction is stored in 32-bit signed integer. Comparison of two POC values are based on the sign of the resultant.

    - unsigned int uiA, uiB; int iC;

    - Difference: iC = (int)(uiA - uiB);

    - uiA is larger than uiB if iC = (int)(uiA - uiB) > 0;

    - uiA is smaller than uiB if iC < 0;

    - uiA and uiB is equal if iC == 0

- The following constraint is removed from the specification.

  - The value of PicOrderCntVal shall be in the range of −231 to 231 − 1, inclusive. In one coded video sequence, the PicOrderCntVal values for any two coded pictures shall be different.

The proposed modification does not affect the syntax nor the result of pixel reconstruction.

# Conclusion

We suggest to change the definition of picture order count (POC) so that the POC value can be cyclic. The register size of the POC value remains 32-bit. No changes of the syntax and the pixel decoding process is required.

# References

[1] "WD5: Working Draft 5 of High-Efficiency Video Coding", JCTVC-G1103, November 2011, Geneva Switzerland

# Patent rights declaration

FUJITSU LIMITED may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).