| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 8th Meeting: San José, CA, USA, 1–10 February, 2012 | Document: JCTVC-H0259_r1 |
| --- | --- |

| Title: | Efficient signaling of transform skipping mode |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jaeil Kim and Munchurl Kim Guseong-dong, Yuseong-Gu, Daejeon, Korea | Tel: Email: | 82-42-350-7519 jaeil1203@kaist.ac.kr, mkim@ee.kaist.ac.kr |
| Source: | KAIST |  |  |

_____________________________

# Abstract

In this contribution, an efficient signaling method of transform skipping modes for non-square quadtree (NSQT) transform is proposed. The proposed method for non-square transform skipping mode is to reduce the number of transform skipping modes by excluding the no-transform mode and to use a 1-bit flag to signal the transform skipping modes of 2D transform, 1D row transform and 1D column transform. The proposed method reduces the computational complexity by limiting the number of transform skip mode trials from 4 to 2 according to the non-square transform types. The encoding complexity by the proposed method is reduced to 2~3% over HM5.0_tsm in the common test condition without degrading BD rate performance

# Introduction

The transform skip mode in HM5.0_tsm includes four trial modes: 2D transform, 1D row transform, 1D column transform, and no-transform. Fig. 1 shows the transform skip mode in HM5.0_tsm [1].

Fig. 1 the transform skipping in HM5.0_tsm software

# Proposed method

For non-square prediction residues, it is likely to have signal distributions along the longer directions of the residual blocks. For skipping an1-D transform in row or column, it is more effective to represent a residual block signal over the 1-D row transform kernel than the 1-D column transform kernel when the row size of the residual block is larger than the column one and vice versa. In this case, 1-D column transform kernel may not be used and vice versa. If we further remove the no-transform mode, then only two transform skip modes, 2D transform and 1-D row transform can only be considered, thus requiring 1-bit flag to signal either one. Fig. 2 illustrates the proposed scheme of NSQT prediction block dependent transform skip mode.

Fig. 2 The proposed scheme of NSQT prediction block dependent transform skip mode

# Experimental results

The proposed method is implemented on the top of HM5.0_tsm software and tested under the common condition [2]. Experimental results are shown in Table 1 and Table 2.

Table 1. Performance of proposed method against HM5.0_tsm

|  | Random Access HE |  |  | Random Access HE-10 |  |  | Low delay B HE |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A (8bit) | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% | 0.0% |
| Class C | 0.0% | -0.1% | -0.1% |  |  |  | 0.0% | -0.2% | 0.0% |
| Class D | 0.0% | 0.0% | 0.2% |  |  |  | 0.0% | 0.3% | 0.4% |
| Class E |  |  |  |  |  |  | -0.1% | -0.3% | -0.3% |
| Overall | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% | 0.1% |
| Class F | 0.0% | 0.0% | 0.0% |  |  |  | 0.2% | 0.5% | 0.5% |
| Enc Time[%] | 98% |  |  | 97% |  |  | 98% |  |  |

Table 2. Performance of proposed method against HM5.0

|  | Random Access HE |  |  | Random Access HE-10 |  |  | Low delay B HE |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A (8bit) | 0.0% | -0.1% | -0.2% | 0.1% | 0.0% | -0.1% |  |  |  |
| Class B | -0.2% | 0.0% | 0.2% | -0.2% | 0.0% | 0.1% | -0.3% | 0.6% | 0.5% |
| Class C | -0.5% | -0.3% | -0.1% |  |  |  | -0.7% | -0.4% | -0.2% |
| Class D | -0.8% | -0.1% | -0.1% |  |  |  | -1.2% | 0.7% | 0.8% |
| Class E |  |  |  |  |  |  | -0.3% | 0.3% | 0.0% |
| Overall | -0.4% | -0.1% | 0.0% | -0.1% | 0.0% | 0.0% | -0.6% | 0.3% | 0.3% |
|  | -0.4% | -0.1% | 0.0% | -0.1% | 0.0% | 0.0% | -0.6% | 0.3% | 0.2% |
| Class F | -2.0% | -1.0% | -1.0% |  |  |  | -4.3% | -2.6% | -2.7% |

# Conclusions

In this contribution, an efficient signaling method of transform skipping modes for NSQT transform is proposed. The proposed method uses non-square prediction block dependent transform skip modes so encoding complexity is reduced to 2~3% over HM5.0_tsm in the common test condition [2] without degrading BD rate performance.

# References

- Marta Mrak, Joel Sole, Jizheng Xu, Ankur Saxena, "Description of Core Experiment 5: Transform Skipping," JCTVC-G1205, 7th JCT-VC Meeting, Geneva, Switzerland, November 2011.

- F. Bossen, “Common HM test conditions and software reference configurations”, JCTVC-G1200, 7th JCT-VC Meeting, Geneva, CH, Nov. 2011.

# Working draft

| transform_coeff( x0, y0, xC, yC, log2TrafoWidth, log2TrafoHeight, trafoDepth, blkIdx ) { | Descriptor |
| --- | --- |
| if( cbf_luma[ x0 ][ y0 ][ trafoDepth ] ¦ ¦ cbf_cb[ x0 ][ y0 ][ trafoDepth ] ¦ ¦ cbf_cr[ x0 ][ y0 ][ trafoDepth ] { |  |
| if( cu_qp_delta_enabled_flag && !IsCuQpDeltaCoded ) { |  |
| cu_qp_delta | ae(v) |
| IsCuQpDeltaCoded = 1 |  |
| } |  |
| if( split_transform_flag[ x0 ][ y0 ][ trafoDepth ] ) { |  |
| if( InterTUSplitDirection = = 2 ) { |  |
| x1 = x0 + ( ( 1 << log2TrafoWidth ) >> 1 ) |  |
| y1 = y0 |  |
| x2 = x0 |  |
| y2 = y0 + ( ( 1 << log2TrafoHeight ) >> 1 ) |  |
| x3 = x1 |  |
| y3 = y2 |  |
| } else { |  |
| x1 = x0 + ( ( 1 << log2TrafoWidth ) >> 2 ) * InterTUSplitDirection |  |
| y1 = y0 + ( ( 1 << log2TrafoHeight) >> 2 ) * ( 1 − InterTUSplitDirection ) |  |
| x2 = x1 + ( ( 1 << log2TrafoWidth ) >> 2 ) * InterTUSplitDirection |  |
| y2 = y1 + ( ( 1 << log2TrafoHeight) >> 2 ) * ( 1 − InterTUSplitDirection ) |  |
| x3 = x2 + ( ( 1 << log2TrafoWidth ) >> 2 ) * InterTUSplitDirection |  |
| y3 = y2 + ( ( 1 << log2TrafoHeight) >> 2 ) * ( 1 − InterTUSplitDirection ) |  |
| log2TrafoHeight = log2TrafoHeight + 2 * InterTUSplitDirection − 1 |  |
| log2TrafoWidth = log2TrafoWidth − 2 * InterTUSplitDirection + 1 |  |
| } |  |
| transform_coeff( x0, y0, x0, y0, log2TrafoWidth − 1, log2TrafoHeight − 1, trafoDepth + 1, 0 ) |  |
| transform_coeff( x1, y1, x0, y0, log2TrafoWidth − 1, log2TrafoHeight − 1, trafoDepth + 1, 1 ) |  |
| transform_coeff( x2, y2, x0, y0, log2TrafoWidth − 1, log2TrafoHeight − 1, trafoDepth + 1, 2 ) |  |
| transform_coeff( x3, y3, x0, y0, log2TrafoWidth − 1, log2TrafoHeight − 1, trafoDepth + 1, 3 ) |  |
| } else { |  |
| log2TrafoSize = ( ( log2TrafoWidth + log2TrafoHeight ) >> 1 ) |  |
| log2TrafoSizeC = ( ( log2TrafoSize = = Log2MinTrafoSizeC ) ? log2TrafoSize : log2TrafoSize – 1 ) |  |
| if ( PredMode = = MODE_INTRA ) { |  |
| scanIdx = ScanType[ log2TrafoSize – 2 ][ IntraPredMode ] |  |
| scanIdxC = ScanType[ log2TrafoSize – 2 ][ IntraPredModeC ] |  |
| } else { |  |
| scanIdx = 0 |  |
| scanIdxC = 0 |  |
| } |  |
| if ( cbf_luma[ x0 ][ y0 ][ trafoDepth ] ) { |  |
| if(log2TrafoWidth != log2TrafoHeight) |  |
| tsm_flag | ae(v) |
| else |  |
| tsm | ae(v) |
| residual_coding( x0, y0, log2TrafoWidth, log2TrafoHeight, scanIdx, 0 ) |  |
| } |  |
| if ( log2TrafoSize > Log2MinTrafoSize ) { [Ed. (WJ): Log2MinTrafoSize or 2?] |  |
| if ( cbf_cb[ x0 ][ y0 ][ trafoDepth ] ) |  |
| residual_coding( x0, y0, log2TrafoSizeC, trafoDepth, scanIdxC, 1 ) |  |
| if ( cbf_cr[ x0 ][ y0 ][ trafoDepth ] ) |  |
| residual_coding( x0, y0, log2TrafoSizeC, trafoDepth, scanIdxC, 2 ) |  |
| } else if ( blkIdx == 3 ) { |  |
| if ( cbf_cb[ x0 ][ y0 ][ trafoDepth ] ) |  |
| residual_coding( xC, yC, log2TrafoSizeC, trafoDepth, scanIdxC, 1 ) |  |
| if ( cbf_cr[ x0 ][ y0 ][ trafoDepth ] ) |  |
| residual_coding( xC, yC, log2TrafoSizeC, trafoDepth, scanIdxC, 2 ) |  |
| } |  |
| } |  |
| } |  |
| } |  |

# Patent rights declaration(s)

KAIST may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).