| TJoint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 8th Meeting: San José, CA, USA, 1–10 February, 2012 | Document: JCTVC-H0260 |
| --- | --- |

| Title: | CE4 : 3.2 Implicit signaling of QP offset based on TU size |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Chanyul Kim Youngo Park Kwangpyo Choi Jaehyun Kim JungHun Park #416 Matean 3-dong, Yeongtong-gu, Suwon-si, Gyeonggi-do, 443-742, Korea | Tel: Email: | dionism@samsung.com youngo.park@samsung.com kp5.choi@samsung.com jhgim@samsung.com jeonghoon@samsung.com |
| Source: | Samsung Electronics Co., Ltd. |  |  |

_____________________________

# Abstract

In this proposal, implicit signal of QP offset based on transform size is presented. This scheme apply for inter coding case only. -0.1%/0.0%/0.0% for RA_HE, -0.2%/0.1%/0.4% for LD_HE, -0.1%/-0.9%/-1.3% for RA_LC and -0.2%/0.1%/0.4% for LD_LC coding gain is obtained without additional complexity and visual quality changing.

# Introduction

Delta-QP (dQP) signaling by the CU level is a method for rate control which is adopted in the current HM. Lack of rate control algorithm on the HM leads to make question “Is it enough for rate control with dQP in CU level?” However, it is too much burden for signaling when QP control is applied to TU level.

We presents the implicit signaling of QP offset based on TU size which does not required additional bit cost. Based on knowledge of rate control algorithm, the static areas have more possibility to be reference area rather than moving region. Therefore, relative QP offset based on absolute TU size is efficient method to enhanced performance without any scarifying bit cost.

The current HM uses different size of CU and TU. Only CU based dQP is not sufficient method for those targets, especially for large CU. We proposed TU size based implicit signal of QP offset which does not require any complexity and additional bits for signaling.

# Algorithm description

The idea of the proposed algorithm is as follows,

- The area which uses the large transform has more possibility to be static region.

- The less QP is apply for large transform blocks, relatively higher QP is apply for small transform block size.

- The static region has more probability to be reference frame than moving area that is why we decrease the QP in static region.

Moreover, the proposed method is only applied for inter frame (RA or LD cases) because static region is only defined in the inter pictures.

The example of QP offset based on TU size is depicted in Figure 1. Assume that CU size is 64, and CU QP is QP_CU, the partitioned transform blocks are applied for different QP with QP_CU. Table 1 shows the implicit QP offset based on TU size.

Table 1. QP offset relative to absolute TU size

|  | 4x4 | 8x8 | 16x16 | 32x32 |
| --- | --- | --- | --- | --- |
| Implicit QP offset | 2x∆ | ∆ | 0 | -∆ |

Figure 1. Example of QP offset based on TU for CU size is 64

# Simulation Results

Using HM5.0, simulation is performed under the common test condition [1]. We have tested various ∆, the performance does not much difference, therefore it is recommend to use ∆ = 1 . The consistent coding gain is obtained in all sequences. It is note that more gain ( -0.4/-0.6/-0.6 RA_HE, -0.6/-1.3/-1.3 LD_HE) is observed in HE case for F-sequences.

Table 2. Result of QP offset based on TU size (∆ =1)

# Conclusion

The implicit QP offset signaling based on TU size is presented. We consider the large transform block as static region in the inter frame. It gives coding gain without additional complexity and visual degradation.

# References

- Frank Bossen, “Common test conditions and software reference configurations”, JCTVC-C500, JCT-VC 3rd Meeting, Guangzhou, October 2010.

# Patent rights declaration(s)

Samsung Electronics Co., Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Annex : WD Text change

### 8.5.1 Scaling and transformation process

Inputs to this process are:

–	a luma location ( xT, yT ) specifying the top-left luma sample of the current transform unit relative to the top-left luma sample of the current picture,

–	a variable trafoDepth specifying the hierarchy depth of the current block relative to the coding unit,

–	a variable cIdx specifying the chroma component of the current block,

–	a variable nW specifying the width of the current transform block.

–	a variable nH specifying the height of the current transform block.

Output of this process is:

–	a modified version of the (nW)x(nH) array of residual samples r with elements rij.

The quantization parameter qP is derived as follows.

–	If cIdx is equal to 0,

qP = QP’Y			(8-1)

–	Otherwise,

qP = QP’C			(8-2)

–	If slice_type !=I and nW== nH,

qP = QP’Y	+ qpOffsetMap[log2TrafoSize – 2]

- If cIdx is not 0

qp = qp’c ( is chroma qp derivation from qP)

–	Otherwise,

–	If cIdx is equal to 0,

qP = QP’Y

–	Otherwise,

qP = QP’C				(8-3)

Table xxx : qpOffsetMap[idx]

| idx | value |
| --- | --- |
| 0 | 2 |
| 1 | 1 |
| 2 | 0 |
| 3 | -1 |

# Annex: Visual (subjective) testing