| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21–30 November, 2011 | Document: JCTVC-G1103_d9 |
| --- | --- |

| Title: | WD5: Working Draft 5 of High-Efficiency Video Coding |  |  |
| --- | --- | --- | --- |
| Status: | Output Document of JCT-VC |  |  |
| Purpose: | Working Draft of HEVC |  |  |
| Author(s) or Contact(s): | Benjamin Bross Fraunhofer HHI Woo-Jin Han Gachon University Jens-Rainer Ohm RWTH Aachen Gary J. Sullivan Microsoft Thomas Wiegand Fraunhofer HHI / TU Berlin | Email: Email: Email: Email: Email: | benjamin.bross@hhi.fraunhofer.de hurumi@gmail.com ohm@ient.rwth-aachen.de garysull@microsoft.com thomas.wiegand@hhi.fraunhofer.de |
| Source: | Editor |  |  |

_____________________________

.

.

.

9.2.3.2.1	Arithmetic decoding process for a binary decision

Inputs to this process are ctxIdxTable, ctxIdx, codIRange, and codIOffset.

Outputs of this process are the decoded value binVal, and the updated variables codIRange and codIOffset.

Error: Reference source not found shows the flowchart for decoding a single decision (DecodeDecision):

- The value of the variable codIRangeLPS is derived as follows.

–	Given the current value of codIRange, the variable qCodIRangeIdx is derived by

qCodIRangeIdx =( codIRange  >>  5 ) & 7		(9-29)

–	Given qCodIRangeIdx and pStateIdx associated with ctxIdxTable and ctxIdx, the value of the variable rangeTabLPS as specified in Error: Reference source not found is assigned to codIRangeLPS:

codIRangeLPS = rangeTabLPS[ pStateIdx >> 2 ][ qCodIRangeIdx ]	(9-30)

- The variable codIRange is set equal to codIRange  codIRangeLPS and the following applies.

–	If codIOffset is greater than or equal to codIRange, the variable binVal is set equal to 1 − valMPS, codIOffset is decremented by codIRange, and codIRange is set equal to codIRangeLPS.

–	Otherwise, the variable binVal is set equal to valMPS.

Given the value of binVal, the state transition is performed as specified in subclause 9.3.3.2.1.1. Depending on the current value of codIRange, renormalization is performed as specified in subclause 9.3.3.2.2.

.

.

.

Table 9-42 – Specification of rangeTabLPS depending on pStateIdx and qCodIRangeIdx

| pStateIdx >> 2 | qCodIRangeIdx |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| 0 | 122 | 138 | 154 | 168 | 182 | 198 | 211 | 226 |
| 1 | 103 | 114 | 129 | 140 | 152 | 166 | 174 | 189 |
| 2 | 85 | 95 | 105 | 116 | 126 | 137 | 145 | 157 |
| 3 | 70 | 79 | 89 | 96 | 105 | 114 | 122 | 130 |
| 4 | 59 | 66 | 74 | 79 | 87 | 95 | 101 | 107 |
| 5 | 49 | 55 | 61 | 66 | 72 | 78 | 83 | 90 |
| 6 | 41 | 46 | 51 | 56 | 60 | 66 | 70 | 74 |
| 7 | 34 | 39 | 43 | 47 | 51 | 55 | 59 | 63 |
| 8 | 29 | 33 | 36 | 40 | 43 | 46 | 50 | 53 |
| 9 | 25 | 28 | 31 | 34 | 37 | 40 | 43 | 45 |
| 10 | 21 | 23 | 26 | 29 | 31 | 33 | 36 | 38 |
| 11 | 19 | 21 | 23 | 25 | 27 | 29 | 32 | 34 |
| 12 | 17 | 18 | 20 | 22 | 24 | 26 | 28 | 30 |
| 13 | 15 | 16 | 18 | 20 | 22 | 23 | 25 | 27 |
| 14 | 13 | 15 | 16 | 18 | 19 | 21 | 22 | 24 |
| 15 | 8 | 9 | 9 | 10 | 11 | 12 | 13 | 14 |

.

.

.