| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 8th Meeting: San José, CA, USA, 1–10 February, 2012 | Document: JCTVC-H0266 |
| --- | --- |

| Title: | CE1: Report of test results related to subtest B2 (8-bit-init) and C3 (Alt. PMU) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Heiner Kirchhoffer, Benjamin Bross, Philipp Helle, Detlev Marpe, Tung Nguyen, Mischa Siekmann, Jan Stegemann, Thomas Wiegand | Email: | Heiner.Kirchhoffer@hhi.fraunhofer.de |
| Source: | Fraunhofer HHI |  |  |

_____________________________

# Abstract

This contribution reports results for CE1 subtest B2 (related to the 8-bit initialization) and subtest C3 (related to the alternative probability model update).

# 8-bit-initialization

In JCTVC-G633 [2], an approach for 8-bit initialization was tested and on the 7th JCTVC meeting, it was adopted in the current WD. It uses a piece-wise linear interpolation when deriving an initial context model state from an initialization value and the QP. In CE1 subtest B2, two alternative sets of parameters are tested for the piece-wise linear interpolation [1], a ‘linear’ mapping and a ‘disabled’ mapping. For each of the two mappings, a new set of context model initialization values was computed and used for the simulations.

The detailed results can be found in the accompanying Excel files.

## ‘Linear’ mapping

The ‘linear’ mapping uses values for segOffset and accumulatedSegOffset according to Table 1. It approximates a linear dependency between QP and probability.

Table 1: Values for segOffset and accumulatedSegOffset for the ‘linear’ mapping

| segmentIdx | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| segOffset[segmentIdx] | 4 | 3 | 3 | 4 | 6 | 8 | 13 | 21 |
| accumulatedSegOffset[segmentIdx] | −2 | 2 | 5 | 8 | 12 | 18 | 26 | 39 |

## ‘Disabled’ mapping

The ‘disabled’ mapping uses values for segOffset and accumulatedSegOffset according to Table 2. It uses the same slope for all line segments and corresponds to disabling the piece-wise linear interpolation.

Table 2: Values for segOffset and accumulatedSegOffset for the “disabled” mapping

| segmentIdx | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| segOffset[segmentIdx] | 8 | 8 | 8 | 8 | 8 | 8 | 8 | 8 |
| accumulatedSegOffset[segmentIdx] | −4 | 4 | 12 | 20 | 28 | 36 | 44 | 52 |

## Test results for the 'linear' mapping (subtest B2, test 1)

The experimental results for the ‘linear’ mapping are shown in Table 3, Table 4, and Table 5 for the QP ranges 17-32, 22-37, and 27-42, respectively. Only in few cases, very small changes in BD bit rates can be observed relative to the HM-5.0 anchor. Averaged BD bit rate and runtime changes are virtually zero.

Table 3: Results for Subtest B2, test 1, QP 17-32

Table 4: Results for Subtest B2, test 1, QP 22-37

Table 5: Results for Subtest B2, test 1, QP 27-42

## Test results for the 'disabled' mapping (subtest B2, test 3)

The experimental results for the ‘disabled’ mapping are shown in Table 6, Table 7, and Table 8 for the QP ranges 17-32, 22-37, and 27-42, respectively. Virtually no change in averaged BD bit rates and runtimes can be observed relative to the ‘linear’ mapping and to the HM-5.0 anchor.

Table 6: Results for Subtest B2, test 3, QP 17-32

Table 7: Results for Subtest B2, test 3, QP 22-37

Table 8: Results for Subtest B2, test 3, QP 27-42

# Alternative probability model update

In JCTVC-G547 [3], a counter-based probability model update for the usage with the M coder is proposed. In subtest C3 it is tested based on HM-5.0 which contains the 8-bit-initialization. To adapt the counter-based probability model update to the 8-bit-initialization as present in HM-5.0, particular parameters for the piece-wise linear interpolation are used in the 8-bit-initialization according to Table 9. It is denoted ‘exponential’ mapping.

Table 9: Values for segOffset and accumulatedSegOffset for the ‘exponential’ mapping

| segmentIdx | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| segOffset[segmentIdx] | 770 | 567 | 374 | 246 | 162 | 107 | 70 | 46 |
| accumulatedSegOffset[segmentIdx] | −385 | 385 | 952 | 1326 | 1572 | 1734 | 1841 | 1911 |

The pseudo-code for calculating pStateIdx is also adapted as follows in order to produce a 12 bit state as required by the counter-based probability model update used in subtest C3:

qpMinus26 = Clip3( 0, 51, SliceQPY ) − 26
val = Clip3( 0, 3839, ( n << 8 ) + m  qpMinus26 + 128 )
val −= 1920
sgn = val >> 11
val = ( val ^ sgn ) + 128
segmentIdx = val >> 8
pStateIdx = accumulatedSegOffset[segmentIdx] + ( (val & 255 ) * segOffset[segmentIdx] >> 8 )
pStateIdx = 2048 + ( pStateIdx ^ sgn )

Note that valMps is not stored for the counter-based probability model update and the 12 bit state pStateIx corresponds linearly to the probability interval [0,1].

The detailed results can be found in the accompanying Excel files.

## Analysis of computational complexity of the alternative PMU

For the alternative probability model update, number of operations for 1 bin decoding, memory size increment for each context model, bit width of calculation, and number of additional counters is evaluated.

### Number of operations for 1 bin decoding

The probability model update operation uses 12 bits for the counter variable c for each context model and a windows size of 64 or 32, i.e., after the encoding or decoding of each bin b, the 12 bit counter c is updated as follows:

c+=((−b)∧WSconst)−(c>>WS)				Eq. (1)

WSconst is a constant that is derived from the window size WS as follows:

WSconst=212−WS−1

For the encoding or decoding of a bin, valMPS and pStateIdx are derived as follows:

valMPS=c>>11pStateIdx=((c−211)XOR(valMPS−1))>>m		Eq. (2)

The variable m is computed as m=11-log2(number of rows in rangeTabLPS).

Executing Eq. (1) requires 1 addition, 2 subtraction, 1 bit shift, and 1 bitwise AND (Note that the negation of the bin b is counted as a subtraction: 0- b). Executing Eq. (2) requires 2 subtractions, 2 bit shifts, and 1 XOR.

Consequently, the number of operations required for decoding one bin are given as:

| Type of operation | Number of operations |
| --- | --- |
| Add/Subtract | 5 |
| Bit shift | 3 |
| AND/XOR | 2 |

### Memory size increment for each context model

The number of bits required for each context model increases from 7 bit to 12 bit.

### Bit-width of calculation description

The bit width of calculation description is 12 bit.

### Number of additional counters

No additional counters are required.

## Test results for different rangeTabLPS dimensions without recalculation of context initialization values (subtest C3, tests 1-4)

The experimental results for rangeTabLPS dimensions 64x4, 32x8, 32x4, and 16x8 without recalculation of context initialization values are shown in Table 10, Table 11, Table 12, and Table 13, respectively. For the counter-based probability model update, a window size of 64 is used. An averaged luminance BD bit rate reduction of around 0.22% can be observed for the 64x4 and the 32x4 table. For the 32x8 and 16x8 table, these bit rate reductions increase to 0.3%. The encoder and decoder runtimes are virtually unchanged.

Table 10: Results for Subtest C3, test 1

Table 11: Results for Subtest C3, test 2

Table 12: Results for Subtest C3, test 3

Table 13: Results for Subtest C3, test 4

## Test results for rangeTabLPS 64x4 and 16x8 with recalculation of context initialization values (subtest C3, tests 5 and 8)

The experimental results for rangeTabLPS dimensions 64x4 and 16x8 with recalculation of context initialization values are shown in Table 14 and Table 15, respectively. For the counter-based probability model update, a window size of 64 is used. The only difference between test 5 and test 1 or test 8 and test 4 are that for tests 5 and 8, new context model initialization values are computed. Simulation results however show that this has virtually no impact on BD bit rate or codec runtimes.

Table 14: Results for Subtest C3, test 5

Table 15: Results for Subtest C3, test 8

## Test results for rangeTabLPS dimensions 16x8, window size 32, witouth and with recalculation of context initialization values (subtest C3, tests 9 and 10)

The experimental results for rangeTabLPS dimensions 16x8 without and with recalculation of context initialization values are shown in Table 16 and Table 17, respectively. For the counter-based probability model update, a window size of 32 is used. The only difference between test 9 and test 4 or test 10 and test 8 are that for tests 9 and 10, a window size of 32 is used while tests 4 and 8 use 64. Simulation results show that BD bit rate is slightly smaller for window size 32 than for window size 64. The recalculation of context initialization values however has almost no impact.

Table 16: Results for Subtest C3, test 9

Table 17: Results for Subtest C3, test 10

# Software

The accompanying patch files contain the software relative to HM-5.0. The tests are configured as follows:

The tests of subtest B2 can be configured in TypeDef.h as follows:

Test 1 (Linear):

#define DISABLED_SEGMENTS         0
#define LINEAR_SEGMENTS           1

Test 3 (disables):

(no changes required)

The tests of subtest C3 can be configured in TypeDef.h as follows:

Test 1:

#define ALT_PE_INITVALS                         0

Test 2:

#define ALT_PE_INITVALS                         0

#define RTABDIM32x8                             1

Test 3:

#define ALT_PE_INITVALS                         0

#define RTABDIM32x4                             1

Test 4:

#define ALT_PE_INITVALS                         0

#define RTABDIM16x8                             1

Test 5:

(no changes required)

Test 8:

#define RTABDIM16x8                             1

Test 9:

#define ALT_PE_INITVALS                         0

#define RTABDIM16x8                             1

#define WS0                                     5

Test 10:

#define RTABDIM16x8                             1

#define WS0                                     5

# Conclusion

Tests related to 8-bit initialization show almost no difference in BD bit rate between the different tested piece-wise linear configurations. However, for the counter-based probability model update, an ‘exponential’ mapping was used that allows the reuse of context initialization values that were computed using the original probability model update of HEVC.

Tests related to the counter-based probability model update show averaged BD bit rate reductions of up to 0.31% for window size 32 and rangeTabLPS table dimensions 16x8 while the effect of recalculating context initialization values was negligible.

# References

- R. Joshi, E. Alshina, H. Sasai, H. Kirchhoffer, J. Lainema, “Description of Core Experiment 1: Entropy Coding”, JCTVC-G1201, Geneva, CH, November 2011.

- H. Kirchhoffer, B. Bross, A. Henkel, D. Marpe, T. Nguyen, M. Preiß, M. Siekmann, J. Stegemann, T. Wiegand, “CE1: Report of test results related to PIPE-based Unified Entropy Coding”, JCTVC-G633, Geneva, CH, November 2011.

- J. Stegemann, H. Kirchhoffer, D. Marpe, T. Wiegand, “Non-CE1: Counter-based probability model update with adapted arithmetic coding engine”, JCTVC-G547, Geneva, CH, November 2011.

# Patent rights declaration(s)

Fraunhofer-Gesellschaft zur Förderung der angewandten Forschung e. V. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).