| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 8th Meeting: San José, CA, USA, 1–10 February, 2012 | Document: JCTVC-H0267 |
| --- | --- |

| Title: | AHG7 : Constraint the number of motion vector for memory bandwidth reduction |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Chansik Park Tetsuo Kosuge Jaehyun Kim Kyohyuk Lee JeongHun Park Chanyul Kim #416 Matean 3-dong, Yeongtong-gu, Suwon-si, Gyeonggi-do, 443-742, Korea | Email: | chansik.park@samsung.com t.kosuge@samsung.com jhgim@samsung.com kyohyuk.lee@samsung.com jeonghoon@samsung.com dionism@samsung.com |
| Source: | Samsung Electronics Co., Ltd. |  |  |

_____________________________

# Abstract

In HEVC, one 64x64 LCU can have 512 motion vectors in case of all 4x4 inter partitions of bi-directional prediction. If all partitions are 4x4 inter mode and have 2 motion vectors for bi-directional prediction, it is not guaranteed that we could implement real time decoder with very high resolution video over full HD due to huge memory bandwidth requirement. In order to solve bandwidth problem, A SPS flag disabling 4x4 inter partitions which is disabled in default configuration (such as inter_4x4_enabled flag in SPS header) was adopted in Torino Meeting related to [1]. There was supplementary information for restricting specific block size to reduce memory bandwidth [2]. However the mode restriction of block size is not the only solution for memory bandwidth problem.

The limitation of the number of motion vector in a LCU rather than block size based mode restriction was presented in this proposal. Actually the limitation of the number of motion vector inherited from the H.264/AVC level consideration using MaxMvsPer2Mb. Assume that LCU size is 64, the memory bandwidth requirement of inter_4x4_enabled flag ==0 is the almost twice larger than restriction the number of motion vector to128. However, the coding efficiency is almost similar.

# Introduction

The main bottleneck of memory bandwidth in the decoder comes from motion compensation process. The worst case can be happened that all motion partitions have the allowable smallest partition size with bi-prediction and any two partitions are not overlapped each other. Assume that all 64x64 LCUs in the resolution of 1920x1088@30fps have the worst case, total 512 motion vectors are to be processed for motion compensation at 333MHz/64-bit memory system. This assumption could be extended to 1.35GHz for 4Kx2K@30fps and 5.4GHz for 8Kx4K@30fps. Required memory driving clock frequency could be unreal as resolution and frame rate go higher.

As mentioned in the abstract, there are two solutions for reducing memory bandwidth.

- Block size based mode restriction (Adopted in HM as a common condition test, whose default setting is inter_4x4_enabled flag ==0, It is note that inter_4x4_disable_flag in the SW)

- limitation of the number of motion vector in a LCU

Assume that the LCU is partitioned as shown in Figure 1., it is no need to disable small PU block which does not cause memory bandwidth bottleneck. In those case, we don’t need to restrict the small PU size which leads to coding performance drop.

Figure1. Why do we need to restrict the number of motion vector rather than small PU limitation?

We have compared those two solutions in terms of coding efficiency and memory bandwidth. The results show that constraint of the number of motion vector keeps the almost same coding efficiency and more memory reduction compared to current HM method.

# Experimental Results

We have tested the variation of two solutions. The current HM used the setting of inter_4x4_enabled flag ==0. The table 1 shows that the restriction of the number of motion vector in a LCU is more comparable to the current HM method in terms of both coding efficiency and memory reduction.

The algorithm for limiting the number of motion vector is an encoder issues and various methods could be used.

We tested two kind of algorithm to analyse the performance for example.

- If the number of motion vector is larger than 128, disabling small PU block is performed as depicted in Table2.

- If the number of motion vector is larger than 128, the remained blocks are coded as intra as shown Table 1.

Table 1. Restriction of the number of mv to 128 vs HM5.0 using (1) algorithm

Table 2. Restriction of the number of mv to 128 vs HM5.0 using (2) algorithm

The theoretical worst case for memory bandwidth is calculated as shown in Table 3. In the case of 128mv, the required bandwidth is almost the half of the current HM.

Table 3. The theoretical required memory bandwidth.

128mv case

Current HM

Figure 2. Bandwidth & BD-rate vs. MV constraint

# Syntax Proposal

We propose removing of the syntax “inter_4x4_enabled_flag” in the SPS

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| profile_idc | u(8) |
| reserved_zero_8bits /* equal to 0 */ | u(8) |
| level_idc | u(8) |
| …. |  |
| inter_4x4_enabled_flag | u(1) |
| ….. |  |

Instead of inter_4x4_enable_flag, restriction_num_mv could be used according to level issues non-normatively. For example, restriction of the number of motion vector would be set using Table3.

Table 3. The meaning of restriction_num_mv

| Restriction_num_mv | The allowable number of motion vector |
| --- | --- |
| 0 | Num = (1<< ((Log2MaxCUSize-Log2MinCUSize +1) <<1 +1)) : No restriction |
| N | Num = (Num >> N) |

# Conclusions

We present the limitation of the number of motion vector in a LCU rather than block size based mode restriction such as inter_4x4_enabled flag in SPS header. The proposed method shows comparable coding efficiency with the current HM.

We recommend that the MV number restriction for memory bandwidth could be considered for level issues rather than inter_4x4_enabled flag ==0.

# References

- B. Bross, “BoG Report on MV Coding and Throughput/Parsing Robustness”, JCTVC-F744, JCT-VC 6th Meeting, Torino, July 2011.

- K. Kondo et al, “Non-CE3: Report on a restriction for small block”, JCTVC-G392, JCT-VC 7th Meeting, Geneva, November 2011.

# Patent Rights Declaration(s)

Samsung Electronics Co., Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# WD changing

See the attached file. Search with inter_4x4_enabled_flag in WD