| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 8th Meeting: San José, CA, USA, 1–10 February, 2012 | Document: JCTVC-H0269 |
| --- | --- |

| Title: | AHG15 : Independent control of Max CU and LCU for small LCU |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jongbum Choi Jaehyun Kim Kyohyuk Lee JeongHoon Park Chanyul Kim #416 Matean 3-dong, Yeongtong-gu, Suwon-si, Gyeonggi-do, 443-742, Korea | Email: | jb79.choi@samsung.com jhgim@samsung.com kyohyuk.lee@samsung.com jeonghoon@samsung.com dionism@samsung.com |
| Source: | Samsung Electronics Co., Ltd. |  |  |

_____________________________

# Abstract

The LCU sizes have a significant impact on HEVC codec data flow and architecture. The LCU size of 64x64 has proven to be a good trade-offs between coding gain and complexity. Smaller size of LCU such as 16x16 has been proven to be a good trade-offs between coding gain and latency for long period of previous standards as mentioned in [3]. The modification of min and max CU size irrelevant to LCU size in the high level syntax compensates for coding loss caused by setting smaller LCU size. The result shows that -0.3/-0.3/-0.4 for AI_HE, -0.6/-0.5/-0.4 for RA_HE, and -0.9/-0.6/0.0 for LD_HE compared to 32x32 LCU, and -1.0/-1.1/-1.3 for AI_HE, -2.1/-1.5/-1.5 for RA_HE, and -2.7/-1.4/2.5 for LD_HE compared to 16x16 LCU.

# Introduction

In the current HM, the Max/Min CU size of HM is defined in SPS header as shown in Table 1.

Table 1. Signalling of max/min CU size in SPS header

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| log2_min_coding_block_size_minus3 | ue(v) |
| log2_diff_max_min_coding_block_size | ue(v) |

The max CU size is calculated as following.

log2(LCU_size) = log2(Max_CU_size) =(log2_min_coding_block_size_minus3 + log2_diff_max_min_coding_block_size +3) –(1)

Max_CU_size is also the same as size of LCU.

In this proposal, base processing unit (BPU) is introduced whereas Max CU size is the LCU size as shown in Table 1.

log2(LCU_size) = log2(Max_CU_size) =(log2_min_coding_block_size_minus3 + log2_diff_max_min_coding_block_size +3) –(2)

log2(BPU_size)   = log2(LCU_size) + log2_diff_lcu_max_coding_block_size                                         -(3)

)

Table 2. BPU,LCU, and Min CU size signalling in SPS

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| log2_diff_bpu_lcu_size | ue(v) |
| log2_min_coding_block_size_minus3 | ue(v) |
| log2_diff_max_min_coding_block_size | ue(v) |

# Algorithm description

The main difference of the proposed method compared to the HM is that Based Processing Unit (BPU) is the large block including several LCUs.

Figure 1. (a)LCU size 64x64 in HM5.0 (c) LCU size 32x32 in HM 5.0 vs (b) BPU size and LCU size are equal to 64x64 in the proposed one (d) BPU size and LCU size different in the proposed one

Assume that LCU size is 64x64 in HM 5.0 as shown in Figure 1.(a), the proposed method works the same way to HM with one bit cost. It is note that the proposed method is the exactly same way to coding as HM when log2_diff_bpu_lcu_size is 0. The LCU size becomes smaller such as 32x32 shown in Figure 1.(c), the proposed method set the BPU and LCU size shown in Figure 1.(d). The LCU is 32x32 which is the same as HM except for coding order. The proposed method shows the exactly same latency compared to LCU 32x32 cases in HM. However the coding gain is observed compared to HM as shown in Table 3.

# Simulation Result

Using HM5.0, simulation is performed under the common test condition [2]. The result shows that coding gain is observed for all configurations. Small LCU size could be emulated as shown in Figure 1.

Table3. (BPU = 64 and LCU = 32) vs (LCU size =32, HM anchor)

Table. 4 (BPU = 64 and LCU = 16 ) vs (LCU size =16, HM anchor)

# Conclusion

This proposal has target for enhancement coding performance in case of small LCU. The experimental result shows that the proposed method improves coding performance all sequence without scarifying latency or other benefit from small LCU size. We anticipate that the proposed method be adopted into next HM.

# References

[1] B. Bross, “WD5: Working Draft 5 of High-Efficiency Video Coding”, JCT-VC document JCTVC-G1103, 7th Meeting: Geneva, CH, 21–30 November, 2011

[2] F. Bossen, “Common test conditions and software reference configurations”, JCT-VC document JCTVC-G1200, 7th Meeting: Geneva, CH, 21–30 November, 2011

[3] M. Zhou et al., “Items to be clarified in HEVC”, JCT-VC document JCTVC-G096, 7th Meeting: Geneva, CH, 21–30 November, 2011

# Patent rights declaration(s)

Samsung Electronics Co., Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Annex : WD Text change