| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 8th Meeting: San José, CA, USA, 1–10 February, 2012 | Document: JCTVC-H0271 |
| --- | --- |

| Title: | AHG9: CABAC with constrained outstanding bits |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Tzu-Der Chuang, Ching-Yeh Chen, Yu-Wen Huang, and Shawmin Lei No. 1, Dusing Rd. 1, Hsinchu Science Park, Hsinchu, Taiwan 30078 | Tel: Email: | Shawmin Lei +886-3-5670766 ext. 25555 {peter.chuang, chingyeh.chen, yuwen.huang, shawmin.lei}@mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

This proposal was presented in JCTVC-F059. In HM, there is no constraint on continuous outstanding bits in CABAC. Theoretically, the number of continuous outstanding bits could be infinite. Practically, the number is up to 189 in our analysis. When a lot of continuous outstanding bits are accumulated, a sudden increase of output bits will occur, which complicates hardware design significantly because of the unpredictable largest number of continuous outstanding bits. In this contribution, two coding interval adjustment procedures are proposed to constrain the number of continuous outstanding bits. One is to divide the coding interval into two parts at 512. The other one is to code a bypass bin. It is reported that the proposed methods cause no differences in coding efficiency and roughly the same run time while at most 18 bits could be outputted simultaneously.

# Introduction

Binary arithmetic coding is a recursive interval-subdividing procedure. An output bitstream is a pointer to a probability of a final coding interval. The probability of a coding interval is specified by range and low. Low is the lower bound of the coding interval, and range is the possible scope of the coding interval. Depending on that the coding symbol is a most probable symbol (MPS) or a least probable symbol (LPS), the next coding interval is updated as one of the two sub-intervals. In High-Efficiency Video Coding test model version 5.0 (HM-5.0), in order to maintain the precision of arithmetic coding, a renormalization process after interval subdivision will be performed as long as the range of a new interval is smaller than 256. The flowchart of recursive renormalization is shown in Figure 1. When one loop of the recursive renormalization process is performed, it is intended to output one bit, but sometimes the bit value cannot be determined in time. Figure 2 shows more details of outputting one bit in the renormalization process. In the renormalization process, if low is smaller than 256, which means the coding interval specified by range and low is fully placed in the lower half, one bit of zero can be outputted. If low is larger than 512, which means the coding interval is fully placed in the upper half, one bit of one can be outputted. However, if low is in the range of [256,512), which means the interval is placed in the middle and across 512, the output bit cannot be determined as zero or one because a carry may be propagated into coded bitstream, and then bits-outstanding (BO) is introduced to handle this case. BO means the bit count of the outstanding bits. In CABAC, coded bits are not outputted until further carry propagation will not change these bits. When an output bit is uncertain, BO will increase by one. After the interval is fully placed in the lower half or the upper half, one bit equal to zero with a series of BO bits equal to one or one bit equal to one with a series of BO bits equal to zero can be outputted.

Figure 1.  Flowchart of the renormalization process in CABAC

Figure 2.  Illustration of outputting one bit in the renormalization process

In HM-5.0, there is no constraint on the BO value, which means if the coding interval is always across 512, BO can be infinite. Unlimited BO will increase the difficulty of hardware implementation because there may be a sudden burst of many outputted bits to the bitstream buffer. In JCTVC-F059 [1], a coding interval adjustment procedure was proposed to constrain the BO value. This contribution proposes the same method again with results on HM-5.0 and another method that uses bypass bin to simplify the coding interval adjustment procedure. The rest of this contribution is organized as follows. The proposed methods will be introduced in Section 2, simulation results will be shown in Section 3, and conclusions will be given in Section 4.

# Proposed methods

## Method 1 – selecting the larger range interval

The BO increases when the coding interval crosses 512. In this contribution, we propose an adaptive coding interval adjustment procedure to avoid crossing 512. First, a maximum constraint on the BO value is defined and called maxBO. When the renormalization operation is done and the BO reaches or exceeds maxBO, a coding interval adjustment procedure is invoked.

Figure 3 shows the concept of proposed coding interval adjustment procedure. If BO is equal to or larger than maxBO after renormalization and the coding interval is not fully placed in the upper or lower half, the coding interval is divided into two parts. The interval of the lower part is [low, 512), and the interval of the upper part is [512, low+range). The binary arithmetic coder selects the larger part of these two intervals as the new coding interval. If the upper part is selected, one bit equal to 1 with BO bits equal to 0 can be outputted; otherwise, one bit equal to 0 with BO bits equal to 1 can be outputted. Selecting either of them can reset BO to 0.

Figure 3.  Concept of proposed adaptive coding interval adjustment procedure

## Method 2 - sending one bypass bin

To simplify the coding interval adjustment procedure, a new method is proposed to code a bypass bin to avoid crossing 512. Figure 4 shows the concept of proposed coding interval adjustment procedure by using bypass bin. If BO is equal to or larger than maxBO after renormalization and the coding interval is not fully placed in the upper or lower half, the coding interval is doubled and divided into two equal length parts (just like the coding procedure of bypass bins). The interval of the lower part is [low*2, low*2+range), and the interval of the upper part is [low*2+range, low*2+range*2). The binary arithmetic coder selects the larger part of these two intervals as the new coding interval. If the upper part is selected, one bit equal to 1 with BO bits equal to 0 can be outputted; otherwise, one bit equal to 0 with BO bits equal to 1 can be outputted. Selecting either of them can reset BO to 0. Compared to method in Sec. 2.1, the range is not changed in this method. The result of RangeLPS derivation for the next bin can be reused. The result will not be changed after the coding interval adjustment.

Figure 4.  Concept of proposed coding interval adjustment procedure by using bypass bin

# Simulation results

The proposed methods are implemented on HM-5.0. All the results were simulated according to the common test conditions defined in JCTVC-G1200 [2]. Table 1 shows the results of proposed method 1 (selecting larger range intervals) when maxBO is set to 12, which means at most 18 bits can be outputted together by the renormalization process. It can be seen that there is no coding efficiency loss. Table 2 shows the results of proposed method 2 (sending one bypass bin) when maxBO is set to 12. No coding efficiency loss is observed while at most 18 bits can be outputted together by the renormalization process. The run times of proposed methods are roughly the same as the anchor.

# Conclusions

After the renormalization process, CABAC could suddenly output an infinite number of bits in theory and hundreds of bits in practical due to continuous outstanding bits. Two coding interval adjustment procedures are proposed to constrain the bit count of continuous outstanding bits. It is reported that there is no change in coding efficiency while at most 18 bits could be outputted at the same time.

# References

- J. Sole, R. Joshi, M. Karczewicz, “CABAC with constrained outstanding bits,” Document of Joint Collaborative Team on Video Coding, JCTVC-G323, November 2011.

- F. Bossen, “Common HM test conditions and software reference configurations,” Document of Joint Collaborative Team on Video Coding, JCTVC-G1200, November 2011.

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Table 1.  Results of the proposed method 1

Table 2.  Results of the proposed method 2