| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 8th Meeting: San José, CA, USA, 1–10 February, 2012 | Document: JCTVC-H0272 |
| --- | --- |

| Title: | CE8.a.1: One-stage SAO and ALF with LCU-based syntax |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Ching-Yeh Chen, Chia-Yang Tsai, Chih-Ming Fu, Yu-Wen Huang, Shawmin Lei MediaTek No. 1, Dusing Rd. 1, Hsinchu Science Park, Hsinchu, Taiwan 30078 In Suk Chong, Marta Karczewicz Qualcomm 5775 Morehouse Dr, San Diego, CA 92121, USA Tomoo Yamakage, Takayuki Itoh, Takashi Watanabe, Takeshi Chujoh TOSHIBA 1, Komukai-Toshiba-cho, Saiwai-ku, Kawasaki, 212-8582, Japan | Tel: Email: | Shawmin Lei +886-3-5670766 ext. 25555 {chingyeh.chen, chiayang.tsai chihming.fu, yuwen.huang, shawmin.lei}@mediatek.com Marta Karczewicz +1-858-658-5673 {ichong, martak}@qualcomm.com Tomoo Yamakage +81-44-549-2152 {tomoo.yamakage, takayuki2.itoh, takashi39.watanabe, takeshi.chujoh}@toshiba.co.jp |
| Source: | MediaTek Inc., Qualcomm Inc., TOSHIBA Corporation |  |  |

_____________________________

# Abstract

In HM-5.0, SAO and ALF use picture-based syntax designs and are separated into two stages. In this contribution, SAO and ALF are combined into one-stage adaptive loop filter (OSALF). OSALF uses LCU-based syntax and has the flexibility to support picture-based and LCU-based encoding. It is reported that in comparison with the JCTVC-G1200 anchors, luma BD-rates of OSALF are 0.0%, 0.0%, 0.2%, and 0.3% for HE-AI, HE-RA, HE-LDB, and HE-LDP, respectively, excluding class F, when a picture-based encoding algorithm is used. The corresponding luma BD-rates become 0.9%, 0.9%, 1.5%, and 2.0% when an LCU-based encoding algorithm is used, which preserves 63%, 82%, 72%, and 87% gains of HM-5.0 SAO and ALF.

# Introduction

In High-Efficiency Video Coding test model version 5.0 (HM-5.0), sample adaptive offset (SAO) and adaptive loop filter (ALF) use picture-based syntax and are separated into two stages. In JCTVC-D119 [1] and JCTVC-G218 [2], SAO and ALF are combined into one-stage adaptive loop filter (OSALF). In JCTVC-F056 [3], JCTVC-G499 [4], and JCTVC-G831 [5], largest coding unit (LCU) based syntax designs are used in order to provide the flexibility to support both picture-based encoding and LCU-based encoding, where the picture-based encoding can achieve better coding efficiency and the LCU-based encoding can use less computing resource and achieve sub-picture latency. In this proposal, OSALF is studied on HM-5.0. The rest of this contribution is organized as follows. The proposed algorithms will be introduced in Section 2, simulation results will be shown in Section 3, and conclusions will be given in Section 4.

# Algorithm description

## One-stage adaptive loop filter

Figure 1 shows the two-stage adaptive loop filter (TSALF) configuration that clearly separates SAO and ALF as adopted in HM-5.0. Figure 2 shows the proposed one-stage adaptive loop filter (OSALF), which allows performing SAO pixel classification and ALF block classification in parallel. In OSALF, local adaptation is allowed at LCU level, and the SAO and ALF can be independently enabled or disabled. When both SAO and ALF are enabled, each LCU can switch among ALF+SAO (Figure 2), SAO (Figure 2, but bypassing ALF filtering and pixel classification), and OFF (completely bypassing all processes in Figure 2).

Figure 1.  Two-stage adaptive loop filter

Figure 2.  One-stage adaptive loop filter

## LCU-based syntax

Sending non-deblocking loop filtering (NDLF) parameters for each LCU is allowed to achieve LCU-based adaptation. In order to reduce side information when multiple LCUs on the same row share the same NDLF parameters, run coding is used, where a run of a current LCU means the number of LCUs that are after the current LCU in scan order and have the same NDLF parameters as the current LCU. Moreover, the run can be predicted from its upper LCU to further reduce bits. When one LCU has different NDLF parameters from its left LCU (i.e. run is zero), a merge-up flag is signaled to allow copying NDLF parameters from its upper LCU. New NDLF parameters can only be coded when the run and the merge-up flag are zero. When the ALF+SAO mode is considered, syntax-wise one LCU can also select a previous (space-delayed) filter stored in a filter buffer.

## Picture-based encoding

The picture-based encoding algorithm is a bottom-up quadtree method, which is similar to the method used in HM-5.0. The NDLF information is coded in adaptation parameter set (APS). In order to reduce the worst case size of APS, the maximum number of offsets per color component per picture and the maximum number of filters per color component per picture are set as 1024 and 16, respectively, and proper values can be discussed during profile and level definitions.

## LCU-based encoding

The LCU-based encoding algorithm selects one set of NDLF parameters derived from the current LCU pixels, the left LCU, the upper LCU, and the filter buffer. The NDLF information is coded in APS when single-slice per picture encoding is used. The maximum number of offsets per color component per picture and the maximum number of filters per color component per picture are set as 2048 and 128, respectively. In order to avoid storing all pixels of one LCU row, the last four pixel rows in each LCU are not used during NDLF parameter estimation because they are not yet processed by deblocking until the lower LCU comes in practical LCU-based encoding systems.

## Sub-picture latency encoding

To achieve sub-picture latency encoding, the LCU-based encoding algorithm must be used. In addition, it is necessary to enable multi-slice per picture encoding and send NDLF information in slice headers. Syntax-wise, one flag is added in sequence parameter set (SPS) to indicate APS or slice headers are used to convey NDLF information. Moreover, in order to parse each slice independently, the number of LCUs is also coded in the slice header. Although only cross-slices loop filtering and LCU-aligned slices are mandatory in CE8.a, non-cross-slices loop filtering and non-LCU-aligned slices are also supported, and run coding and merge-up flags are properly handled at slice boundaries.

# Simulation results

Table 1 shows results of OSALF with LCU-based syntax, picture-based encoding, single-slice per picture, and anchor using JCTVC-G1200 [6]. The luma BD-rates excluding class F are 0.0%, 0.0%, 0.2%, and 0.3% for HE-AI, HE-RA, HE-LDB, and HE-LDP, respectively. It can be seen that the proposed LCU-based OSALF syntax can provide similar coding efficiency as the picture-based NDLF syntax in HM-5.0.

Table 2 shows results of OSALF with LCU-based syntax, LCU-based encoding, single-slice per picture, and anchor using JCTVC-G1200. The luma BD-rates excluding class F are 0.9%, 0.9%, 1.5%, and 2.0% for HE-AI, HE-RA, HE-LDB, and HE-LDP, respectively. The capability of supporting LCU-based OSALF encoding is verified. Moreover, according to the corresponding luma BD-rates of disabling SAO and ALF in HE (2.4%, 5.1%, 5.3%, 15.3%), the proposed LCU-based OSALF encoding preserves 63%, 82%, 72%, and 87% gains of the SAO and ALF in HM-5.0.

Table 3 shows the results of OSALF with LCU-based syntax, LCU-based encoding, multi-slice per picture, and anchor using JCTVC-G1200 with multi-slice per picture and disabling SAO and ALF. The slice settings are cross-slices loop filtering, LCU-aligned slices, and fixed numbers of LCUs per slices, which are 64, 104, 28, 60, and 96 for 1920x1080, 832x480, 416x240, 1280x720, and 1024x768, respectively. AI and RA conditions are skipped. The luma BD-rates are -3.1% and -10.5% for HE-LDB and HE-LDP, respectively, and the capability of achieving sub-picture encoding latency is verified.

Table 4 shows the same test as Table 3 with a different anchor, which is JCTVC-G1200 with multi-slice per picture, SAO-on, and ALF-off. The luma BD-rates are -0.7% and -1.4% for HE-LDB and HE-LDP, respectively. The run time data in Table 4 are not accurate.

# Conclusions

MediaTek, Qualcomm, and Toshiba proposed LCU-based SAO and ALF syntax, which can support both picture-based and LCU-based encoding methods. Moreover, a parallel configuration that can combine SAO and ALF into one-stage is also introduced. It is reported that in comparison with the picture-based SAO and ALF in HM-5.0, the proposed picture-based encoding has similar coding efficiency, and the proposed LCU-based encoding preserves 67-88% gains of the SAO and ALF in HM-5.0.

# Patent rights declarations

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Qualcomm Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

TOSHIBA Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

- C.-Y. Chen, C.-M. Fu, C.-Y. Tsai, Y.-W. Huang, S. Lei, M. Karczewicz, I. S. Chong, T. Yamakage, T. Chujoh, T. Watanabe, “CE8 Subset2: A joint proposal on improving the adaptive loop filter in TMuC0.9 by MediaTek, Qualcomm, and Toshiba,” Document of Joint Collaborative Team on Video Coding, JCTVC-D119, January 2011.

- C.-Y. Chen, C-Y Tsai, C.-M. Fu, Y.-W. Huang, S. Lei, I. S. Chong, M. Karczewicz, T. Yamakage, T. Itoh, T. Watanabe, T. Chujoh “Non-CE8: One-stage non-deblocking loop filtering,” Document of Joint Collaborative Team on Video Coding, JCTVC-G218, November 2011.

- C.-M. Fu, C.-Y. Chen, C-Y Tsai, Y.-W. Huang, S. Lei, I. S. Chong, M. Karczewicz, “Sample adaptive offset with LCU-based syntax,” Document of Joint Collaborative Team on Video Coding, JCTVC-F056, July 2011.

- A. Fuldseth, G. Bjøntegaard, “Improved ALF with low latency and reduced complexity,” Document of Joint Collaborative Team on Video Coding, JCTVC-G499, November 2011.

- C.-M. Fu, C.-Y. Chen, C-Y Tsai, Y.-W. Huang, S. Lei, I. S. Chong, M. Karczewicz, “Non-CE8: Sample adaptive offset with LCU-based syntax,” Document of Joint Collaborative Team on Video Coding, JCTVC-G831, November 2011.

- F. Bossen, “Common HM test conditions and software reference configurations,” Document of Joint Collaborative Team on Video Coding, JCTVC-G1200, November 2011.

Table 1.  Results of OSALF with LCU-based syntax, picture-based encoding, single-slice per picture, and anchor using JCTVC-G1200

Table 2.  Results of OSALF with LCU-based syntax, LCU-based encoding, single-slice per picture, and anchor using JCTVC-G1200

Table 3.  Results of OSALF with LCU-based syntax, LCU-based encoding, multi-slice per picture, and anchor using JCTVC-G1200 with multi-slice per picture and disabling SAO and ALF

Table 4.  Results of OSALF with LCU-based syntax, LCU-based encoding, multi-slice per picture, and anchor using JCTVC-G1200 with multi-slice per picture, SAO-on, and ALF-off (run time data not accurate)