#### Sequence parameter set RBSP syntax

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| .... |  |
| adaptive_loop_filter_enabled_flag | u(1) |
| if (adaptive_loop_filter_enabled_flag ){ |  |
| adaptive_loop_filter_present_flag | u(1) |
| alf_mode_filt_enable_flag | u(1) |
| alf_mode_sao_enable_flag | u(1) |
| } |  |
| ... |  |
| rbsp_trailing_bits( ) |  |
| } |  |

#### Adaptive loop filter parameter syntax

| sao_alf_param() { | Descriptor |
| --- | --- |
| alf_enable_flag[0] = 1 |  |
| alf_enable_flag[1] | u(1) |
| alf_enable_flag[2] | u(1) |
| for (i=0; i<3; i++){ |  |
| if ( alf_enable_flag[i] ) |  |
| alf_one_unit_flag[i] | u(1) |
| else |  |
| alf_one_unit_flag[i] = 1 |  |
| } |  |
| num_lcu = num_Lcu_in_width * num_Lcu_in_height |  |
| if (adaptive_loop_filter_present_flag) { |  |
| num_lcu = alf_num_lcu_minus_1 + 1 | u(v) |
| } |  |
| for (i=0; i< num_lcu; i++) { |  |
| rx = (i + first_lcu_in_slice_addr) % num_Lcu_in_width |  |
| ry = (i + first_lcu_in_slice_addr) / num_Lcu_in_width |  |
| sao_alf_unit ( rx, ry, 0 ) |  |
| if ( alf_cb_enable_flag ) |  |
| sao_alf_unit ( rx, ry, 1 ) |  |
| if ( alf_cr_enable_flag ) |  |
| sao_alf_unit ( rx, ry, 2 ) |  |
| } |  |
| } |  |

| sao_alf_unit( rx, ry, c) { | Descriptor |
| --- | --- |
| if (alf_one_unit_flag[c] ) { |  |
| if (rx==0 && ry == 0) { |  |
| alf_mode_index [ c ] [ ry ] [ rx ] | ue(v) |
| if (mode_index [ c ] [ ry ] [ rx ] == MODE_FILT){ |  |
| sao_alf_filt_info ( rx, ry, c ) |  |
| } else if (mode_index [ c ] [ ry ] [ rx ] == MODE_SAO){ |  |
| sao_alf_sao_info ( rx, ry, c ) |  |
| } |  |
| } |  |
| } else { |  |
| if ( rx == 0 && ry > 0 ) { |  |
| repeat_row_flag [ c ] [ ry ] | u(1) |
| } else if ( rx==0 && ry == 0 ) { |  |
| repeat_row_flag [ c ] [ ry ] = 0 |  |
| } |  |
| if ( !repeat_row_flag [ c ] [ ry ] ) { |  |
| if ( rx ==0 ¦¦ run [ c ] [ ry ][ rx ] < 0 ) { |  |
| if ( ry == 0 ) { |  |
| run [ c ] [ ry ][ rx ] = 0 + run_diff [ c ] [ ry ][ rx ] | u(v) |
| } else if ( rx == num_Lcu_in_width - 1) { |  |
| run [ c ] [ ry ][ rx ] = 0 |  |
| } else { |  |
| run [ c ] [ ry ][ rx ] = run [ c ] [ ry - 1 ][ rx ] + run_diff [ c ] [ ry ][ rx ] | s(v) |
| } |  |
| } |  |
| if (rx < num_Lcu_in_width – 1 ) { |  |
| for ( k = 0; k <= run [ c ] [ ry ] [ rx ]; k++ ) { |  |
| run [ c ] [ ry ] [ rx+k+1 ] = run [ c ] [ ry ] [ rx+k ] – 1 |  |
| } |  |
| } |  |
| merge_up_flag [ c ] [ ry ] [ rx ] = 0 |  |
| if ( ry > 0 ) |  |
| merge_up_flag [ c ] [ ry ] [ rx ] |  |
| if ( !merge_up_flag [ c ] [ ry ] [ rx ] ) { |  |
| alf_mode_index [ c ] [ ry ] [ rx ] | ue(v) |
| if (mode_index [ c ] [ ry ] [ rx ] == MODE_FILT){ |  |
| sao_alf_filt_info ( rx, ry, c ) |  |
| } else if (mode_index [ c ] [ ry ] [ rx ] == MODE_SAO){ |  |
| sao_alf_sao_info ( rx, ry, c ) |  |
| } |  |
| } |  |
| } else { |  |
| run [ c ] [ ry ] [ rx ] = run [ c ] [ ry – 1 ] [ rx ] |  |
| } |  |
| } |  |
| } |  |

| sao_alf_filt_info ( rx, ry, c) { | Descriptor |
| --- | --- |
| alf_new_filter_set | u(1) |
| if ( alf_new_filter_set == false ) { |  |
| alf_stored_filter_set_idx | u(v) |
| } else { |  |
| alf_filter_shape_flag[ c ][ ry ][ rx ] | u(1) |
| if ( c == 0 ) { |  |
| alf_no_filters_minus1 | ue(v) |
| if (alf_no_filters_minus1 == 1) |  |
| alf_start_second_filter | ue(v) |
| else if (alf_no_filters_minus1 > 1) { |  |
| for (i=1; i< 15; i++) |  |
| alf_filter_pattern[ c ][ ry ][ rx ] [i] | u(1) |
| } |  |
| alf_filt_offset_type[ c ][ ry ][ rx ] | u(3) |
| if (AlfNumFilters > 1) |  |
| alf_pred_method[ c ][ ry ][ rx ] | u(1) |
| for (i=0; i< AlfNumFilters; i++) |  |
| alf_nbSPred [ c ][ ry ][ rx ] [i] | u(1) |
| if (AlfNumFilters > 1) { |  |
| alf_min_kstart_minus1 | ue(v) |
| for (i=0; i < AlfMaxDepth; i++) |  |
| alf_golomb_index_bit[i] | u(1) |
| } |  |
| for (i=0; i< AlfNumFilters; i++) { |  |
| for (j=0; j< AlfCodedLength; j++) |  |
| alf_filt_coeff[ c ][ ry ][ rx ][ i ][ j ] | ge(v) |
| for( k = 0; k< AlfFiltOffsetLength; k++ ) |  |
| alf_filt_offset[ c ][ ry ][ rx ][ i ][ k ] | ge(v) |
| } |  |
| } else { |  |
| alf_filt_offset_type[ c ][ ry ][ rx ] | u(3) |
| for (j=0; j< AlfCodedLength; j++) |  |
| alf_filt_coeff[ c ][ ry ][ rx ][ j ] | se(v) |
| for( k = 0; k< AlfFiltOffsetLength; k++ ) |  |
| alf_filt_offset[ c ][ ry ][ rx ][ k ] | se(v) |
| } |  |
| } |  |
| } |  |

| sao_alf_sao_info ( rx, ry, c ){ | Descriptor |
| --- | --- |
| alf_sao_submode[ c ][ ry ][ rx ] | ue(v) |
| for( i = 0; i <SAOOffsetLength; i++ ) |  |
| alf_sao_offset[ c ][ ry ][ rx ][ i ] | se(v) |
| } |  |

#### Adaptive loop filter parameter semantics

adaptive_loop_filter_present_flag equal to 1 specifies that the bitstream of adaptive loop filter shall be located at slice header; equal to 0 specifies that the bitstream of adaptive loop filter shall be located at APS.

alf_mode_filt_enable_flag equal to 1 specifies that MODE_FILT shall be allowed in adaptive loop filter; equal to 0 specifies that MODE_FILT shall not be allowed in adaptive loop filter.

alf_mode_sao_enable_flag equal to 1 specifies that MODE_SAO shall be allowed in adaptive loop filter; equal to 0 specifies that MODE_SAO shall not be allowed in adaptive loop filter.

alf_enable_flag[i] equal to 1 specifies that adaptive loop filter shall be applied on the color component i; equal to 0 specifies that adaptive loop filter shall be skipped on the color component i.

alf_one_unit_flag[i] equal to 1 specifies that all of LCUs in the color component i of one slice shall be filtered by the same ALF parameter set; equal to 0 specifies that ALF parameter sets in the color component i shall be adaptive LCU by LCU.

alf_num_lcu_minus_1 specifies the number of LCUs in one slice. The number of LCUs in one slice is equal to alf_num_lcu_minus_1 + 1.

alf_mode_index [ c ] [ ry ] [ rx ] specifies that the Alf mode of the LCU with indices rx and ry for the color component c.

Table 7-1 – Specification of alf_mode_index

| - alf_mode_index | - Mode of ALF |
| --- | --- |
| - 0 | - MODE_SAO |
| - 1 | - MODE_OFF |
| - 2 | - MODE_FILT |

repeat_row_flag [ c ] [ ry ] equal to 1 specifies that the Alf parameter sets of LCUs in the LCU row ry shall be the same as those of above LCUs for the color component c and no more information shall be decoded for the LCU row ry; equal to 0 specifies that Alf parameter sets of LCUs in the LCU row ry of the color component c shall be decoded.

run [ c ] [ ry ][ rx ] indicates the number of following LCUs to be applied with the ALF parameter set of current LCU with indices rx and ry of the color component c.

run_diff [ c ] [ ry ][ rx ] indicates the difference between the run of current LCU with indices rx and ry of the color component c and the run of the above LCU with indices rx and ry-1 of the color component c.

merge_up_flag [ c ] [ ry ][ rx ] equal to 1 denotes current LCU with indices rx and ry of the color component c uses the ALF parameter set of the above LCU with indices rx and ry-1 of the color component c; equal to 0 denotes that current LCU has one new ALF parameter set.

alf_stored_filter_set_idx equal to 1 denotes current LCU with indices rx and ry of the color component c uses the stored filter set with the index equal to alf_stored_filter_set_idx of the color component c; equal to 0 denotes that current LCU has one new Alf filter set. The stored filter sets shall be reset before parsing one slice, and it shall be updated when one new Alf filter set is decoded.

alf_filter_shape_flag [ c ] [ ry ][ rx ] equal to 0 denotes that the filter shape 0 shall be used in this ALF parameter set; equal to 1 denotes that the filter shape 1 shall be used for current LCU with indices rx and ry of the color component c. The number of coded filter coefficients AlfCodedLength is equal to 9 or 9 for filter shape 0 or filter shape 1, respectively. The number of filter coefficients AlfLength is equal to 17 or 17 for filter shape 0 or filter shape 1, respectively.

alf_filter_pattern[ c ] [ ry ][ rx ] [ i ] specifies the filter index array corresponding to i-th variance index for current LCU with indices rx and ry of the color component c, when alf_no_filters_minus1 is greater than 1. The number of filter sets AlfNumFilters is derived as follows:

- If alf_no_filters_minus1 is less than 2,

AlfNumFilters = alf_no_filters_minus1 + 1		(7-1)

- Otherwise (alf_no_filters_minus1 is greater than 2)

AlfNumFilters = i alf_filter_pattern[ i ] with i = 0..15	(7-2)

alf_filt_offset_type[ c ][ ry ][ rx ] indicates the offset type for the LCU with indices rx and ry of the color component c.

alf_pred_method[ c ] [ ry ][ rx ]  specifies whether the filter coefficients are coded in a predictive way for current LCU with indices rx and ry of the color component c.

alf_min_kstart_minus1 plus 1 specifies the minimum order k of k-th order exponential golomb code for the luma filter coefficients for the adaptive loop filter.

alf_golomb_index_bit specifies the difference in order k of k-th order exponential golomb code for the different groups of the luma filter coefficients. Note that there are several groups of the luma filter coefficients where each group may have different order k. The default value of k-th order exponential golomb code is specified in Table 7-2.

Table 7-2 The default value for the k-th order of i-th Coded Coeff

|  | i-th Coded Coeff |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Filter shape | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 |
| 0 | 1 | 3 | 1 | 3 | 4 | 3 | 3 | 4 | 1 |
| 1 | 1 | 2 | 3 | 4 | 1 | 2 | 3 | 4 | 1 |

| 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 | 20 | 21 | 22 | 23 | 24 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 3 | 1 | 1 | 3 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 |
| 3 | 1 | 1 | 3 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 |

alf_filt_coeff[ c ] [ ry ][ rx ] [ i ][ j ] specifies the j-th filter coefficient of i-th filter for the adaptive loop filtering process for current LCU with indices rx and ry of the color component c.

alf_filt_offset[ c ][ ry ][ rx ][ i ][ k ] specifies the k-th filter offset of i-th filter for the adaptive loop filtering process for current LCU with indices rx and ry of the color component c.

An array AlfFiltOffsetVal is specified as

- AlfFiltOffsetVal[ c ] [ ry ][ rx ][ 0 ] = 0		(7-3)

- AlfFiltOffsetVal[ c ] [ ry ][ rx ][ i + 1 ] =

- alf_filt_offset[ c ] [ ry ][ rx ][ i ] << (ALF_NUM_BIT_SHIFT – 2)

- with i = 0..NumAlfOffsetCategory – 1		(7-4)

- The number of categories, NumAlfOffsetCategory , is specified in Table 7-2.

- Table 7-2 – Specification of AlfSaoMode

| - alf_filt_offset_type [ c ] [ ry ][ rx ] | - NumAlfOffsetCategory | - Edge type (informative) |
| --- | --- | --- |
| - 0 | - 4 | - 1D 0-degree edge |
| - 1 | - 4 | - 1D 90-degree edge |
| - 2 | - 4 | - 1D 135-degree edge |
| - 3 | - 4 | - 1D 45-degree edge |
| - 4 | - 16 | - Central band |
| - 5 | - 16 | - Side band |

alf_sao_submode [ c ] [ ry ][ rx ] indicates the offset type for the LCU with indices rx and ry of the color component c.

alf_sao_offset[ c ][ ry ][ rx ][ i ] indicates the offset value of i-th category for the LCU with indices rx and ry of the color component c.

The variable bitDepth is derived as follows.

- If c is equal to 0, bitDepth is set equal to BitDepthY..

- Otherwise (c is equal to1 or 2), bitDepth is set equal to BitDepthC.

The offset value shall be in the range of [ -( 1 << ( SaoBitsRange – 1) ), ( 1<< ( SaoBitsRange – 1) ) – 1 ] where

SaoBitRange = Min( bitDepth, 10 ) – 4		(7-5)

An array AlfSaoOffsetVal is specified as

AlfSaoOffsetVal[ c ] [ ry ][ rx ][ 0 ] = 0		(7-6)

AlfSaoOffsetVal[ c ] [ ry ][ rx ][ i + 1 ] =

alf_sao_offset[ c ] [ ry ][ rx ][ i ] << ( bitDepth – Min( bitDepth, 10 ) )

with i = 0..NumAlfSaoCategory – 1		(7-7)

The number of categories, NumAlfSaoMode, is specified in Table 7-2

Table 7-2 – Specification of AlfSaoMode

| - alf_sao_submode [ c ] [ ry ][ rx ] | - NumAlfSaoCategory | - Edge type (informative) |
| --- | --- | --- |
| - 0 | - 0 | - Not applied |
| - 1 | - 4 | - 1D 0-degree edge |
| - 2 | - 4 | - 1D 90-degree edge |
| - 3 | - 4 | - 1D 135-degree edge |
| - 4 | - 4 | - 1D 45-degree edge |
| - 5 | - 16 | - Central band |
| - 6 | - 16 | - Side band |

### Adaptive loop filter process

An adaptive loop filtering process shall be conditionally performed on a treeblock basis after the completion of the deblocking filtering process for the entire decoded picture, with all treeblocks in a picture processed in order of increasing treeblock addresses.

Each treeblock is processed on a coding unit basis with the same order as decoding process.

This process is invoked when both adaptive_loop_filter_enabled_flag and alf_enable_flag[0] are equal to 1.

This process is performed on a coding unit basis after the completion of the slice construction process prior to adaptive loop filter process for the entire decoded slice, with all coding units in a slice processed in order of coding unit scan order.

The current coding unit with the luma location ( xC, yC ) specifying the top-left luma sample is belonged to the current LCU with indices rx and ry, and the Alf parameter set of this current LCU, including alf_mode_index[c][ry][rx], alf_filter_pattern[c][ry][rx], alf_coeff_luma[c][ry][rx], alf_coeff_chroma[c][ry][rx], alf_sao_submode[c][ry][rx], and alf_sao_offset[c][ry][rx], shall be used in the following process for three color components, respectively. When the Alf mode of current coding unit alf_mode_index[c][ry][rx] is equal to MODE_FILT, Alf filtering process shall be applied including filter coefficient derivation specified in subclause 8.6.3.22, filter index derivation for luma specified in subclause 8.6.3.23, filtering process for luma specified in subclause 8.6.3.24, and filtering process for chroma specified in subclause 8.6.3.25. When the Alf mode of current coding unit alf_mode_index[c][ry][rx] is equal to MODE_SAO, Alf offset process shall be applied and the processes specified in subclause 8.6.3.6 is invoked. Besides, there are three cases to disable Alf process. The first one is that pcm_loop_filter_disable_flag is equal to 1 and pcm_flag is equal to 1. The second one is that the Alf mode of current coding unit alf_mode_index[c][ry][rx] is equal to MODE_OFF. The last one is alf_enable_flag[c] is equal to zero. In these three cases, no Alf process shall be applied and the output data are equal to the input data.

When interpreting the luma samples of the coding unit as to be filtered in MODE_FILT, depending on alf_cu_control_flag, the following applies.

- If alf_cu_control_flag is equal to 0, the luma samples are filtered,

- Otherwise (alf_cu_control_flag is equal to 1), if AlfCuFlag[ xC ][ yC ] is equal to 1 where xC and yC are the top-left luma sample of the current coding unit relative to the top left luma sample of the current picture, the luma samples are filtered.

- Otherwise, the luma samples are not filtered.

For the luma samples of the coding unit interpreted as to be filtered in MODE_FILT, the filtering process for luma samples specified in subclause Error: Reference source not found is invoked with the luma location ( xC, yC ) specifying the top-left luma sample of the current coding unit relative to the top left luma sample of the current picture, the coding unit size log2CUSize, and the filter index array fIdx as inputs and the output is the modified filtered picture, recFiltPictureL.

For the chroma samples of the coding unit interpreted as to be filtered in MODE_FILT, the filtering process for chroma samples specified in subclause Error: Reference source not found is invoked with the chroma location ( xC/2, yC/2 ) specifying the top-left chroma sample of the current coding unit relative to the top left chroma sample of the current picture, the coding unit size log2CUSize-1 and the chroma component index cIdx equal to 1 as inputs and the output is the modified filtered picture, recFiltPictureCb.

For the chroma samples of the coding unit interpreted as to be filtered in MODE_FILT, the filtering process for chroma samples specified in subclause Error: Reference source not found is invoked with the chroma location ( xC/2, yC/2 ) specifying the top-left chroma sample of the current coding unit relative to the top left chroma sample of the current picture, the coding unit size log2CUSize-1 and the chroma component index cIdx equal to 2 as inputs and the output is the modified filtered picture, recFiltPictureCr.

For the sample of the coding unit interpreted as to be filtered in MODE_SAO, the Alf offset process specified in subclause 8.6.3.5 is invoked with the location ( xC, yC ) for luma or ( xC/2, yC/2 ) for chroma specifying the top-left sample of the current coding unit relative to the top left sample of the current picture, the coding unit size log2CUSize for luma or log2CUSize-1 for chroma, and the color component c as inputs and the output is the modified filtered picture, recFiltPicture.

[Ed.: (WJ) recPicture: deblocked/output picture and recFiltPicture: ALFed picture]

[Ed.: (WJ) depending aps_adaptive_loop_filter_flag, recFiltPicture should be copied to recPicture in subclause Error: Reference source not found]

#### Derivation process for filter coefficients

Inputs to this process is the color component c and outputs of this process are filter coefficients cL for the luma samples or filter coefficients cC for the chroma samples. AlfCodedLength and AlfLength shall be decided based on alf_filter_shape_flag [ c ] [ ry ][ rx ].

If the color component c is equal to zero, then the luma filter coefficients cL with elements cL[ i ][ j ], i = 0..AlfNumFilters – 1, j = 0..AlfCodedLength – 1 is derived as follows:

- If alf_pred_method is equal to 0 or the value of i is equal to 0,

cL[ i ][ j ] = alf_filt_coeff [ c ][ ry ][ rx ][ i ][ j ]		(8-8)

- Otherwise (alf_pred_method is equal to 1 and the value of i is greater than 1),

cL[ i ][ j ] = alf_filt_coeff [ c ][ ry ][ rx ][ i ][ j ] + cL[ i – 1 ][ j ]	(8-9)

Considering the symmetry of the filter, the luma filter coefficients cL with elements cL[ i ][ j ], i = 0..AlfNumFilters – 1 is derived as follows:

cL[ i ][ AlfLength – 1 ] = cL[ i ][ AlfCodedLength – 1 ]	(8-10)

cL[ i ][ AlfLength – 2 – j ] = cL[ i ][ j ] for j = 0..AlfCodedLength – 2	(8-11)

Otherwise (the color component c is equal to 1 or 2), the chroma filter coefficients cC with elements cC[ i ], i = 0..AlfCodedLength – 1 is derived as follows:

- If i is equal to AlfCodedLength – 1, the coefficient cC[i] is derived as

cC[ i ] = 256 – sum – alf_filt_coeff [ c ][ ry ][ rx ][ i ]	(8-12)

where

sum = alf_filt_coeff [ AlfCodedLength – 2 ] + j( alf_filt_coeff [ j ] << 1 ) 	(8-13)

with j = 0..AlfCodedLength – 3

- Otherwise (i is less than AlfCodedLength – 1),

cC[ i ] = alf_filt_coeff [ c ][ ry ][ rx ][ i ]		(8-14)

Considering the symmetry of the filter, the chroma filter coefficients cC with elements cC[ i ], i = 0..AlfLength – 1 is derived as follows:

cC[ i ][ AlfLength – 1 ] = cC[ i ][ AlfCodedLength – 1 ]	(8-15)

cC[ i ][ AlfLength – 2 – j ] = cC[ i ][ j ] for j = 0..AlfCodedLength – 2	(8-16)

8.6.3.4     Filtering process for luma samples

Inputs of this process are:

–	an array cL specifying filter coefficients,

–	an array AlfFiltOffsetVal specifying offset values,

–	a luma location ( xC, yC ) specifying the top-left luma sample of the current coding unit relative to the top left luma sample of the current picture,

–	a variable log2CUSize specifying the size of the current coding unit,

–	a filter index array of (nS)x(nS), fIdx.

Output of this process is the filtered reconstruction of luma picture.

The boundary padding process specified in subclause Error: Reference source not found is invoked with the luma location ( xC, yC ), the size of coding unit log2CUSize and the chroma component index cIdx set equal to 0, and the output is assigned to the luma sample array s’’. [Ed. (WJ): s’’ is now a picture-size array, but actually CU size + appropriate margin is enough]

Let recFiltPicture represents the processed sample array of the current picture of luma and alfOffsetType is set equal to alf_filt_offset_type [ c ][ ry ][ rx ].

Depending on the value of alfOffsetType, the variable alfOffset is derived by following:

–	If alfOffsetType is equal to one of the values of 0, 1, 2, 3, the following ordered steps apply:

- Arrays hPos[2] and vPos[2] are specified in Table 8-1.

- A variable edgeIdx is specified as

edgeIdx = 2 + ∑k( Sign( recPicture[ xC + i, yC + j ] –

recPicture[ xC + i + hPos[ k ], yC + j + vPos[ k ] ] ) ) with k = 0..1	(8-17)

- The variable alfOffset is derived as

alfOffset = AlfFiltOffsetVal [ edgeTable[ edgeIdx ] ]	(8-18)

with i = 0..nS-1 and j = 0..nS-1 where 	edgeTable[5] = { 1, 2, 0, 3, 4}.

–	Otherwise, if alfOffsetType is equal to one of the values of 4 or 5, the variable alfOffset is derived by following:

- A variable bandShift is set equal to BitDepthY – 5

- The variable alfOffset is derived as

alfOffset = AlfFiltOffsetVal [ bandTable[ alfSaoType – 5 ][ bandIdx ] ]	(8-19)

with i = 0..nS-1 and j = 0..nS-1 where bandIdx is set equal to ( recPicture[ xC + i, yC + j ] >> bandShift ) and bandTable is specified in Table 8-2.

A variable nS is set equal to ( 1 << log2CUSize ) and a variable alfTap is set equal to ( alf_length_luma_minus_5_div2 << 1 ) + 5.

Each sample of luma picture recFiltPictureL[ xC + x ][ yC + y ] with x, y = 0..(nS)-1, is derived as follows.

recFiltPictureL[xC+x][yC+y]i=0N−1s''[xC+x+horPos[i],yC+y+verPos[i]]∗cL[fIdx[x][y]][i]+alfOffset	(8-20)

where N = AlfNumCoeffLuma-1, ALF_NUM_BIT_SHIFT = 8, and horPos[i] and verPos[i] are specified in Error: Reference source not found and Error: Reference source not found, respectively.

8.6.3.5     Filtering process for chroma

Inputs of this process are:

–	an array cL specifying filter coefficients,

–	an array alfFiltOffsetVal specifying offset values,

–	a chroma location ( xC, yC ) specifying the top-left luma sample of the current coding unit relative to the top left chroma sample of the current picture,

–	a variable log2CUSize specifying the size of the current coding unit.

–	a variable cIdx specifying the chroma component index.

Output of this process is the filtered reconstruction of chroma picture.

The boundary padding process specified in subclause Error: Reference source not found is invoked with the chroma location ( xC, yC ), the size of coding unit log2CUSize and the chroma component index cIdx, and the output is assigned to the luma sample array s’’. [Ed. (WJ): s’’ is now a picture-size array, but actually CU size + appropriate margin is enough]

Let recFiltPicture represents the processed sample array of the current picture of luma and alfOffsetType is set equal to alf_filt_offset_type [ c ][ ry ][ rx ].

Depending on the value of alfOffsetType, the variable alfOffset is derived by following:

–	If alfOffsetType is equal to one of the values of 0, 1, 2, 3, the following ordered steps apply:

- Arrays hPos[2] and vPos[2] are specified in Table 8-1.

- A variable edgeIdx is specified as

edgeIdx = 2 + ∑k( Sign( recPicture[ xC + i, yC + j ] –

recPicture[ xC + i + hPos[ k ], yC + j + vPos[ k ] ] ) ) with k = 0..1	(8-21)

- The variable alfOffset is derived as

alfOffset = alfOffsetVal [ edgeTable[ edgeIdx ] ]	(8-22)

with i = 0..nS-1 and j = 0..nS-1 where 	edgeTable[5] = { 1, 2, 0, 3, 4}.

–	Otherwise, if alfOffsetType is equal to one of the values of 4 or 5, the variable alfOffset is derived by following:

- A variable bandShift is set equal to BitDepthY – 5

- The variable alfOffset is derived as

alfOffset = alfOffsetVal [ bandTable[ alfSaoType – 5 ][ bandIdx ] ]	(8-23)

with i = 0..nS-1 and j = 0..nS-1 where bandIdx is set equal to ( recPicture[ xC + i, yC + j ] >> bandShift ) and bandTable is specified in Table 8-2.

A variable nS is set equal to ( 1 << log2CUSize ) and a variable alfTapChroma is set equal to ( alf_length_chroma_minus_5_div2 << 1 ) + 5.

Filtered samples of chroma picture recFiltPicture[ xC + x ][ yC + y ] with x, y = 0..(nS)-1, are derived as follows.

recFiltPicture[xC+x][yC+y]=i=0N−1s''[xC+x+horPos[i],yC+y+verPos[i]]∗cC[i]+alfOffset	(8-24)

where N = AlfNumCoeffLuma-1, ALF_NUM_BIT_SHIFT = 8, and horPos[i] and verPos[i] are specified in Error: Reference source not found and Error: Reference source not found, respectively.

##### Alf Offset Process

Inputs to this process are:

–	a variable c specifying color component index,

–	a sample position ( xC, yC ),

–	an array alfSaoOffsetVal specifying offset values,

–	a block size nS.

Output of this process is a modified picture buffer for the color component c.

Let recFiltPicture represents the processed sample array of the current picture of color component c and alfSaoType is set equal to alf_sao_submode[ c ][ ry ][ rx ].

Variable bitDepth is set equal to BitDepthY if cIdx is equal to 0, otherwise, set equal to BitDepthC.

Depending on the value of AlfSaoType, the following applies:

–	If alfSaoType is equal to one of the values of 1, 2, 3 or 4, the following ordered steps apply:

- Arrays hPos[2] and vPos[2] are specified in Table 8-1.

- A variable edgeIdx is specified as

edgeIdx = 2 + ∑k( Sign( recPicture[ xC + i, yC + j ] –

recPicture[ xC + i + hPos[ k ], yC + j + vPos[ k ] ] ) ) with k = 0..1	(8-25)

- The reconstructed picture buffer is modified as

recFiltPicture [ xC + i, yC + j ] = recPicture[ xC + i, yC + j ] + alfSaoOffsetVal [ edgeTable[ edgeIdx ] ]	(8-26)

with i = 0..nS-1 and j = 0..nS-1 where 	edgeTable[5] = { 1, 2, 0, 3, 4}.

–	Otherwise, if alfSaoType is equal to one of the values of 5 or 6, the following ordered steps applies:

- A variable bandShift is set equal to BitDepthY – 5 if cIdx is equal to 0, otherwise, set equal to BitDepthC – 5.

- The reconstructed picture buffer is modified as

recFiltPicture [ xC + i, yC + j ] = recPicture[ xC + i, yC + j ] +

alfSaoOffsetVal [ bandTable[ alfSaoType – 5 ][ bandIdx ] ]	(8-27)

with i = 0..nS-1 and j = 0..nS-1 where bandIdx is set equal to ( recPicture[ xC + i, yC + j ] >> bandShift ) and bandTable is specified in Table 8-2.

Table 8-1 – Specification of hPos[2] and vPos[2] according to the type of sample adaptive offset process

| sao_type_idx[ cIdx ][ saoDepth ][ rx ][ ry ] | 1 | 2 | 3 | 4 |
| --- | --- | --- | --- | --- |
| hPos[0] | -1 | 0 | -1 | 1 |
| hPos[1] | 1 | 0 | 1 | -1 |
| vPos[0] | 0 | -1 | -1 | 1 |
| vPos[1] | 0 | 1 | 1 | -1 |

Table 8-2 – Specification of array bandTable according to bandIdx

| bandIdx | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| bandTable[0][bandIdx] | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 |
| bandTable[1][bandIdx] | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| bandIdx | 16 | 17 | 18 | 19 | 20 | 21 | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 | 30 | 31 |
| bandTable[0][bandIdx] | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| bandTable[1][bandIdx] | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 |