| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 8th Meeting: San José, CA, USA, 1–10 February, 2012 | Document: JCTVC-H0275 |
| --- | --- |

| Title: | Non-CE10: Introduction of strong filter clipping in deblocking filter |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Masaru Ikeda Teruhiko Suzuki Sony City Osaki. 2-10-1 Osaki Shinagawa-ku, Tokyo, 141-8610 Japan | Tel: Email: | +81-50-3750-2743 Masaru.Ikeda@jp.sony.com teruhikos@jp.sony.com |
| Source: | Sony Corporation |  |  |

_____________________________

# Abstract

This contribution addresses strong filter operation in deblocking filter.

In this proposal, clipping process with using 2*tc is proposed to be introduced to strong filter operation. This proposed technique can prevent pixels from being filtered too much without someone’s intention and enable keeping the visual quality.

It is reported that the average results except class-F are provided as follows: AI_HE: 0.0%, RA_HE: 0.0%, RA_HE10: 0.0%, LB_HE: 0.0%, AI_LC: 0.0%, RA_LC: 0.0% and LB_LC: 0.0% and the average ones including class-F are provided as follows: AI_HE: 0.0%, RA_HE: -0.1%, RA_HE10: 0.0%, LB_HE: -0.1%, LP_HE: -0.1%, AI_LC: -0.1%, RA_LC: -0.1%, LB_LC: -0.2% and LP_LC: -0.1%.

# Introduction

In HM-5.0, simplified deblocking decision is introduced as described in JCTVC-G590 [1]. In this proposal, strong filter decision is done for segment of 4-lines instead of each line. Therefore it may occur that strong decision for the segment is different from one for each line, and then strong filter is applied to the line to be performed by weak filter. And the decision itself may make a mistake, and then strong filter is performed. In such cases, pixels are filtered too much without someone’s intent.

Pixel values changed by strong filter were checked in common test conditions. The maximum values changed by the filtering are shown in Table 1 and Table 2. Under AI_HE/QP=22, the maximum values are 16, 9, 29, 7, 15 and 190 for class-A, B, C, D, E and F respectively. Under AI_HE/QP=27, the values are 25, 37, 40, 13, 16 and 190 for class-A, B, C, D, E and F respectively. Under AI_HE/QP=32, the values are 23, 42, 54, 57, 20 and 190 for class-A, B, C, D, E and F respectively. Under AI_HE/QP=37, the values are 29, 50, 109, 54, 28 and 191 for class-A, B, C, D, E and F respectively. The maximum values are found to be large in especially class-F.

|  | AI_HE |  |  |  | RA_HE |  |  |  | LB_HE |  |  |  | LP_HE |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| QP | 22 | 27 | 32 | 37 | 22 | 27 | 32 | 37 | 22 | 27 | 32 | 37 | 22 | 27 | 32 | 37 |
| A.Traffic | 10 | 25 | 23 | 20 | 4 | 10 | 11 | 13 |  |  |  |  |  |  |  |  |
| A.PeopleOnStreet | 16 | 17 | 23 | 29 | 12 | 19 | 19 | 18 |  |  |  |  |  |  |  |  |
| A.NebutaFestival | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |  |  |  |  |  |  |  |  |
| A.SteamLocomotiveTrain | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |  |  |  |  |  |  |  |  |
| B.Kimono1 | 5 | 9 | 16 | 14 | 4 | 10 | 8 | 14 | 3 | 5 | 9 | 12 | 5 | 10 | 9 | 13 |
| B.ParkScene | 7 | 18 | 35 | 38 | 4 | 11 | 18 | 33 | 3 | 12 | 23 | 34 | 5 | 11 | 19 | 37 |
| B.Cactus | 9 | 21 | 25 | 50 | 7 | 9 | 20 | 37 | 6 | 8 | 15 | 37 | 7 | 8 | 18 | 33 |
| B.BasketballDrive | 8 | 23 | 40 | 47 | 6 | 17 | 18 | 67 | 9 | 15 | 33 | 37 | 9 | 13 | 31 | 33 |
| B.BQTerrace | 9 | 37 | 42 | 49 | 15 | 29 | 25 | 84 | 7 | 55 | 53 | 73 | 12 | 38 | 50 | 82 |
| C.BasketballDrill | 11 | 26 | 36 | 72 | 4 | 22 | 34 | 34 | 3 | 14 | 34 | 16 | 5 | 24 | 34 | 30 |
| C.BQMall | 11 | 20 | 54 | 109 | 6 | 13 | 22 | 29 | 10 | 15 | 31 | 37 | 5 | 10 | 32 | 49 |
| C.PartyScene | 10 | 40 | 53 | 69 | 4 | 28 | 25 | 34 | 3 | 7 | 20 | 44 | 5 | 10 | 31 | 30 |
| C.RaceHorses | 29 | 13 | 42 | 54 | 9 | 9 | 19 | 35 | 27 | 11 | 22 | 30 | 3 | 7 | 21 | 34 |
| D.BasketballPass | 5 | 13 | 55 | 54 | 6 | 9 | 11 | 33 | 3 | 6 | 11 | 20 | 4 | 6 | 16 | 18 |
| D.BQSquare | 7 | 12 | 57 | 47 | 4 | 6 | 15 | 36 | 3 | 8 | 20 | 22 | 2 | 6 | 18 | 20 |
| D.BlowingBubbles | 5 | 9 | 22 | 35 | 4 | 6 | 40 | 16 | 3 | 6 | 14 | 15 | 2 | 5 | 14 | 13 |
| D.RaceHorses | 3 | 8 | 31 | 26 | 3 | 7 | 8 | 42 | 2 | 6 | 19 | 19 | 2 | 8 | 14 | 18 |
| E.vidyo1 | 15 | 16 | 16 | 27 |  |  |  |  | 4 | 5 | 15 | 17 | 4 | 6 | 14 | 16 |
| E.vidyo3 | 10 | 11 | 20 | 25 |  |  |  |  | 4 | 7 | 9 | 14 | 4 | 6 | 11 | 14 |
| E.vidyo4 | 8 | 13 | 18 | 28 |  |  |  |  | 5 | 5 | 11 | 17 | 6 | 11 | 16 | 20 |
| F.BasketballDrillText | 15 | 26 | 37 | 72 | 13 | 24 | 37 | 33 | 13 | 26 | 23 | 48 | 8 | 14 | 28 | 25 |
| F.ChinaSpeed | 189 | 190 | 190 | 191 | 178 | 189 | 188 | 179 | 166 | 176 | 189 | 182 | 161 | 183 | 175 | 169 |
| F.SlideEditing | 136 | 134 | 134 | 154 | 136 | 126 | 128 | 131 | 136 | 127 | 117 | 158 | 136 | 120 | 117 | 158 |
| F.SlideShow | 190 | 188 | 134 | 127 | 129 | 129 | 191 | 190 | 191 | 188 | 181 | 131 | 191 | 188 | 181 | 130 |

Table 1: Maximum values of pixel changed by strong filter in HE conditions

|  | AI_LC |  |  |  | RA_LC |  |  |  | LB_LC |  |  |  | LP_LC |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| QP | 22 | 27 | 32 | 37 | 22 | 27 | 32 | 37 | 22 | 27 | 32 | 37 | 22 | 27 | 32 | 37 |
| A.Traffic | 9 | 13 | 20 | 26 | 5 | 11 | 10 | 15 |  |  |  |  |  |  |  |  |
| A.PeopleOnStreet | 16 | 19 | 23 | 29 | 16 | 19 | 21 | 31 |  |  |  |  |  |  |  |  |
| A.NebutaFestival | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |  |  |  |  |  |  |  |  |
| A.SteamLocomotiveTrain | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |  |  |  |  |  |  |  |  |
| B.Kimono1 | 5 | 11 | 16 | 26 | 5 | 13 | 11 | 24 | 4 | 6 | 9 | 15 | 3 | 5 | 10 | 31 |
| B.ParkScene | 6 | 18 | 35 | 54 | 5 | 10 | 21 | 30 | 3 | 6 | 21 | 33 | 4 | 10 | 19 | 37 |
| B.Cactus | 7 | 15 | 34 | 41 | 6 | 9 | 19 | 37 | 6 | 11 | 18 | 32 | 6 | 12 | 26 | 25 |
| B.BasketballDrive | 7 | 19 | 36 | 50 | 9 | 17 | 24 | 39 | 6 | 13 | 24 | 36 | 7 | 19 | 45 | 46 |
| B.BQTerrace | 7 | 28 | 44 | 58 | 7 | 14 | 36 | 42 | 7 | 50 | 63 | 47 | 8 | 13 | 67 | 47 |
| C.BasketballDrill | 8 | 26 | 35 | 26 | 4 | 29 | 34 | 34 | 5 | 22 | 20 | 15 | 4 | 21 | 19 | 29 |
| C.BQMall | 10 | 21 | 42 | 109 | 8 | 31 | 31 | 40 | 5 | 10 | 40 | 24 | 7 | 9 | 40 | 48 |
| C.PartyScene | 10 | 32 | 52 | 75 | 23 | 28 | 26 | 41 | 3 | 8 | 22 | 38 | 4 | 12 | 14 | 29 |
| C.RaceHorses | 6 | 15 | 42 | 53 | 5 | 9 | 15 | 28 | 3 | 7 | 15 | 37 | 4 | 9 | 27 | 41 |
| D.BasketballPass | 4 | 9 | 56 | 35 | 3 | 8 | 12 | 35 | 3 | 7 | 17 | 19 | 6 | 8 | 11 | 27 |
| D.BQSquare | 6 | 19 | 37 | 44 | 10 | 9 | 20 | 33 | 4 | 6 | 9 | 14 | 3 | 5 | 13 | 20 |
| D.BlowingBubbles | 4 | 34 | 21 | 49 | 3 | 5 | 11 | 34 | 2 | 5 | 8 | 14 | 3 | 7 | 13 | 28 |
| D.RaceHorses | 2 | 5 | 16 | 24 | 4 | 6 | 15 | 25 | 3 | 5 | 12 | 17 | 2 | 9 | 19 | 20 |
| E.vidyo1 | 14 | 16 | 16 | 32 |  |  |  |  | 3 | 6 | 15 | 22 | 4 | 7 | 15 | 14 |
| E.vidyo3 | 11 | 13 | 21 | 20 |  |  |  |  | 5 | 6 | 8 | 20 | 5 | 8 | 11 | 19 |
| E.vidyo4 | 9 | 13 | 29 | 28 |  |  |  |  | 5 | 10 | 12 | 15 | 4 | 8 | 15 | 18 |
| F.BasketballDrillText | 10 | 26 | 37 | 39 | 7 | 19 | 29 | 28 | 5 | 22 | 15 | 19 | 8 | 21 | 28 | 28 |
| F.ChinaSpeed | 189 | 189 | 184 | 190 | 188 | 188 | 181 | 171 | 164 | 171 | 175 | 165 | 182 | 183 | 166 | 164 |
| F.SlideEditing | 135 | 134 | 137 | 150 | 135 | 127 | 133 | 126 | 135 | 138 | 117 | 122 | 134 | 137 | 136 | 107 |
| F.SlideShow | 188 | 127 | 128 | 151 | 128 | 188 | 128 | 128 | 128 | 189 | 129 | 131 | 128 | 189 | 129 | 128 |

Table 2: Maximum values of pixel changed by strong filter in LC conditions

# Proposed modification

In order not to be filtered too much, the following clipping process is proposed to be introduced to strong filter operation. The clipping value is set to 2*tc, which is larger than weak filter’s, in order not to be filtered too weakly.

p0 i’ = p0i + Clip(-2*tc)-(2*tc) ( ( p2i + 2*p1i - 6*p0i + 2*q0i + q1i + 4 ) >> 3 )		(1)

p1 i’ = p1i + Clip(-2*tc)-(2*tc) ( ( p2i – 3*p1i + p0i + q0i + 2 ) >> 2 )			(2)

p2 i’ = p2i + Clip(-2*tc)-(2*tc) ( ( 2*p3i - 5*p2i + p1i + p0i + q0i + 4 ) >> 3 )		(3)

q0 i’ = q0i + Clip(-2*tc)-(2*tc) ( ( p1i + 2*p0i - 6*q0i + 2*q1i + q2i + 4 ) >> 3 )		(4)

q1 i’ = q1i + Clip(-2*tc)-(2*tc) ( ( p0i + q0i – 3*q1i + q2i + 2 ) >> 2 )			(5)

q2 i’ = q2i + Clip(-2*tc)-(2*tc) ( ( p0i + q0i + q1i - 5*q2i + 2*q3i + 4 ) >> 3 )		(6)

# Experimental results

The experiment is done for mandatory conditions based on JCTVC-G1210 [2] with using HM-5.0. The detail of test sequences, configurations and cases is described in JCTVC-G1200 [3]. Both encoding and decoding time are measured with using the same machine as anchor and the decoding one is done without YUV output file.

|  | All Intra HE |  |  | All Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class C | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class D | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class E | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class F |  |  |  |  |  |  |
| Overall | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 100% |  |  | 98% |  |  |
| Dec Time[%] | 100% |  |  | 99% |  |  |

Table 2-1: Results for Intra

|  | Random Access HE |  |  | Random Access LC |  |  | Random Access HE-10 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | 0.0% | 0.0% | -0.1% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class B | 0.0% | -0.1% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class C | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% |  |  |  |
| Class D | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| Class E |  |  |  |  |  |  |  |  |  |
| Class F |  |  |  |  |  |  |  |  |  |
| Overall | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 100% |  |  | 100% |  |  | 100% |  |  |
| Dec Time[%] | 99% |  |  | 99% |  |  | 100% |  |  |

Table 2-2: Results for RandomAccess

|  | Low delay B HE |  |  | Low delay B LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.0% | 0.1% | 0.5% | 0.0% | -0.2% | -0.3% |
| Class C | 0.0% | -0.2% | -0.1% | 0.0% | 0.1% | -0.1% |
| Class D | 0.0% | -0.2% | 0.4% | -0.1% | 0.2% | -0.1% |
| Class E | 0.1% | 0.1% | -0.2% | 0.0% | -0.5% | 0.0% |
| Class F |  |  |  |  |  |  |
| Overall | 0.0% | -0.1% | 0.2% | 0.0% | -0.1% | -0.2% |
|  | 0.0% | -0.1% | 0.2% | 0.0% | -0.1% | -0.1% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 99% |  |  |

Table 2-3: Results for LowDelayB

|  | All Intra HE |  |  | All Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class C | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class D | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class E | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class F | -0.6% | 0.0% | 0.0% | -0.5% | 0.0% | 0.0% |
| Overall | -0.1% | 0.0% | 0.0% | -0.1% | 0.0% | 0.0% |
|  | -0.1% | 0.0% | 0.0% | -0.1% | 0.0% | 0.0% |
| Enc Time[%] | 100% |  |  | 98% |  |  |
| Dec Time[%] | 100% |  |  | 99% |  |  |

Table 2-4: Results for Intra (Full)

|  | Random Access HE |  |  | Random Access LC |  |  | Random Access HE-10 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | 0.0% | 0.0% | -0.1% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class B | 0.0% | -0.1% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class C | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% |  |  |  |
| Class D | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| Class E |  |  |  |  |  |  |  |  |  |
| Class F | -0.4% | -0.1% | -0.1% | -0.4% | -0.1% | -0.1% |  |  |  |
| Overall | -0.1% | 0.0% | 0.0% | -0.1% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | -0.1% | -0.1% | 0.0% | -0.1% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 100% |  |  | 100% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 99% |  |  | 100% |  |  |

Table 2-5: Results for RandomAccess (Full)

|  | Low delay B HE |  |  | Low delay B LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.0% | 0.1% | 0.5% | 0.0% | -0.2% | -0.3% |
| Class C | 0.0% | -0.2% | -0.1% | 0.0% | 0.1% | -0.1% |
| Class D | 0.0% | -0.2% | 0.4% | -0.1% | 0.2% | -0.1% |
| Class E | 0.1% | 0.1% | -0.2% | 0.0% | -0.5% | 0.0% |
| Class F | -0.4% | 0.1% | -0.5% | -0.7% | -0.5% | -0.7% |
| Overall | -0.1% | 0.0% | 0.0% | -0.2% | -0.2% | -0.3% |
|  | -0.1% | 0.0% | 0.0% | -0.2% | -0.2% | -0.2% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |

Table 2-6: Results for LowDelayB (Full)

|  | Low delay P HE |  |  | Low delay P LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.0% | 0.0% | -0.1% | 0.0% | 0.0% | -0.1% |
| Class C | 0.1% | 0.1% | 0.0% | 0.0% | 0.0% | 0.1% |
| Class D | 0.0% | 0.2% | 0.3% | -0.1% | 0.1% | 0.1% |
| Class E | -0.1% | 0.1% | 0.2% | -0.1% | -0.6% | -0.5% |
| Class F | -0.4% | 0.1% | 0.0% | -0.5% | -0.3% | -0.5% |
| Overall | -0.1% | 0.1% | 0.0% | -0.1% | -0.1% | -0.1% |
|  | -0.1% | 0.1% | 0.0% | -0.1% | -0.1% | -0.1% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 99% |  |  |

Table 2-7: Results for LowDelayP (Full)

The average experimental results (except class-F) are provided as follows: AI_HE: 0.0%, RA_HE: 0.0%, RA_HE10: 0.0%, LB_HE: 0.0%, AI_LC: 0.0%, RA_LC: 0.0% and LB_LC: 0.0%.

The average experimental results (including class-F) are provided as follows: AI_HE: 0.0%, RA_HE: -0.1%, RA_HE10: 0.0%, LB_HE: -0.1%, LP_HE: -0.1%, AI_LC: -0.1%, RA_LC: -0.1%, LB_LC: -0.2% and LP_LC: -0.1%.

BD-bitrates for class-F, in which maximum values of pixel changed by strong filter are large, are saved and ones for other classes are kept.

# WD text

In this proposal, the following modifications with yellow highlight are presented.

###### Filtering process for a luma sample

Inputs of this process are:

–	sample values, pi and qi with i = 0..3,

–	variables dEp1 and dEq1 containing decisions to filter pixels p1 and q1 respectively,

–	a variable tC.

Output of this process is:

–	number of filtered samples nDp and nDq,

–	filtered sample values, pi’ and qj’ with i = 0..nDp – 1, j = 0..nDq – 1

Depending on dE, the following applies:

–	When the variable dE is equal to 2, the following strong filtering applies while nDp and nDq are set equal to 3:

p0’ = p0 + Clip3( -2*tc, 2*tc, ( p2 + 2*p1 - 6*p0 + 2*q0 + q1 + 4 ) >> 3 )		(8-1)

p1’ = p1 + Clip3( -2*tc, 2*tc, ( p2 – 3*p1 + p0 + q0 + 2 ) >> 2 )		(8-2)

p2’ = p2 + Clip3( -2*tc, 2*tc, ( 2*p3 - 5*p2 + p1 + p0 + q0 + 4 ) >> 3 )		(8-3)

q0’ = q0 + Clip3( -2*tc, 2*tc, ( p1 + 2*p0 - 6*q0 + 2*q1 + q2 + 4 ) >> 3 )		(8-4)

q1’ = q1 + Clip3( -2*tc, 2*tc, ( p0 + q0 – 3*q1 + q2 + 2 ) >> 2 )		(8-5)

q2’ = q2 + Clip3( -2*tc, 2*tc, ( p0 + q0 + q1 - 5*q2 + 2*q3 + 4 ) >> 3 )		(8-6)

–	Otherwise, the following weak filtering applies while nDp and nDq are set equal to 0:

 = ( 9 * ( q0 –  p0 ) – 3 * ( q1 – p1 ) + 8 ) >> 4		(8-7)

  - When abs() is less than tc*10, the following ordered steps apply:

    - The filtered sample values p0’ and q0’ are specified as follows:

 = Clip3( -tc, tc,  )		(8-8)

p0’ = Clip1Y( p0 +  )		(8-9)

q0’ = Clip1Y( q0 -  )		(8-10)

    - If dEp1 is equal to 1, the filtered sample value pi’ is specified as follows:

p = Clip3( -(tc >> 1), tc >> 1, ( ( ( p2 + p0 + 1 ) >> 1 ) – p1 +  ) >>1 )	(8-11)

pi’ = Clip1Y( p1 + p )		(8-12)

    - If dEq1 is equal to 1, the filtered sample value qi’ is specified as follows:

q = Clip3( -(tc >> 1), tc >> 1, ( ( ( q2 + q0 + 1 ) >> 1 ) – q1 –  ) >>1 )	(8-13)

qi’ = Clip1Y( q1 + q )		(8-14)

nDp is set equal to dEp1+1 and nDq is set equal to dEq1+1.

# Conclusions

This contribution presents the introduction of strong filter clipping in deblocking filter. The clipping can save too much filtering without someone’s intent and keep the visual quality. It is suggested that the clipping values are set to 2*tc to keep filter strength compared with weak filter because the weak one has clipping with using tc. It is reported that BD-bitrates including class-F are AI_HE: 0.0% , RA_HE: -0.1%, RA_HE10: 0.0%, LB_HE: -0.1%, LP_HE: -0.1%, AI_LC: -0.1%, RA_LC: -0.1%, LB_LC: -0.2% and LP_LC: -0.1%. It is proposed to adopt the introduction of strong filter clipping into the next version of HM.

# References

[1] JCT-VC, “CE12 Subtest1: Results for modified decisions for deblocking”, JCTVC-G590, 7th. JCT-VC Meeting, Geneva, CH, November, 2011

[2] JCT-VC, “Description of Core Experiment 10 (CE10): Deblocking Filtering”, JCTVC-G1210, 7th. JCT-VC Meeting, Geneva, CH, November, 2011

[3] JCT-VC, “Common HM test conditions and software reference configurations”, JCTVC-G1200, 7th. JCT-VC Meeting, Geneva, CH, November, 2011

# Patent rights declaration(s)

Sony Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).