| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 8th Meeting: San José, CA, USA, 1–10 February, 2012 | Document: JCTVC-H0276 |
| --- | --- |

| Title: | CE8.e.1: Offset coding in SAO |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Chih-Ming Fu, Yu-Wen Huang, Shawmin Lei MediaTek No. 1, Dusing Rd. 1, Hsinchu Science Park, Hsinchu, Taiwan 30078 In Suk Chong, Marta Karczewicz Qualcomm 5775 Morehouse Dr, San Diego, CA 92121, USA | Tel: Email: | Shawmin Lei +886-3-5670766 ext. 25555 {chihming.fu, yuwen.huang, shawmin.lei}@mediatek.com Marta Karczewicz +1-858-658-5673 {ichong, martak}@qualcomm.com |
| Source: | MediaTek Inc., Qualcomm Inc. |  |  |

_____________________________

# Abstract

In this contribution, a new codeword design can be used to better fit the offset distribution than that in HM-5.0, and an offset prediction technique can be used to reduce offset information. Simulation results reportedly show that the bit rate is reduced by 0.1% with unchanged run time. Moreover, the salt and pepper noises reported in JCTVC-G680 are reduced. In order to totally avoid potential salt and pepper noises, a non-normative method limiting edge offset ranges can be combined without less than 0.1% coding efficiency change, as a non-CE part of this contribution.

# Introduction

In High Efficiency Video Coding test model version 5.0 (HM-5.0), sample adaptive offset (SAO) is applied to luma and chroma, and each component is independently processed. SAO can divide one picture into multiple LCU-aligned regions, and each region can select one SAO type among two band offset (BO) types, four edge offset (EO) types, and no processing (OFF). For each to-be-processed pixel, BO uses the pixel intensity to classify the pixel into a band. The pixel intensity range is equally divided into 32 bands, as shown in Figure 1. After pixel classification, one offset is derived for all pixels of each band, and the offsets of center 16 bands or outer 16 bands are selected and coded. As for EO, it uses two neighboring pixels of a to-be-processed pixel to classify the pixel into a category. Similarly, one offset is derived for all pixels of each category after pixel classification except for category 0, which is forced to use zero offset. Table 1 shows the EO pixel classification, where “c” denotes the pixel to be classified. Figure 2 shows four EO types with different selections of neighboring pixels.

Figure 1.  Pixel intensity range equally divided into 32 bands by BO

Table 1.  EO pixel classification

Figure 2.  Four EO types

In HM-5.0, SAO information is coded in adaptation parameter set (APS), and offsets are represented by signed exp-Golomb codes, where a zero offset has the shortest codeword and the codeword length gets longer when the offset magnitude increases. However, it was pointed out in JCTVC-G222 [1] that the current codewords may not fit the offset distribution well. In this contribution, the new offset coding method in JCTVC-G222 is studied on HM-5.0. The rest of this contribution is organized as follows. The proposed offset coding in SAO will be described in Section 2, simulation results will be shown in Section 3, and conclusions will be given in Section 4.

# Algorithm description

## Offset codeword design

The offset coding in BO is not changed in our design. The offset coding in EO is as follows:

If ( ( offset – b ) < 0 ), index = – ( ( offset – b ) << 1 ) – a

Else, index = ( ( offset – b ) << 1 ) - 1 + a

The index is used as the input of exp-Golomb code table. The parameters, a and b, are shown in Table 2 for different EO categories, where b denotes the offset value of the highest probability and a controls the priorities of two offset values of the same magnitude.

Table 2.  Parameters of the codeword design

|  | 10-bit pixels | 8-bit pixels |
| --- | --- | --- |
| EO category 1 | b = 2, a = 1 | b = 1, a = 1 |
| EO category 2 | b = 1, a = 1 | b = 1, a = 0 |
| EO category 3 | b = -1, a = 0 | b = -1, a = 1 |
| EO category 4 | b = -2, a = 0 | b = -1, a = 0 |

## Offset prediction

To predict the offset value, we use the neighboring region information. If the current region and the left region in the same depth select EO, category 1 and category 2 of the current region will be predicted by corresponding offsets of the left region, and in the current region the category 3 and category 4 will be predicted by category 2 and category 1 offset magnitudes of opposite signs. If the current region applies BO, the offset of the second band will be predicted by that of the first band, and the offset of the 16th band will be predicted by that of the 15th band.

## Offset range constraints

Salt and pepper noises can be sometimes observed in flat regions when pictures are enlarged, as reported in JCTVC-G680 [2]. In fact, the salt and pepper noises can be significantly reduced by the proposed offset codeword design. In order to totally avoid potential salt and pepper noises, a very simple method to constrain offset ranges is proposed on the encoder side, which is non-normative. For 8-bit videos, category 1 and category 2 cannot use offsets smaller than -1 and 0, respectively, and category 3 and category 4 cannot use offsets larger than 0 and 1, respectively. For 10-bit videos, category 1 and category 2 cannot use offsets smaller than -5 and -1, respectively, and category 3 and category 4 cannot use offsets larger than 1 and 5, respectively.

# Simulation results

Table 1 shows results of the proposed offset coding in SAO. The JCTVC-G1200 anchor [3] is used for comparison. The luma BD-rates excluding class F are 0.0%, -0.1%, -0.1%, -0.1%, -0.7%, -1.2%, -2.3%, and -8.0% for HE-AI, HE-RA, HE-LDB, HE-LDP, LC-AI, LC-RA, LC-LDB, and LC-LDP, respectively. After careful viewing, it is reported that most salt and pepper noises are removed.

Table 2 shows results of the proposed offset coding in combination with the offset range constraints. The luma BD-rates excluding class F are 0.0%, -0.1%, 0.0%, 0.0%, -0.7%, -1.2%, -2.2%, and -8.0% for HE-AI, HE-RA, HE-LDB, HE-LDP, LC-AI, LC-RA, LC-LDB, and LC-LDP, respectively. Although the coding efficiency in terms of BD-rates is slightly decreased after applying the offset range constraints, it is reported that no salt and pepper noise is observed.

# Conclusions

MediaTek and Qualcomm proposed new offset coding in SAO. It is reported that the luma bit rate is reduced by 0.1% with reduced pepper and salt noise and the same run time on average.

# Patent rights declarations

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Qualcomm Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

- C.-M. Fu, Y.-W. Huang, S. Lei, I. S. Chong, M. Karczewicz, “Non-CE8: Offset coding in SAO,” Document of Joint Collaborative Team on Video Coding, JCTVC-G222, November 2011.

- W.-S. Kim, D.-K. Kwon, “Non-CE8: Method of visual coding artifact removal for SAO,” Document of Joint Collaborative Team on Video Coding, JCTVC-G680, November 2011.

- F. Bossen, “Common HM test conditions and software reference configurations,” Document of Joint Collaborative Team on Video Coding, JCTVC-G1200, November 2011.

Table 1.  Results of the proposed offset coding

Table 2.  Results of the proposed offset coding and the offset range constraints