| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 8th Meeting: San José, CA, USA, 1–10 February, 2012 | Document: JCTVC-H0277 |
| --- | --- |

| Title: | Non-CE8: Additional simulation results of SAO and ALF |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Tzu-Der Chuang, Yi-Wen Chen, Jian-Liang Lin, Yu-Wen Huang, Shawmin Lei No. 1, Dusing Rd. 1, Hsinchu Science Park, Hsinchu, Taiwan 30078 | Tel: Email: | Shawmin Lei +886-3-5670766 ext. 25555 {peter.chuang, yiwen.chen, jl.lin, yuwen.huang, shawmin.lei}@mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

SAO and ALF are completely new coding tools in HEVC. This contribution shows results of “SAO off & ALF off,” “SAO off & ALF on,” “SAO on & ALF off,” and “SAO on & ALF on” for not only common test condition (CTC) sequences but also additional sequences that were commonly seen during key technical area (KTA) study period. The “KTA” sequences include fifteen 720p and ten 1080p video clips. Excluding class F of CTC sequences and the two AI conditions (i.e. HE-RA, HE-LDB, HE-LDP, LC-RA, LC-LDB, and LC-LDP included), the results can be briefly summarized as follows. SAO provides 1-8% and 2-13% coding gains for CTC and KTA sequences, respectively, and occupies 0-3% encoding time and 1-5% decoding time. ALF provides 3-9% and 4-16% coding gains for CTC and KTA sequences, respectively, and occupies 4-10% encoding time and 14-23% decoding time. Both SAO and ALF are more beneficial for HD sequences than for low resolution sequences. The subjective quality improvement of SAO has been verified in CE3. Many examples of visual quality improvement of “SAO on & ALF on” over “SAO on & ALF off” are shown in this contribution. Based on the results, the proponents advocate adopting “SAO on & ALF on” in HE conditions and a profile that pursuits high coding efficiency tools (e.g. main/high) and “SAO on & ALF off” in LC conditions and a profile that prefers low complexity tools (e.g. baseline).

# Test conditions

We use High-Efficiency Video Coding test model version 5.0 (HM-5.0) to conduct experiments for sample adaptive offset (SAO) and adaptive loop filter (ALF). The anchors follow the settings in the common test conditions (CTC) defined in JCTVC-G1200 [1]. Six conditions are used: high efficiency random access (HE-RA), high efficiency low delay B (HE-LDB), high efficiency low delay P (HE-LDP), low complexity random access (LC-RA), low complexity low delay B (LC-LDB), and low complexity low delay P (LC-LDP). Besides CTC sequences, all 720p and 1080p sequences we obtained during the key technical area (KTA) study period are also tested, which can represent more practical cases that High-Efficiency Video Coding (HEVC) will be applied. In the rest of this contribution, we call these additional 720p and 1080p sequences as “KTA” sequences. In today’s market of portable devices, 720p has become the mainstream picture size, and 1080p will become popular within a few years. In the TV market, 1080p has been the major format for several years. In CTC sequences, sizes of 416x240, 832x480, and 1024x768 are still used, which seem to be a bit too small for HEVC applications. Table 1 shows the KTA sequences including fifteen 720p and ten 1080p video clips.

Table 1.  List of KTA sequences (name_widthxheight_frames.yuv)

| 720p | BigShips_1280x720_600.yuv |
| --- | --- |
|  | City_1280x720_900.yuv |
|  | Crew_1280x720_600.yuv |
|  | Cyclists_1280x720_600.yuv |
|  | Harbour_1280x720_600.yuv |
|  | Jets_1280x720_550.yuv |
|  | Night_1280x720_460.yuv |
|  | Optis_1280x720_600.yuv |
|  | Panslow_1280x720_550.yuv |
|  | Preakness_1280x720_600.yuv |
|  | Raven_1280x720_600.yuv |
|  | Sailormen_1280x720_600.yuv |
|  | Sheriff_1280x720_600.yuv |
|  | ShuttleStart_1280x720_600.yuv |
|  | Spincalendar_1280x720_550.yuv |
| 1080p | blue_sky_1920x1080_106.yuv |
|  | pedestrian_area_1920x1080_375.yuv |
|  | riverbed_1920x1080_250.yuv |
|  | rush_hour_1920x1080_500.yuv |
|  | station2_1920x1080_313.yuv |
|  | sunflower_1920x1080_500.yuv |
|  | toys_and_calendar_1920x1080_250.yuv |
|  | tractor_1920x1080_690.yuv |
|  | vintage_car_1920x1080_300.yuv |
|  | walking_couple_1920x1080_250.yuv |

# Simulation results

Table 2 and Table 3 show the results of “SAO off & ALF off” for CTC and KTA sequences, respectively. Table 4 and Table 5 show the results of “SAO off & ALF on” for CTC and KTA sequences, respectively. Table 6 and Table 7 show the results of “SAO on & ALF off” for CTC and KTA sequences, respectively. Table 8 and Table 9 show the results of “SAO on & ALF on” for CTC and KTA sequences, respectively. All data can be found in the attached Excel files. Please note that sometimes run time data may not be accurate in some particular conditions due to unexpected stability problems, but an average run time of many conditions after excluding outliers still can roughly represent the complexity of SAO or ALF.

Table 2.  Results of “SAO off & ALF off” for CTC sequences

Table 3.  Results of “SAO off & ALF off” for KTA sequences

Table 4.  Results of “SAO off & ALF on” for CTC sequences

Table 5.  Results of “SAO off & ALF on” for KTA sequences

Table 6.  Results of “SAO on & ALF off” for CTC sequences

Table 7.  Results of “SAO on & ALF off” for KTA sequences

Table 8.  Results of “SAO on & ALF on” for CTC sequences

Table 9.  Results of “SAO on & ALF on” for KTA sequences

# Objective performance summary

Table 10 summaries the objective performance of SAO from HE conditions in Table 4 and Table 5 (“SAO off & ALF on” versus “SAO on & ALF on”) and LC conditions in Table 6 and Table 7 (“SAO on & ALF off” versus “SAO off & ALF off”). Roughly speaking, SAO provides 1-8% and 2-13% coding gains for CTC and KTA sequences, respectively, and occupies 0-3% encoding time and 1-5% decoding time.

Table 11 summaries the objective performance of ALF from HE conditions in Table 6 and Table 7 (“SAO on & ALF off” versus “SAO on & ALF on”) and LC conditions in Table 4 and Table 5 (“SAO off & ALF on” versus “SAO off & ALF off”). Roughly speaking, ALF provides 3-9% and 4-16% coding gains for CTC and KTA sequences, respectively, and occupies 4-10% encoding time and 14-23% decoding time.

Table 12 summaries the objective performance of SAO and ALF from HE conditions in Table 2 and Table 3 (“SAO off & ALF off” versus “SAO on & ALF on”) and LC conditions in Table 8 and Table 9 (“SAO on & ALF on” versus “SAO off & ALF off”). Roughly speaking, SAO and ALF provide 5-15% and 7-28% coding gains for CTC and KTA sequences, respectively, and occupy 3-11% encoding time and 16-30% decoding time.

Table 10.  Summary of objective performance of SAO

| Condition | Luma Bitrate Diff. | Enc. Time Diff. | Dec. Time Diff. |
| --- | --- | --- | --- |
| HE-RA, CTC, Table 4 | +1.3% | -0% | -3% |
| HE-RA, KTA, Table 5 | +1.9% | -0% | -3% |
| LC-RA, CTC, Table 6 | -1.2% | +0% | +3% |
| LC-RA, KTA, Table 7 | -2.1% | -3% (outlier) | +1% |
| HE-LDB, CTC, Table 4 | +2.1% | -0% | -3% |
| HE- LDB, KTA, Table 5 | +2.6% | -0% | -2% |
| LC- LDB, CTC, Table 6 | -2.2% | +1% | +4% |
| LC- LDB, KTA, Table 7 | -3.3% | -2% (outlier) | +2% |
| HE-LDP, CTC, Table 4 | +2.6% | -0% | -3% |
| HE- LDP, KTA, Table 5 | +3.4% | +3% (outlier) | -2% |
| LC- LDP, CTC, Table 6 | -7.9% | +1% | +5% |
| LC- LDP, KTA, Table 7 | -12.9% | -2% (outlier) | +3% |

Table 11.  Summary of objective performance of ALF

| Condition | Luma Bitrate Diff. | Enc. Time Diff. | Dec. Time Diff. |
| --- | --- | --- | --- |
| HE-RA, CTC, Table 6 | +3.3% | -6% | -15% |
| HE-RA, KTA, Table 7 | +4.7% | -7% | -16% |
| LC-RA, CTC, Table 4 | -3.9% | +9% | +20% |
| LC-RA, KTA, Table 5 | -5.4% | +6% | +18% |
| HE-LDB, CTC, Table 6 | +2.8% | -4% | -14% |
| HE- LDB, KTA, Table 7 | +4.3% | -4% | -16% |
| LC- LDB, CTC, Table 4 | -3.5% | +6% | +17% |
| LC- LDB, KTA, Table 5 | -5.2% | +3% | +18% |
| HE-LDP, CTC, Table 6 | +4.2% | -8% | -14% |
| HE- LDP, KTA, Table 7 | +8.7% | -7% | -19% |
| LC- LDP, CTC, Table 4 | -9.3% | +10% | +18% |
| LC- LDP, KTA, Table 5 | -16.4% | +8% | +23% |

Table 12.  Summary of objective performance of SAO and ALF

| Condition | Luma Bitrate Diff. | Enc. Time Diff. | Dec. Time Diff. |
| --- | --- | --- | --- |
| HE-RA, CTC, Table 2 | +5.1% | -7% | -19% |
| HE-RA, KTA, Table 3 | +7.6% | -7% | -21% |
| LC-RA, CTC, Table 8 | -5.0% | +9% | +22% |
| LC-RA, KTA, Table 9 | -6.9% | +6% | +22% |
| HE-LDB, CTC, Table 2 | +5.3% | -4% | -16% |
| HE- LDB, KTA, Table 3 | +8.2% | -3% | -18% |
| LC- LDB, CTC, Table 8 | -5.4% | +6% | +21% |
| LC- LDB, KTA, Table 9 | -7.5% | +3% | +22% |
| HE-LDP, CTC, Table 2 | +15.3% | -7% | -17% |
| HE- LDP, KTA, Table 3 | +28.6% | -6% | -22% |
| LC- LDP, CTC, Table 8 | -11.5% | +9% | +20% |
| LC- LDP, KTA, Table 9 | -19.0% | +11% | +30% |

# Subjective quality evaluation

Figures will be provided in the next release.

# Conclusions

Based on the simulation results of SAO and ALF on HM-5.0, the proponents advocate adopting “SAO on & ALF on” in HE conditions and a profile that pursuits high coding efficiency tools (e.g. main/high) and “SAO on & ALF off” in LC conditions and a profile that prefers low complexity tools (e.g. baseline).

# Patent rights declarations

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

- F. Bossen, “Common HM test conditions and software reference configurations,” Document of Joint Collaborative Team on Video Coding, JCTVC-G1200, November 2011.