| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 8th Meeting: San José, CA, USA, 1–10 February, 2012 | Document: JCTVC-H0278 |
| --- | --- |

| Title: | Non-CE9: CU-based parallel merge mode |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jian-Liang Lin, Yi-Wen Chen, Yu-Wen Huang, Shawmin Lei No. 1, Dusing Rd. 1, Hsinchu Science Park, Hsinchu, Taiwan 30078 | Tel: Email: | Shawmin Lei +886-3-5670766 ext. 25555 {jl.lin, yiwen.chen, yuwen.huang, shawmin.lei}@mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

The derivation of reference indices for temporal merging candidates in HM-5.0 causes a motion data dependency issue in PU merge. The merging candidate list construction process of the second PU needs to wait until the motion data of the first PU is available. In this contribution, the derivation of reference index is modified to remove the motion data dependency and to allow parallel encoding and decoding of the first and second PUs in PU merge. Simulation results reportedly show that the CU-based parallel merge can be achieved without coding efficiency loss.

# Introduction

In High-Efficiency Video Coding test model version 5.0 (HM-5.0), the reference index of the left position, A1, as shown in Figure 1, is used as the reference index of the temporal merging candidates in merge and skip modes if the left position is available and non-intra. Otherwise, the reference index of the temporal merging candidate is set as 0.

Use left reference index as default. Set as 0 only if left is not available

- If refIdxLXA is not equal to -1, the following applies.

  - refIdxLX = refIdxLXA

- Otherwise, the following applies.

  - refIdxLX = 0

Figure 1.  The derivation of the reference index for a temporal merging candidate

Figure 2 shows the positions used to derive the reference indices for temporal merging candidates in PU merge. According to this design, the merging candidate list construction process of the first and second Nx2N PU cannot be implemented in parallel because the reference index of A1 position cannot be accessed before the motion data of the first PU is available. This issue causes motion data dependency in Nx2N PU merge as shown in Figure 3.

Figure 2.  Illustration of reference indices for temporal merging candidates in PU merge

Figure 3.  Motion data dependency in Nx2N PU merge

To remove the motion data dependency in PU merge, we simply modify the derivation of reference indices for temporal merging candidates to allow parallel encoding/decoding of the first and second PUs in PU merge, as illustrated in Figure 4.

Figure 4.  CU-based parallel merge

# Proposed modifications

The proposed method is very simple. For 2NxN, Nx2N, 2NxnU, 2NxnD, nLx2N, and nRx2N, we always set the reference index of the 2nd PU as zero. In this way, the 2nd PU is not dependent on the 1st PU in skip and merge modes.

# Simulation results

In the following simulations, the common test conditions specified in JCTVC-G1200 [1] are used. Table 1 summarizes the results of the proposed method and shows no coding efficiency loss.

# Conclusions

MediaTek proposed to use reference indices equal to zero for temporal merging candidates in the 2nd PUs of 2NxN, Nx2N, 2NxnU, 2NxnD, nLx2N, and nRx2N modes. It is reported that there is no coding efficiency loss, while the 2nd PU merge can be processed in parallel with the 1st PU.

# Patent rights declarations

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

- F. Bossen, “Common HM test conditions and software reference configurations,” Document of Joint Collaborative Team on Video Coding, JCTVC-G1200, November 2011.

Table 1.  Results of the proposed method