| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 8th Meeting: San José, CA, USA, 1–10 February, 2012 | Document: JCTVC-H0279 |
| --- | --- |

| Title: | Non-CE9: CU-based parallel inter mode |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jian-Liang Lin, Yi-Wen Chen, Tzu-Der Chuang, Yu-Wen Huang, Shawmin Lei No. 1, Dusing Rd. 1, Hsinchu Science Park, Hsinchu, Taiwan 30078 | Tel: Email: | Shawmin Lei +886-3-5670766 ext. 25555 {jl.lin, yiwen.chen, peter.chuang, yuwen.huang, shawmin.lei}@mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

In HM-5.0 merge mode, the spatial motion vector neighbours located in the 1st PU are excluded from the merging candidate list for the 2nd PU. However, those spatial motion vector neighbours are still used in inter mode, which prevents the 2nd PU from being processed in parallel with the 1st PU. In this contribution, the spatial motion vector neighbours located in the 1st PU are excluded to remove data dependency between the 1st PU and the 2nd PU. Simulation results reportedly show 0.0-0.2% bit rate increase while the inter mode allows parallel processing of the 1st and 2nd PUs. Moreover, the motion vector neighbours used in merge and inter modes are unified.

# Introduction

In merge mode, the left position of the 2nd Nx2N/nLx2N/nRx2N PU and the above position of the 2nd 2NxN/2NxnU/2NxnD PU are removed from the merging candidate list. However, those positions are still used to derive motion vector predictor candidates for inter mode. With this design, the motion vector predictor list construction process of the 1st and 2nd PUs cannot be processed in parallel because the 2nd PU needs to wait until the motion data of the 1st PU is available.

Figure 1 illustrates the spatial motion vector neighbours in 2NxN and Nx2N partitions. In this example, the spatial motion vector neighbour A1 for the 2nd Nx2N PU and B1 for the 2nd 2NxN PU cannot be accessed before the motion data of the 1st PU is available. This causes motion data dependency in inter mode between two PUs and prevents parallel processing for the two PUs. In this contribution, we propose a very simple solution to this problem. The rest of this contribution is organized as follows. The proposed method will be described in Section 2, simulation results will be shown in Section 3, and conclusions will be given in Section 4.

Figure 1.  Illustration of spatial motion vector neighbours in 2NxN and Nx2N partitions

# Proposed method

For the 2nd PU, we simply remove the spatial motion vector neighbour located in the 1st PU. In this way, the two PUs can be processed in parallel. Moreover, the spatial motion vector neighbours of the merging candidate list for merge mode and those of the motion vector predictor list for inter mode are unified.

# Simulation results

The common test conditions specified in JCTVC-G1200 [1] are used. Table 1 summarizes the results of the proposed method and shows 0.2%, 0.2%, 0.1%, 0.0%, 0.1%, 0.0% bit rate increases in HE-RA, LC-RA, HE-LDB, LC-LDB, HE-LDP, and LC-LDP, respectively.

# Conclusions

MediaTek propose to remove the spatial motion vector neighbours located in the 1st PU from the spatial motion vector predictor candidates for the 2nd PU. This not only allows parallel processing of the 1st and 2nd PUs but also unifies the spatial motion vector neighbours of merge mode and and those of inter mode. Simulation results reportedly show 0.0-0.2% bit rate increases with unchanged run time in comparison with HM-5.0.

# Patent rights declarations

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

- F. Bossen, “Common HM test conditions and software reference configurations,” Document of Joint Collaborative Team on Video Coding, JCTVC-G1200, November 2011.

Table 1.  Results of the proposed method