| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 8th Meeting: San José, CA, USA, 1–10 February, 2012 | Document: JCTVC-H0280 |
| --- | --- |

| Title: | CE10.2.1: Reducing one pixel line buffer by modified deblocking filter for horizontal LCU boundaries |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Chih-Wei Hsu, Ching-Yeh Chen, Yu-Wen Huang, Shawmin Lei MediaTek No. 1, Dusing Rd. 1, Hsinchu Science Park, Hsinchu, Taiwan 30078 Masaru Ikeda, Teruhiko Suzuki Sony Sony City Osaki. 2-10-1 Osaki Shinagawa-ku, Tokyo, 141-8610, Japan | Tel: Email: | Shawmin Lei +886-3-5670766 ext. 25555 {cw.hsu, chingyeh.chen, yuwen.huang, shawmin.lei}@mediatek.com Teruhiko Suzuki +81-50-3750-2740 {Masaru.Ikeda, teruhikos}@jp.sony.com |
| Source: | MediaTek Inc., Sony Corporation |  |  |

_____________________________

# Abstract

This contribution modifies the deblocking in HM-5.0. The luma vertical filtering for the upper side of the horizontal LCU boundary reads three pixels and writes two pixels, and the rest parts of deblocking are not changed. The proposed method can save one pixel line buffer. It is reported that no changes in BD-rates, run times, and subjective quality are observed. Moreover, the luma virtual boundary of adaptive loop filtering is shifted to three pixels above the horizontal LCU boundary to match the proposed deblocking. Simulation results also reportedly show no changes in BD-rates, run times, and subjective quality.

# Introduction

Deblocking filter (DF) contains horizontal filtering across vertical edges and vertical filtering across horizontal edges. When the largest coding units (LCUs) are processed by DF in a raster scan order, each LCU needs pixels from its upper LCU. Therefore, pixel line buffers of sizes proportional to the picture width are required. In High Efficiency Video Coding test model version 5.0 (HM-5.0), for luma vertical filtering, four intermediate pixels (i.e. pixels after horizontal filtering) on each side of an edge are read for filtering decisions, and at most three pixels are filtered and written back, which requires four luma line buffers. Because reading four pixels and writing three pixels are required above horizontal LCU boundaries in HM-5.0, we call it “Read 4 and Write 3” case, which is R4W3 in short. In JCTVC-G230 [1], modified DF with R3W2 was proposed for the upper side of the horizontal LCU boundary. In this contribution, JCTVC-G230 is tested on top of HM-5.0. The rest of this contribution is organized as follows. Details of the proposed method will be described in Section 2, simulation results will be shown in Section 3, and conclusions will be given in Section 4.

# Proposed method

In the following, Figure 1 can be rotated in the clockwise direction by 90 degrees and then flipped in the horizontal direction for demonstrating a horizontal edge where p pixels and q pixels are above and below the edge, respectively. The proposed method is applied only for horizontal LCU boundaries, and the DF in HM-5.0 is applied for rest edges. In HM-5.0, filtering decisions are applied for luma only and are bypassed for chroma. Intermediate pixels are used in filtering decisions, which contain two parts: filter usage (on/off) decision and filter strength (strong/weak) selection. Eight columns are divided into two segments, where each segment contains four columns. Each segment has its own filtering decisions.

Figure 1.  Pixels involved in filtering an edge

The filter usage decision is as follows. First, eight values are calculated from samples of four columns:

dp0=p20−2⋅p10+p00

dq0=q20−2⋅q10+q00

dp3=p23−2⋅p13+p03

dq3=q23−2⋅q13+q03

dp4=p24−2⋅p14+p04

dq4=q24−2⋅q14+q04

dp7=p27−2⋅p17+p07

dq7=q27−2⋅q17+q07

By the threshold operation

dp0+dq0+dp3+dq3<β

it is decided if the first segment (i.e. the four columns with the indexes 0, 1, 2, and 3) is filtered or not.

By the threshold operation

dp4+dq4+dp7+dq7<β

it is decided if the second segment (i.e. the four columns with the indexes 4, 5, 6, and 7) is filtered or not.

If the above decision is true for a segment of 4 columns, a filter strength selection is performed.

The first segment of columns 0-3 is filtered with strong filtering if the following two conditions

are true. Otherwise, weak filtering is applied.

The second segment of columns 4-7 is filtered with strong filtering if the following two conditions

are true. Otherwise, weak filtering is applied.

In the proposed R3W2 DF, only three pixels (p0-p2) above the horizontal LCU boundary are used in the filtering decisions. Therefore, the filter usage decision is kept unchanged. However, the proposed filter strength selection is different.

The first segment of columns 0-3 is filtered with strong filtering if the following two conditions

are true. Otherwise, weak filtering is applied.

The second segment of columns 4-7 is filtered with strong filtering if the following two conditions

are true. Otherwise, weak filtering is applied.

Moreover, the luma filtering operations are also modified with changing at most two pixels on the upper side of the horizontal LCU boundary. The original strong and weak filters for luma are described as follows.

Original strong filter for luma:

p2i=Clip0,255,2⋅p3i+3⋅p2i+p1i+p0i+q0i+4>>3

p1i=Clip0,255,p2i+p1i+p0i+q0i+2>>2

p0i=Clip0,255,p2i+2⋅p1i+2⋅p0i+2⋅q0i+q1i+4>>3

q0i=Clip0,255,p1i+2⋅p0i+2⋅q0i+2⋅q1i+q2i+4>>3

q1i=Clip0,255,p0i+q0i+q1i+q2i+2>>2

q2i=Clip0,255,p0i+q0i+q1i+3⋅q2i+2⋅q3i+4>>3

Original weak filter for luma:

Δ=Clip−tc,tc,9⋅q0i−p0i−3⋅q1i−p1i+8>>4

p0i=Clip0,255,p0i+Δ

q0i=Clip0,255,q0i−Δ

p0 and q0 will be modified if abs() < (thr1) ,where thr1 is set equal to 10*tc.

Δ1=Clip−tc2,tc2,(p0i+p2i+1)>>1−p1i+Δ>>1

p1i=Clip0,255,p1i+Δ1

p1 will be modified if abs() < (thr1) and dp<thr2, where

dp=p20−2⋅p10+p00+p23−2⋅p13+p03 for the first segment of columns 0-3, and

dp=p24−2⋅p14+p04+p27−2⋅p17+p07 for the second segment of columns 4-7, and

thr2 is set equal to (β+(β>>1))>>3

Δ2=Clip−tc2,tc2,(q0i+q2i+1)>>1−q1i+Δ>>1

q1i=Clip0,255,q1i+Δ2

q1 will be modified if abs() < (thr1) dq<thr2, where

dq=q20−2⋅q10+q00+q23−2⋅q13+q03 for the first segment of columns 0-3, and

dq=q24−2⋅q14+q04+q27−2⋅q17+q07 for the second segment of columns 4-7, and

thr2 is set equal to (β+(β>>1))>>3

Figure 2 shows waveforms of pre-deblocking (black waveform with blue pixels) and post-deblocking (green waveform with green pixels) in strong filtering. When the example of the simple waveform (p2i=p1i=p0i, q0i=q1i=q2i) in Figure 2 is considered, in HM-5.0 pixel p2i is changed by strong filtering to p’2i and the difference between p’2i and p2i is as follows.

p’2i - p2i 	= (2*p3i + 3*p2i + p1i + p0i + q0i + 4)>>3 - p2i

= (2*p3i - 5*p2i + p1i + p0i + q0i + 4)>>3

= (-p1i + q0i + 4)>>3				(p2i=p1i=p0i, q0i=q1i=q2i)

= (-D + 4)*1/8					(D= p0i - q0i)

Figure 2.  Pixels performed by strong filter

In HM-5.0, pixels of p1i, p0i, q0i, q1i, and q1i are also changed in a similar way as follows.

p’1i – p1i 	= (-2*D + 4)*1/8

p’0i – p0i 	= (-3*D + 4)*1/8

q’0i – q0i 	= (3*D + 4) *1/8

q’1i – q1i 	= (2*D + 4) *1/8

q’2i – q2i 	= (D + 4) *1/8

The proposed R3W2 deblocking does not change p2i. The straightforward filtered waveform (p2i-p1’i-p0’i) would not be as smooth as the originally filtered waveform (p2’i-p1’i-p0’i). If p1i is changed into the central position p1”i (the red pixel in Figure 2) between p2i and p0’i, the newly filtered waveform (p2i-p1”i-p0’i) will become smoother than the straightforward filtered waveform (p2i-p1’i-p0’i). Therefore, we propose the filtering operation of p1i as follows.

p’’1i – p1i 	= (-3*D + 2*4) *1/16

= (-3*( p0i - q0i) + 8)*1/16

= (13* p0i + 3*q0i + 8)>>4 – p1i

= (4* p2i + 5*p1i + 4*p0i + 3*q0i + 8)>>4 – p1i		(a)

Each coefficient in (a) is decided based on both the same pixels as the original filtering operation of p1i and the weight of central position p1i.

Now the luma filtering operations are modified as follows.

Proposed strong filter for luma:

p1i=Clip0,255,4⋅p2i+5⋅p1i+4⋅p0i+3⋅q0i+8>>4

p0i=Clip0,255,p2i+2⋅p1i+2⋅p0i+2⋅q0i+q1i+4>>3

q0i=Clip0,255,p1i+2⋅p0i+2⋅q0i+2⋅q1i+q2i+4>>3

q1i=Clip0,255,p0i+q0i+q1i+q2i+2>>2

q2i=Clip0,255,p0i+q0i+q1i+3⋅q2i+2⋅q3i+4>>3

Since two pixels above the horizontal LCU boundaries are allowed to be changed, the proposed weak filter for luma is the same as that in HM-5.0.

Please note that p and q pixels are pixels after horizontal deblocking. The proposed filtering decisions and filtering operations only use p0-p2 and q0-q3. Therefore, only three luma line buffers are required, which saves one luma line compared with HM-5.0.

# Simulation results

Experiments were conducted according to the common test conditions defined in JCTVC-G1200 [2], and the software is based on HM-5.0. Simulation results of the proposed method are shown in Table 1. The modification does not cause any noticeable change in terms of BD-rate and run time. After careful viewing, we also conclude the visual quality of the proposed method is the same as HM-5.0 under common test conditions.

When adaptive loop filtering (ALF) is enabled, its luma virtual boundary (VB) [3] should be shifted from four to three pixels above the horizontal LCU boundary in order to match the proposed DF. Simulation results of the proposed DF in combination with the shifted VB are shown in Table 2. No noticeable changes in BD-rates, run times, and visual quality are observed.

# Conclusions

In this contribution, deblocking filter is changed from “read four pixels write three pixels (R4W3)” to “read three pixels write two pixels (R3W2)” on the upper side of the horizontal LCU boundary. The proposed method can save one pixel line buffer. Simulation results reportedly show no changes in bit rates and run times. The subjective quality is also not changed, which needs to be confirmed by the viewing session in San Jose.

# References

- C.-W. Hsu, Y.-W. Huang, S. Lei, M. Ikeda, T. Suzuki, S. Park, B. Jeon, “Non-CE12.3: Reducing pixel line buffers by modifying DF to R3W2 for horizontal LCU edges,” Document of Joint Collaborative Team on Video Coding, JCTVC-G230, November 2011.

- F. Bossen, “Common HM test conditions and software reference configurations,” Document of Joint Collaborative Team on Video Coding, JCTVC-G1200, November 2011.

- C.-Y. Chen, C.-M. Fu, C.-Y. Tsai, Y.-W. Huang, S. Lei, S. Esenlik, M. Narroschke, T. Wedi, I. S. Chong, M. Karczewicz, “Non-CE8.c.7: Single-source SAO and ALF virtual boundary processing with cross9x9,” Document of Joint Collaborative Team on Video Coding, JCTVC-G212, November 2011.

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Sony Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Table 1.  Results of the proposed DF

Table 2.  Results of the proposed DF in combination with the modified VB