| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 8th Meeting: San José, CA, USA, 1–10 February, 2012 | Document: JCTVC-H0281 |
| --- | --- |

| Title: | Non-CE10: Removing QP line buffer |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yi-Wen Chen, Jian-Liang Lin, Yu-Wen Huang, Shawmin Lei No. 1, Dusing Rd. 1, Hsinchu Science Park, Hsinchu, Taiwan 30078 | Tel: Email: | Shawmin Lei +886-3-5670766 ext. 25555 {yiwen.chen, jl.lin,yuwen.huang, shawmin.lei}@mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

In HM-5.0, deblocking filter (DF) employs the average of QPs on both sides of an edge to look up the β and tc parameters. When LCUs are processed by DF in a raster scan order, each LCU needs the QP values from its upper LCU. Therefore, a line buffer of 3.9K bits is required to store QPs of upper LCUs for 4Kx2K videos in hardware. Since the QP line buffer is only utilized by DF, in this contribution, a simplified DF is proposed to remove the QP line buffer as follows. When processing the horizontal edges between a current LCU and its upper LCU, only the QP of the current LCU is used. Simulation results reportedly show that the proposed method introduces no meaningful change in BD-rates and run times when QP adaptation is enabled on top of the JCTVC-G1200 common test conditions. The proponents also claim the visual quality is the same as HM-5.0.

# Introduction

In High-Efficiency Video Coding test model version 5.0 (HM-5.0), deblocking filter (DF) is utilized as an in-loop filter to reduce the blocking artifacts. DF contains horizontal filtering across vertical edges and vertical filtering across horizontal edges. Individual tc and ß parameters are derived for the blocks corresponding to an edge to be deblocked. In case of edges belonging to two blocks with different quantization parameter (QP) values, QPP and QPQ, the average of QPP and QPQ is employed to look up the β and tC parameters for the current edge to be processed: QP = ( QPP + QPQ +1)>>1. For example, as shown in Figure 1, each block represents the minimum dQP signaling block and has its own QP value. When DF is processing the horizontal edge between block 3 and block 7, the average of QP-3 and QP-7 is used to derive the parameters for deblocking. Similarly, the average of QP 6 and QP-7 is used to derive the parameters for deblcking the vertical edge between block 6 and block 7. When the largest coding units (LCUs) are processed by DF in a raster scan order, each LCU needs QP values from its upper LCU, which results in additional line buffer for storing QP values from the upper LCU row. When the size of minimum dQP block is 8x8, each LCU requires 8x6 (6 bits is used to represent QP values) bits to store all QP values for deblocking. In this manner, a line buffer of 3.9K bits is required to store QP values of upper LCU row for real-time processing of 4Kx2K videos in hardware. In this contribution, we propose a method to remove the QP line buffer. The rest of this contribution is organized as follows. Details of the proposed methods will be described in Section 2, simulation results will be shown in Section 3, and conclusions will be given in Section 4.

Figure 1.  An example for deblocking process when explicit signaling of dQP is enabled

# Proposed method

In this proposal, we propose to remove the line buffer needed for storing the QP values for the deblocking of horizontal edges between two LCUs. For each to-be-deblocked edge that needs to access the QP values of the upper LCU row, DF will employ only the QP below the horizontal LCU boundary. For example, as shown in Figure 1, if the horizontal edge between block 3 and block 7 is an LCU boundary, DF will employ only QP-7 to look up the β and tc parameters.

# Simulation results

Experiments were conducted according to the common test conditions defined in JCTVC-G1200 [1] except that QP adaptation is enabled to demonstrate the influence of the proposed method. Specifically, the anchor is based on HM-5.0 with MaxQPAdaptationRange=6, MaxCUDQPDepth=3, and with QP adaptation based on the psycho-visual model TM5Step3. The proposed method is already implemented in the anchor software. Simulation results of the proposed method are shown in Table 1. The proposed method does not cause noticeable changes in BD-rates and run times. With careful evaluation of the subjective quality, we did not observe any undesirable visual artifact caused by the proposed method. The visual quality of the proposed method is the same as that of anchor.

# Conclusions

In this contribution, the QP line buffer can be removed by using QP values below a horizontal edge processed by deblocking only when the edge is a horizontal LCU boundary. Simulation results reportedly show no change in bit rates and run times and the same subjective quality in comparison with HM-5.0.

# References

- F. Bossen, “Common HM test conditions and software reference configurations,” Document of Joint Collaborative Team on Video Coding, JCTVC-G1200, November 2011.

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Table 1.  Results of the QP line buffer removal