This section describes the working draft changes for luma. The changes for chroma conform to the luma changes.

8.6.1.4.1 Decision process for luma block edge

Inputs of this process are:

–	a luma location ( xC, yC ) specifying the top-left luma sample of the current coding unit relative to the top left luma sample of the current picture,

–	a luma location ( xB, yB ) specifying the top-left luma sample of the current block relative to the top left luma sample of the current coding unit,

–	a variable verticalEdgeFlag,

–	a variable bS specifying the boundary filtering strength,

Output of this process is:

–	variables dE, dEp1 and dEq1 containing decisions,

–	one-dimensional array of size (8), dS containing decisions.

Let s’ represent the luma sample array recPictureL of the current picture.

Depending on verticalEdgeFlag, the following applies:

–	If verticalEdgeFlag is equal to 1, the following ordered steps apply:

- The sample values pi,k and qi,k with i = 0..3 and k = 2,5 are derived as follows:

qi,k = s’[ xC + xB +i, yC + yB + k ]		(8-1)

pi,k = s’[ xC + xB – i – 1, yC + yB + k ]		(8-2)

- The quantization parameter QPP of the neighboring block is assigned the QPY value of the coding unit containing sample p0,0

- The loop filter quantization parameter QPL is derived as follows:

QPL = ( ( QPY + QPP + 1) >>1 )

- A variable β is specified as Error: Reference source not found with loop filter quantization parameter QPL as input.

- A variable βP is specified as Error: Reference source not found with quantization parameter QPP as input.

- A variable βQ is specified as Error: Reference source not found with luma quantization parameter QPY as input.

- A variable tC is specified as follows:

  - If bS is greater than 2, the variable tC is specified as Error: Reference source not found with quantization parameter Clip3(0, 55, QPL + 2 ) as input,

  - Otherwise (bS is equal or less than 2), the variable tC is specified as Error: Reference source not found with quantization parameter QPL as input.

- The variables dp, dq and d are derived as follows:

dp = | p2,2 – 2*p1,2 + p0,2 | + | p2,5 – 2*p1,5 + p0,5 |		(8-3)

dq = | q2,2 – 2*q1,2 + q0,2 | + | q2,5 – 2*q1,5 + q0,5 |		(8-4)

d = dp + dq		(8-5)

- The variables dE, dEp1 and dEq1 are set equal to 0.

- If bS is not equal to 0 and d is less than β, the following ordered steps apply:

- for each sample location ( xC + xB, yC + yB + k ), k = 0..7, the following ordered steps apply:

  - The decision process for a luma sample specified in subclause Error: Reference source not found is invoked with sample values pi,k, qi,k with i = 0..3, the boundary filtering strength bS and the variables d, β and tC as inputs and a decision dSam as output.

  - The variable dS[k] is set equal to dSam

- The variable dE is set equal to 1.

- If dp is less than ( βP + ( βP >> 1 ) ) >> 3, the variable dEp1 is set equal to 1.

- If dq is less than ( βQ + ( βQ >> 1 ) ) >> 3, the variable dEq1 is set equal to 1.

–	Otherwise (verticalEdgeFlag is equal to 0), the following ordered steps apply:

- The sample values pi,k and qi,k with i = 0..3 and k = 2,5 are derived as follows:

qi,k = s’[ xC + xB +k, yC + yB + i ]		(8-6)

pi,k = s’[ xC + xB +k, yC + yB – i – 1 ]		(8-7)

- The quantization parameter QPP of the neighboring block is assigned the QPY value of the coding unit containing sample p0,0

- The loop filter quantization parameter QPL is derived as follows:

If (xC + xB +k >> Log2MaxCuSize ) is equal to (xC >> Log2MaxCuSize ) and ( yC + yB – i – 1 >> Log2MaxCuSize ) is equal to (yC >> Log2MaxCuSize )

QPL = ( ( QPY + QPP +1 ) >> 1 )

else

QPL =  QPY

- A variable β is specified as Error: Reference source not found with loop filter quantization parameter QPL as input.

- A variable βP is specified as Error: Reference source not found with quantization parameter QPP as input.

- A variable βQ is specified as Error: Reference source not found with quantization parameter QPY as input.

- A variable tC is specified as follows:

  - If bS is greater than 2, the variable tC is specified as Error: Reference source not found with quantization parameter Clip3(0, 55, QPL + 2 ) as input,

  - Otherwise (bS is equal or less than 2), the variable tC is specified as Error: Reference source not found with loop filter quantization parameter QPL as input.

- The variables dp, dq and d are derived as follows:

dp = | p2,2 – 2*p1,2 + p0,2 | + | p2,5 – 2*p1,5 + p0,5 |		(8-8)

dq = | q2,2 – 2*q1,2 + q0,2 | + | q2,5 – 2*q1,5 + q0,5 |		(8-9)

d = dp + dq		(8-10)

- The variables dE, dEp1 and dEq1 are set equal to 0.

- If bS is not equal to 0 and d is less than β, the following ordered steps apply:

- For each sample location ( xC + xB + k, yC + yB ), k = 0..7, the following ordered steps apply:

  - The decision process for a luma sample specified in subclause Error: Reference source not found is invoked with sample values pi,k, qi,k with i = 0..3, the boundary filtering strength bS and the variables d, β and tC as inputs and a decision dSam as output.

  - The variable dS[k] is set equal to dSam.

- The variable dE is set equal to 1.

- If dp is less than ( βP + ( βP >> 1 ) ) >> 3, the variable dEp1 is set equal to 1.

- If dq is less than ( βQ + ( βQ >> 1 ) ) >> 3, the variable dEq1 is set equal to 1.