| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 8th Meeting: San José, CA, USA, 1–10 February, 2012 | Document: JCTVC-H0282 |
| --- | --- |

| Title: | CE11.1.2: Simplified method for coding 4x4 & 8x8 significance maps |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Shih-Ta Hsiang, T.-D. Chuang and Shawmin Lei MediaTek Inc. No. 1, Dusing Rd. 1, Hsinchu Science Park Hsinchu, Taiwan 30078 | Tel: Email: | Shawmin Lei +886-3-5670766 ext. 25555 {shih-ta.hsiang, shawmin.lei}@mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

This contribution proposes a modified method to simplify the context modeling scheme for coding significance maps. The proposed method employs the same context configuration map for coding 4x4 and 8x8 transform block types and for coding Luma and Chroma components. The context selection can be determined by simple logic without a use of look-up tables. The total number of the contexts is increased by 7 (2 for Luma and 5 for Chroma) compared to HM5.0. The simulation results reportedly show Y BD-rate increases 0.0%, 0.0%, and 0.1% for HE-AI, HE-RA and HE-LDB, and 0.1%, 0.1%, and 0.1% for LC-AI, LC-RA and LC-LDB under the common test condition.

# Introduction

This contribution reports the CE11 results for our proposed method to simplify coding significance maps. The proposed method has the coefficients from the highest frequency region share the same context in order to reduce the number of contexts and simplify context selection, as illustrated in Figure 1.

Figure 1: The proposed set of context configuration maps BR6.

The actual context assignment for the configuration maps in Figure is provided in Figure 2. The context index uiCtx can be easily derived by

if ( uiLog2BlkSize < 4)

{

uiCtx =  uiLog2BlkSize == 2? ( uiPosY << 2 ) + uiPosX : ((uiPosY >> 1) << 2) + (uiPosX >> 1);

uiCtx = (uiLog2BlkSize == 2? 0 : 11) + (uiCtx < 10 ? (uiCtx == 7? 10 : uiCtx) : (uiCtx == 12? 7 : 10));

}

Figure 1: The proposed set of context configuration maps BR6

# Experimental Results

The proposed context modeling method has been implemented using HM-5.0 as software base and experimentally evaluated according to the common test conditions defined in JCTVC-G1200 Error: Reference source not found. In addition, we further provide the simulation results over a high bitrate range using QP = 12, 17, 22, and 27. The experimental results are provided in the included spread sheets and summarized in Tables 1 and 2.

Table 1: Summary of the BD-rate results according to the common test condition

|  | All Intra HE |  |  | All Intra LC |  |  | All Intra HE-10 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A (8bit) | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| Class B | 0.0% | 0.1% | 0.1% | 0.1% | 0.0% | 0.0% |  |  |  |
| Class C | 0.0% | 0.1% | 0.1% | 0.1% | 0.1% | 0.0% |  |  |  |
| Class D | 0.0% | 0.2% | 0.1% | 0.1% | 0.1% | 0.0% |  |  |  |
| Class E | 0.0% | 0.1% | 0.2% | 0.0% | 0.0% | 0.0% |  |  |  |
| Overall | 0.0% | 0.1% | 0.1% | 0.1% | 0.0% | 0.0% |  |  |  |
|  | 0.0% | 0.1% | 0.1% | 0.1% | 0.0% | 0.0% |  |  |  |
| Class F | 0.0% | -0.1% | 0.1% | 0.1% | 0.0% | 0.1% |  |  |  |
| Enc Time[%] | 99.2% |  |  | 101.0% |  |  |  |  |  |
| Dec Time[%] | 99.7% |  |  | 100.8% |  |  |  |  |  |
|  | Random Access HE |  |  | Random Access LC |  |  | Random Access HE-10 |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A (8bit) | 0.0% | 0.1% | -0.2% | 0.0% | 0.1% | -0.3% | 0.1% | 0.0% | 0.6% |
| Class B | 0.0% | 0.1% | 0.0% | 0.1% | 0.2% | 0.3% | 0.0% | 0.1% | 0.1% |
| Class C | 0.0% | 0.0% | 0.3% | 0.1% | 0.2% | 0.2% |  |  |  |
| Class D | 0.0% | 0.3% | 0.4% | 0.2% | 0.6% | 0.2% |  |  |  |
| Class E |  |  |  |  |  |  |  |  |  |
| Overall | 0.0% | 0.1% | 0.2% | 0.1% | 0.3% | 0.1% | 0.0% | 0.1% | 0.3% |
|  | 0.0% | 0.1% | 0.2% | 0.1% | 0.3% | 0.1% | 0.1% | 0.1% | 0.3% |
| Class F | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| Enc Time[%] | 100.6% |  |  | 100.3% |  |  | 99.1% |  |  |
| Dec Time[%] | 98.8% |  |  | 100.2% |  |  | 98.8% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  | Low delay B HE-10 |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |  |  |  |
| Class B | 0.0% | 0.4% | 0.7% | 0.1% | 0.3% | 0.5% |  |  |  |
| Class C | 0.1% | 0.3% | 0.6% | 0.2% | 1.0% | 0.7% |  |  |  |
| Class D | 0.1% | 0.5% | 0.6% | 0.2% | 1.9% | 0.8% |  |  |  |
| Class E | 0.1% | 0.2% | 0.0% | 0.0% | 0.4% | -0.4% |  |  |  |
| Overall | 0.1% | 0.4% | 0.5% | 0.1% | 0.9% | 0.5% |  |  |  |
|  | 0.1% | 0.4% | 0.5% | 0.1% | 0.8% | 0.4% |  |  |  |
| Class F | -0.1% | 0.0% | -0.3% | 0.2% | -0.5% | 0.2% |  |  |  |
| Enc Time[%] | 101.4% |  |  | 100.6% |  |  |  |  |  |
| Dec Time[%] | 100.6% |  |  | 99.7% |  |  |  |  |  |
|  | Low delay P HE |  |  | Low delay P LC |  |  | Low delay P HE-10 |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |  |  |  |
| Class B | 0.0% | 0.3% | 0.3% | 0.1% | 0.5% | 0.2% |  |  |  |
| Class C | 0.1% | 0.4% | 0.1% | 0.1% | 0.6% | 0.5% |  |  |  |
| Class D | 0.1% | 0.9% | 0.6% | 0.1% | 1.2% | 0.6% |  |  |  |
| Class E | 0.0% | 0.3% | 0.7% | 0.1% | 0.4% | -0.8% |  |  |  |
| Overall | 0.1% | 0.5% | 0.4% | 0.1% | 0.7% | 0.2% |  |  |  |
|  | 0.1% | 0.5% | 0.4% | 0.1% | 0.6% | 0.3% |  |  |  |
| Class F | -0.1% | 0.1% | -0.2% | 0.4% | 0.0% | 0.7% |  |  |  |
| Enc Time[%] | 100.6% |  |  | 100.6% |  |  |  |  |  |
| Dec Time[%] | 100.2% |  |  | 100.2% |  |  |  |  |  |

Table 2: Summary of the BD-rate results for QP = 12, 17, 22, 27

|  | All Intra HE |  |  | All Intra LC |  |  | All Intra HE-10 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A (8bit) | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| Class C | 0.1% | 0.1% | 0.1% | 0.1% | 0.1% | 0.1% |  |  |  |
| Class D | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| Class E | 0.0% | 0.1% | 0.1% | 0.0% | 0.0% | 0.0% |  |  |  |
| Overall | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
|  | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| Class F | 0.0% | 0.0% | 0.0% | 0.1% | 0.1% | 0.1% |  |  |  |
| Enc Time[%] | 99% |  |  | 97% |  |  |  |  |  |
| Dec Time[%] | 96% |  |  | 98% |  |  |  |  |  |
|  | Random Access HE |  |  | Random Access LC |  |  | Random Access HE-10 |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A (8bit) | 0.0% | 0.1% | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% | 0.1% | 0.1% |
| Class B | 0.0% | 0.1% | 0.1% | 0.0% | 0.2% | 0.3% | 0.0% | 0.1% | 0.0% |
| Class C | 0.1% | 0.3% | 0.2% | 0.1% | 0.4% | 0.4% |  |  |  |
| Class D | 0.1% | 0.3% | 0.4% | 0.1% | 0.6% | 0.6% |  |  |  |
| Class E |  |  |  |  |  |  |  |  |  |
| Overall | 0.0% | 0.2% | 0.2% | 0.1% | 0.3% | 0.3% | 0.0% | 0.1% | 0.0% |
|  | 0.0% | 0.2% | 0.2% | 0.1% | 0.3% | 0.3% | 0.0% | 0.0% | 0.1% |
| Class F | -0.1% | 0.0% | 0.0% | 0.1% | 0.1% | 0.2% |  |  |  |
| Enc Time[%] | 98% |  |  | 97% |  |  | 101% |  |  |
| Dec Time[%] | 98% |  |  | 97% |  |  | 101% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  | Low delay B HE-10 |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |  |  |  |
| Class B | 0.0% | 0.1% | 0.2% | 0.1% | 0.2% | 0.2% |  |  |  |
| Class C | 0.1% | 0.2% | 0.3% | 0.2% | 0.6% | 0.6% |  |  |  |
| Class D | 0.1% | 0.5% | 0.5% | 0.1% | 1.0% | 0.9% |  |  |  |
| Class E | 0.1% | 0.2% | 0.9% | 0.0% | 0.6% | 0.7% |  |  |  |
| Overall | 0.1% | 0.2% | 0.4% | 0.1% | 0.6% | 0.6% |  |  |  |
|  | 0.1% | 0.3% | 0.4% | 0.1% | 0.6% | 0.6% |  |  |  |
| Class F | 0.0% | 0.0% | 0.1% | 0.5% | 0.5% | 0.3% |  |  |  |
| Enc Time[%] | 97% |  |  | 100% |  |  |  |  |  |
| Dec Time[%] | 96% |  |  | 98% |  |  |  |  |  |
|  | Low delay P HE |  |  | Low delay P LC |  |  | Low delay P HE-10 |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |  |  |  |
| Class B | 0.0% | 0.0% | 0.2% | 0.1% | 0.2% | 0.2% |  |  |  |
| Class C | 0.1% | 0.2% | 0.3% | 0.2% | 0.6% | 0.4% |  |  |  |
| Class D | 0.1% | 0.4% | 0.4% | 0.1% | 0.6% | 0.8% |  |  |  |
| Class E | 0.0% | 0.1% | 1.0% | 0.2% | 0.5% | 0.2% |  |  |  |
| Overall | 0.1% | 0.2% | 0.4% | 0.1% | 0.4% | 0.4% |  |  |  |
|  | 0.1% | 0.2% | 0.4% | 0.1% | 0.4% | 0.4% |  |  |  |
| Class F | -0.1% | 0.0% | 0.1% | 0.4% | 0.2% | 0.2% |  |  |  |
| Enc Time[%] | 99% |  |  | 98% |  |  |  |  |  |
| Dec Time[%] | 101% |  |  | 98% |  |  |  |  |  |

# References

- Shih-Ta Hsiang and Shawmin Lei, “Non-CE11: Context reduction for coding transform coefficients,” Document of Joint Collaborative Team on Video Coding, JCTVC-G448, 7th Meeting: Geneva, CH, 21-30 November, 2011.

- F. Bossen, “Common HM test conditions and software reference configurations,” Document of Joint Collaborative Team on Video Coding, Document of Joint Collaborative Team on Video Coding, JCTVC-G1200, 7th Meeting: Geneva, CH, 21-30 November, 2011.

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).