## Inverse scanning process for transform coefficients

Inputs to this process are:

–	a variable nW specifying the width of the current transform unit,

–	a variable nH specifying the height of the current transform unit.

–	a list of (nW)x(nH) values transCoeffLevel[ xT ][ yT ][ trafoDepth ][ cIdx ].

–	a variable cIdx specifying the chroma component of the current block.

Output of this process is a variable c containing a two-dimensional array of (nW)x(nH) values.

The variable nS is set equal to Sqrt(nW*nH).

The variable scanIdx, indicating wich scan is used to scan the transform coefficients (diagonal, horizontal or vertical), is derived as follows.

- If PredMode is equal to MODE_INTRA, the following applies.

  - If cIdx is equal to 0, scanIdx is set to ScanType[ Log2( nS >> 2 ) ][ IntraPredMode ].

  - Otherwise (cIdx is equal to 1 or 2), scanIdx is set to ScanType[ Log2( nS >> 2 ) ][ IntraPredModeC ]

- Otherwise (PredMode is not equal to MODE_INTRA),

If nW is equal to nH, scanIdx is set to 3,

Otherwise if nW is larger than nH, scanIdx is set to 2, otherwise scanIdx is set to 1.

The variable c[ i ][ j ] which is located in the ( i, j ) position in the array c is derived as follows.

- The transform coefficient level at scanning position n is mapped to the position ( i, j ) in the array c using the scan specified by scanIdx as follows.

i = ScanOrder[ Log2( nS >> 2 ) ][ Log2( nS >> 2 ) ][ scanIdx ][ n ][ 0 ]			
j = ScanOrder[ Log2( nS >> 2 ) ][ Log2( nS >> 2 ) ][ scanIdx ][ n ][ 1 ]			
c[ i ][ j ] = transCoeffLevel[ xT ][ yT ][ trafoDepth ][ cIdx ][ n ],			
with n = 0.. ( nS * nS ) – 1	(8-1)

###### Derivation process of ctxIdxInc for the syntax element significant_coeff_flag

Inputs to this process are the color component index cIdx, the current coefficient scan position ( xC , yC ) and the transform block size log2TrafoSize and transform block width nW and height nH .

Output of this process is ctxIdxInc.

The variable sigCtx depends on the current position ( xC, yC ), the transform block size and previously decoded bins of the syntax element significant_coeff_flag. For the derivation of sigCtx, the following applies.

- If log2TrafoSize is less than or equal to 3, sigCtx is derived as follows.

shift  	=  log2TrafoSize  = =  3 ? 1 : 0
sigCtx  =  ( shift  *  15 )  +  ( ( yC  >>  shift )  <<  2 )  +  ( xC  >>  shift )	(9-2)

- Otherwise if nW is not equal to nH, sigCtx is derived as follows

shift  	=  log2TrafoSize  = =  3 ? 1 : 0
limit    =  4 << shift
xM = ( xC  >  limit ) ? limit : xC
yM = ( yC  >  limit ) ? limit : yC
sigCtx  =  ( shift  *  15 )  +  ( ( yC  >>  shift )  <<  2 )  +  ( xC  >>  shift )	(9-3)