| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 8th Meeting: San José, CA, USA, 1–10 February, 2012 | Document: JCTVC-H0284 |
| --- | --- |

| Title: | Non-CE11: Unification of transform coefficient coding |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Chih-Wei Hsu, Tzu-Der Chuang, Yu-Wen Huang, Shawmin Lei No. 1, Dusing Rd. 1, Hsinchu Science Park, Hsinchu, Taiwan 30078 | Tel: Email: | Shawmin Lei +886-3-5670766 ext. 25555 {cw.hsu, peter.chuang, yuwen.huang, shawmin.lei}@mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

In HM-5.0, one TU is divided into subsets for coefficient coding. The subset partitioning of 8x8 TU is different from other TUs. Moreover, for position-based context selections of 4x4 and 8x8 TUs, different context selection look-up tables are used. In this contribution, two unifications are proposed for the coefficient coding. One is to unify the subset partitioning of 8x8 TU with other TUs. The other is to unify the position-based context selection look-up tables between 4x4 and 8x8 TUs. Simulation results reportedly show no meaningful differences in BD-rates and run times in comparison with HM-5.0.

# Introduction

In High-Efficiency Video Coding test model version 5.0 (HM-5.0), transform coefficients are coded transform unit (TU) by TU using context-based adaptive binary arithmetic coding (CABAC). In each TU, a last_significant_coeff_x and a last_significant_coeff_y are transmitted first to indicate the last non-zero coefficient position in a certain scan order. Then, the TU is divided into several subsets when the TU size is larger than 4x4. For one 8x8 TU, the 64 coefficients are divided into 4 subsets according to the diagonal scan through the entire 8x8 TU. Each subset contains 16 continuous coefficients. However, for TU sizes larger than 8x8 (e.g. 16x16, 32x32) and non-square TUs (e.g. 16x4, 4x16, 32x8, 8x32), the TU is divided into 4x4 sub-blocks as proposed in JCTVC-G323 [1] and JCTVC-G1038 [2]. Each sub-block is a coefficient subset. For each subset, a significance map (significant_coeff_flag) indicating whether each coefficient is nonzero is first coded. Next, coeff_abs_level_greater1_flag, coeff_abs_level_greater2_flag, coeff_abs_level_minus3, and coeff_sign_flag are utilized to represent level information of each non-zero coefficient.

Moreover, if the TU size equals to 16x16, 32x32, 16x4, 4x16, 32x8, or 8x32, a 2-level significance map proposed in JCTVC-G644 [3] is used with the following two steps. First, one significant_coeffgroup_flag is coded for each sub-block prior to the level coding of the sub-block. If significant_coeffgroup_flag equals to 0, it indicates the entire 4x4 sub-block is zero and the level coding of the sub-block can be skipped. Please note that the significant_coeffgroup_flag is inferred as 1 for the upper left sub-block that contains the DC coefficient. Next, the significant_coeff_flag is coded with context models.

Different context selection methods are adopted for different TU sizes as proposed in JCTVC-G1015 [4]. For the TU size equals to 4x4 or 8x8, the context selection is based on coefficient positions with different look-up tables. Figure 1 shows the position-based context selection look-up table for 4x4 TU. Figure 2 shows the position-based context selection look-up table for 8x8 TU. A number denotes a context used for a position of a TU.

Figure 1.  Position-based context selection look-up table for 4x4 TU

Figure 2.  Position-based context selection look-up table for 8x8 TU

For other TU sizes, the neighbor-based context selection is adopted. Figure 3 shows an example of the neighbor-based context selection. One context is used for the DC coefficient. For other AC coefficients, the context selection depends on neighboring coefficients. For example, the number of non-zero coefficients of I, H, F, E, and B are used for the context selection. If none of them is non-zero, the context #0 is used for X. If one or two of them are non-zero, the context #1 is used for X. Otherwise the context #2 is used for X. As shown in Figure 3, the luma TU is divided into two regions (Region 1 & Region 2) for AC coefficients, where each region has a context set containing three contexts (#0, #1, #2), while the chroma TU uses only one context set for AC coefficients.

Figure 3.  Neighbor-based context selection

In this contribution, we propose two unifications for the above coefficient coding processes in HM-5.0. One is to unify the subset partitioning of 8x8 TU with other TUs. The other is to unify the position-based context selection look-up table between 4x4 and 8x8 TUs. The proposed unified methods are studied on top of HM-5.0. The rest of this contribution is organized as follows. Details of the proposed methods will be described in Section 2, simulation results will be shown in Section 3, and conclusions will be given in Section 4.

# Proposed methods

## Unification of subset partitioning for 8x8 TU

Transform coefficients in a TU are divided into several subsets, and each subset performs similar coding operations. However, in HM-5.0, there are two subset partitioning methods. One is to partition subsets using a diagonal scan through the entire 8x8 TU, where each subset contains continuous coefficients in the scan order. The other is to partition subsets using a 2-level diagonal scan with 4x4 sub-blocks. From hardware implementation point of view, it is preferable to unify the subset partitioning methods. Then most of the hardware logics with the same operations can be shared.

Hence, we propose to unify the subset partitioning for 8x8 TU with other TU sizes using 4x4 sub-blocks. That is, instead of using the diagonal scan through the entire 8x8 TU, the 8x8 TU will be partitioned into 4x4 sub-blocks, and the 2-level diagonal scan will be used. In that way, 4x4 sub-blocks will be used as subsets for coefficient coding, which is unified with larger TU sizes and non-square TUs. The 2-level diagonal scanning for 8x8 TU is shown in Figure 4.

Figure 4.  Proposed 2-level diagonal scan for 8x8 TU

## Unification of significance map context selection for 4x4 and 8x8 TUs

When the position-based context selection method is used for 4x4 and 8x8 TUs, look-up tables will be needed to store the context selection for each coefficient position. In HM-5.0, there are three look-up tables, i.e. one for 4x4 luma component, one for 4x4 chroma components, and one for both 8x8 luma and chroma components, as shown in Figure 1 and Figure 2. From hardware implementation point of view, the more the look-up tables, the higher the hardware cost. It is preferable to keep the number of look-up tables as small as possible.

Consequently, we propose to unify position-based context selections between different TU sizes (4x4 and 8x8) and different color components (luma and chroma) in order to save the number of required look-up tables. Two methods are stated as follows.

In method 1, we reuse the context selection look-up table of 4x4 luma TU for both 8x8 luma and chroma TUs. For one 8x8 TU including both luma and chroma components, except the DC position, each of the rest positions will perform 2:1 down-scaling in both horizontal and vertical directions to get the down-scaled position, and then the down-scaled position will be used as the input of the 4x4 luma context selection look-up table to get the context. In this method, one 8x8 look-up table can be saved. The unified context selection look-up table for 4x4 and 8x8 TUs is shown in Figure 5.

In method 2, on top of method 1, we also reuse the context selection look-up table of 4x4 luma TU for 4x4 chroma TU. For one 4x4 chroma TU, all positions will use the 4x4 luma context selection look-up table to get contexts. In this method, one 8x8 look-up table and one 4x4 chroma look-up table can be saved. The unified context selection look-up table for 4x4 and 8x8 TUs is shown in Figure 6.

We also propose to use the same pattern in method 2 but without DC context for 8x8 TU. The further unified context selection look-up table for 4x4 and 8x8 TUs is shown in Figure 7.

(a) 4x4 Luma SigMap Context Grouping     (b) 4x4 Chroma SigMap Context Grouping

(c) 8x8 Luma SigMap Context Grouping

Figure 5.  Method 1: unified context selection look-up table for (a) 4x4 luma and (c) 8x8 luma and chroma; And, context selection map for (b) 4x4 chroma

(a) 4x4 Luma and Chroma SigMap Context Grouping

(b) 8x8 Luma and Chroma SigMap Context Grouping

Figure 6.  Method 2: unified context selection look-up table for (a) 4x4 luma and chroma and (b) 8x8 luma and chroma

(a) 4x4 Luma and Chroma SigMap Context Grouping

(b) 8x8 Luma and Chroma SigMap Context Grouping

Figure 6.  Method 2: unified context selection look-up table for (a) 4x4 luma and chroma and (b) 8x8 luma and chroma

# Simulation results

Experiments were conducted according to the common test conditions defined in JCTVC-G1200 [5], and the software is based on HM-5.0. Simulation results of the unification of subset partitioning for 8x8 TU with other TUs are shown in Table 1. Simulations results of the unification of significance map context selection for 4x4 luma TU and 8x8 luma and chroma TUs are shown in Table 2. Simulation results of the unification of significance map context selection for 4x4 luma and chroma TUs and 8x8 luma and chroma TUs are shown in Table 3. Table 4 shows the unification of significance map context selection for 4x4 luma and chroma TUs and 8x8 luma and chroma TUs without individual DC context in 8x8 TU. All the modifications do not cause any noticeable change in terms of BD-rates and run times.

# Conclusions

In this contribution, two unifications are proposed for the coefficient coding in HM-5.0. One is to unify the subset partitioning of 8x8 TU with other TUs. The other is to unify the position-based context selection look-up tables between 4x4 and 8x8 TUs. Simulation results reportedly show no meaningful differences in BD-rates and run times in comparison with HM-5.0.

# References

- J. Sole, R. Joshi, M. Karczewicz, “Non-CE11: Diagonal sub-block scan for HE residual coding,” Document of Joint Collaborative Team on Video Coding, JCTVC-G323, November 2011.

- J. Sole, “JCT-VC break-out report: Harmonization of NSQT with residual coding,” Document of Joint Collaborative Team on Video Coding, JCTVC-G1038, November 2011.

- N. Nguyen, T. Ji, D. He, G. Martin-Cocher, L. Song, “Multi-level significant maps for large transform units,” Document of Joint Collaborative Team on Video Coding, JCTVC-G644, November 2011.

- C. Auyeung, J. Xu, G. Korodi, J. Zan, D. He, Y. Piao, E. Alshina, J. Min, J. Park, “A combined proposal from JCTVC-G366, JCTVC-G657, and JCTVC-G768 on context reduction of significance map coding with CABAC,” Document of Joint Collaborative Team on Video Coding, JCTVC-G1015, November 2011.

- F. Bossen, “Common HM test conditions and software reference configurations,” Document of Joint Collaborative Team on Video Coding, JCTVC-G1200, November 2011.

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Table 1.  Results of the unification of subset partitioning for 8x8 TU with other TUs

Table 2.  Results of the unification of significance map context selection for 4x4 luma TU and 8x8 luma and chroma TUs

Table 3.  Results of the unification of significance map context selection for 4x4 luma and chroma TUs and 8x8 luma and chroma TUs

Table 4.  Results of the unification of significance map context selection for 4x4 luma and chroma TUs and 8x8 luma and chroma TUs without one individual DC context for 8x8 TU