| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 8th Meeting: San José, CA, USA, 1–10 February, 2012 | Document: JCTVC-H0285 |
| --- | --- |

| Title: | Non-CE11 : Block-based significance map context selection |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Tzu-Der Chuang, Chih-Wei Hsu, Ching-Yeh Chen, Yu-Wen Huang, Shawmin Lei No. 1, Dusing Rd. 1, Hsinchu Science Park, Hsinchu, Taiwan 30078 | Tel: Email: | Shawmin Lei +886-3-5670766 ext. 25555 {peter.chuang, cw.hsu, chingyeh.chen, yuwen.huang, shawmin.lei}@mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

In HM-5.0 transform coefficient coding, 4x4 subblock coding is adopted for large TUs and non-square TUs, where the entire TU is divided into two regions for luma. Each region has its own context set for significance map coding. However, the region boundary is a straight line that is not along subblock boundaries and separates the TU into a upper left region and a lower right region. Therefore, some subblocks may need two context sets for the significance map coding, and each coefficient in these subblocks has to check which region it belongs to. In this contribution, a block-based significance map context selection is proposed. The region boundary is along the subblock boundary. In this way, for every subblock, only one context set is required, and it is not necessary to check regions for any coefficient in the subblock. As a further unification, the significant_coeffgroup_flag of subblocks in the upper left region can be always inferred as 1. Simulation results reportedly show no meaningful differences in BD-rates and run times in comparison with HM-5.0.

# Introduction

In High-Efficiency Video Coding test model version 5.0 (HM-5.0), transform coefficients are coded transform unit (TU) by TU using context-based adaptive binary arithmetic coding (CABAC). For large TU (e.g. 16x16, 32x32) and non-square TUs (e.g. 16x4, 4x16, 32x8, 8x32), the TU is divided into 4x4 subblocks as proposed in JCTVC-G323 [1] and JCTVC-G1038 [2]. For each subblock, a significant_coeffgroup_flag is coded to indicate whether this 4x4 subblock has at least one non-zero coefficient or not (as described in JCTVC-G644 [3]). If significant_coeffgroup_flag equals to 1, the significance map (significant_coeff_flag) indicating whether each coefficient is nonzero is first coded. Then, coeff_abs_level_greater1_flag, coeff_abs_level_greater2_flag, coeff_abs_level_minus3, and coeff_sign_flag are utilized to represent level information of each non-zero coefficient. Please note that the significant_coeffgroup_flag is inferred as 1 for the upper left subblock that contains the DC coefficient.

For large TUs and non-square TUs, the entire TU is divided into two regions for luma, as shown in Figure 1. Each region has its own context set for significance map coding. The region boundary is along a straight line that can be represented as x+y=TU_width/4. However, in HM-5.0, the subblock coding is adopted for large TUs and non-square TUs. Some subblocks (e.g. the red block in Figure 1) will cover both region-1 and region-2 coefficients. In these subblocks, two context sets are required, while only one context sets is required for other subblocks. Moreover, in these subblocks, it has to check the criterion of “x+y < (TU_width >> 2)” for every coefficient for context set selection. Extra storage for context sets and extra logic operations for region check are required for these subblocks. In this contribution, we propose a block-based significance map context selection method to solve problems. The rest of this contribution is organized as follows. Details of the proposed method will be described in Section 2, simulation results will be shown in Section 3, and conclusions will be given in Section 4.

Figure 1.  Illustration of context set regions for 16x16 TU and 32x32 TU

# Proposed method

## Block-based significance map context selection

We change the region boundary from a straight line to a few subblock boundaries, as shown in Figure 2. The region selection becomes block-based according to “blk_x + blk_y < (TU_width >> 4)”, where blk_x and blk_y are subblock horizontal index and subblock vertical index, respectively. If a subblock satisfies blk_x + blk_y < (TU_width >> 4), it is a region-1 subblock and uses the region-1 context set for significance map coding; otherwise, it belongs to region-2 and uses the region-2 context set for significance map coding. In this way, only one context set is required for every subblock. Inside each subblock, region check can be totally avoided, so the processing throughput can be improved. Each subblock does not need any extra storage for context sets or extra logic operations for region check.

## Inferred significant_coeffgroup_flag for region-1 subblocks

In HM-5.0, the significant_coeffgroup_flag is inferred as 1 for the upper left subblock that contains the DC coefficient. Hence, the significant_coeffgroup_flag of the region-1 subblock in 16x16 TU is inferred as 1, as shown in Figure 2. However, the significant_coeffgroup_flag of is inferred as 1 for only the first region-1 subblock in 32x32 TU. Therefore, we further unify the significant_coeffgroup_flag coding with the proposed block-based significance map context selection method. The significant_coeffgroup_flag of all region-1 subblocks in 32x32 TU are inferred as 1.

Figure 2.  Block-based significance map context selection for 16x16 TU and 32x32 TU

# Simulation results

The proposed method is implemented on HM-5.0. All the results were simulated according to the common test conditions defined in JCTVC-G1200 [4]. Table 1 shows the results of the block-based significance map context selection. No bit rate increase is observed. All encoding and decoding times are roughly unchanged. Table 2 shows the results of the block-based significance map context selection and the inferred significant_coeffgroup_flag for all region-1 subblocks. Only 0.1% bit rate increase in HE-LDB is found, and the rest conditions do not suffer coding efficiency loss. There are no meaningful changes in run times.

# Conclusions

In this contribution, a block-based significance map context selection is proposed. The region boundary of context set selection is changed to be along subblock boundaries. In this way, for every subblock, only one context set is required, and it is not required to perform the region check for every coefficient inside the subblock. As a further unification, the significant_coeffgroup_flag is inferred as 1 for all subblocks in the upper left region. Simulation results reportedly show no meaningful differences in BD-rates and run times in comparison with HM-5.0.

# References

- J. Sole, R. Joshi, M. Karczewicz, “Non-CE11: Diagonal sub-block scan for HE residual coding,” Document of Joint Collaborative Team on Video Coding, JCTVC-G323, November 2011.

- J. Sole, “JCT-VC break-out report: Harmonization of NSQT with residual coding,” Document of Joint Collaborative Team on Video Coding, JCTVC-G1038, November 2011.

- N. Nguyen, T. Ji, D. He, G. Martin-Cocher, L. Song, “Multi-level significant maps for large transform units,” Document of Joint Collaborative Team on Video Coding, JCTVC-G644, November 2011.

- F. Bossen, “Common HM test conditions and software reference configurations,” Document of Joint Collaborative Team on Video Coding, JCTVC-G1200, November 2011.

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Table 1.  Results of the block-based significance map context selection method

Table 2.  Results of the block-based significance map context selection and the inferred significant_coeffgroup_flag for region-1 subblocks