| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 8th Meeting: San José, CA, USA, 1–10 February, 2012 | Document: JCTVC-H0286 |
| --- | --- |

| Title: | Non-CE11 : Parallel context assignment for significance map coding |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Tzu-Der Chuang, Chih-Wei Hsu, Ching-Yeh Chen, Yu-Wen Huang, Shawmin Lei No. 1, Dusing Rd. 1, Hsinchu Science Park, Hsinchu, Taiwan 30078 | Tel: Email: | Shawmin Lei +886-3-5670766 ext. 25555 {peter.chuang, cw.hsu, chingyeh.chen, yuwen.huang, shawmin.lei}@mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

In HM-5.0 coefficient coding, the neighboring coded significance flags are used for the significance map coding in large TUs and non-square TUs. Since the context selection may depend on the second and the third previously coded coefficients, only 2-bin parallel context assignment can be achieved. This contribution proposes a new context formation pattern that removes the dependency of three previously decoded bins, which can allow 4-bin parallel context assignment. In addition, a 2-level diagonal-vertical scan and a corresponding context formation pattern are also proposed. By doing this, not only 4-bin parallel context assignment can be supported but also the coefficient scan can be simplified with the vertical scan in each 4x4 subblock. Simulation results reportedly show 0.2% bit rate increase in HE conditions and 0.1% bit rate increase in LC conditions.

# Introduction

For the significant_coeff_flag coding of large transform units (TUs, e.g. 16x16, 32x32) and non-square TUs (e.g. 16x4, 4x16, 32x8, 8x32) in High-Efficiency Video Coding test model version 5.0 (HM-5.0), the context selection depends on neighboring values. The context formation pattern of this neighbor-based context selection is shown in Figure 1 and Figure 2(a). For coding coefficient X, five coefficients including I, H, F, E, and B are used for the context formation. In the 7th JCT-VC meeting in Geneva, the subblock coding is adopted for large TUs and non-square TUs [1] [2]. In order to achieve 2-bin parallel context assignment, the context formation pattern is modified as the pattern of Figure 2(b) for the blue coefficients in Figure 2(c). For other white coefficients in Figure 2(c), the pattern of Figure 2(a) is used.

Figure 1.  Context formation pattern of neighbor-based context selection

Figure 2.  Context formation patterns for 2-bin parallel context assignment

It can be seen that the parallelism of context assignment for significance map is limited to two bins in HM-5.0. In this contribution, we propose a new context formation pattern to support 4-bin parallel context assignment with the two-level diagonal scan in HM-5.0. We also propose a new simplified scan called 2-level diagonal-vertical scan with its corresponding context formation pattern, which can still support 4-bin parallel context assignment. The rest of this contribution is organized as follows. Details of the proposed methods will be described in Section 2, simulation results will be shown in Section 3, and conclusions will be given in Section 4.

# Proposed methods

## Context formation pattern for 2-level diagonal scan

In order to support 4-bin parallel context assignment for significance map coding, the data dependency for three previously decoded bins should be removed. The coefficients I, H, and F should not be used in the context formation pattern. Therefore, we propose a new context formation pattern, as shown in Figure 3. This pattern is similar to the original pattern in Figure 2(a) but shifts the position of I, H, F, E, and B to the right side by one pixel.

Figure 3.  Context formation pattern for 2-level diagonal scan

## Context formation pattern for 2-level diagonal-vertical scan

To simplify the coefficient coding in subblocks, we propose a new coefficient scan called 2-level diagonal-vertical scan, which uses vertical scan within 4x4 subblocks, as shown in Figure 4. In comparison with diagonal scan, the data access for vertical scan is more regular and requires fewer logic gates for address generator in hardware implementation. Since the vertical scan is used in subblocks, we propose a context formation pattern that removes the dependency for the coefficients in the same scan line, as shown in Figure 5. Four bins in the same vertical scan line can be processed in parallel.

Figure 4.  2-level diagonal-vertical scan for 16x16 TU

Figure 5.  Context formation pattern for 2-level diagonal-vertical scan

# Simulation results

The proposed methods are implemented on HM-5.0. All the results were simulated according to the common test conditions defined in JCTVC-G1200 [3]. Table 1 shows the results of the proposed context formation pattern in 2-level diagonal scan. The bit rate is increased by 0.2% in HE conditions and 0.1% in LC conditions. Table 2 shows the simulation results of the proposed 2-level diagonal-vertical scan with corresponding context formation pattern. The bit rate is increased by 0.2% in HE conditions and 0.1% in LC conditions. Both methods can allow 4-bin parallel context assignment. All run times are roughly unchanged.

# Conclusions

In this contribution, a new context formation patterns for significance map coding is first proposed to support 4-bin parallel context assignment. For each bin of significance flag, the data dependency of three previously decoded bins is removed. Simulation results reportedly show 0.1-0.2% bit rate increase and no run time change. To not only allow 4-bin parallel context assignment but also simplify the subblock coding process, a 2-level diagonal-vertical scan using vertical scan of coefficients within each subblock is proposed with a corresponding context formation pattern. It is reported that the bit rate increase is also 0.1-0.2% with unchanged run times.

# References

- J. Sole, R. Joshi, M. Karczewicz, “Non-CE11: Diagonal sub-block scan for HE residual coding,” Document of Joint Collaborative Team on Video Coding, JCTVC-G323, November 2011.

- J. Sole, “JCT-VC break-out report: Harmonization of NSQT with residual coding,” Document of Joint Collaborative Team on Video Coding, JCTVC-G1038, November 2011.

- F. Bossen, “Common HM test conditions and software reference configurations,” Document of Joint Collaborative Team on Video Coding, JCTVC-G1200, November 2011.

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Table 1.  Results of the proposed context formation pattern for 2-level diagonal scan [run times to be updated]

Table 2.  Results of the proposed context formation pattern for 2-level vertical scan [run times to be updated]