| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 8th Meeting: San José, CA, USA, 1–10 February, 2012 | Document: JCTVC-H0287 |
| --- | --- |

| Title: | Non-CE11: Modified method for two-level coding of significance maps |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Shih-Ta Hsiang, Tzu-Der Chuang, Shawmin Lei No. 1, Dusing Rd. 1, Hsinchu Science Park Hsinchu, Taiwan 30078 | Tel: Email: | Shawmin Lei +886-3-5670766 ext. 25555 {shih-ta.hsiang, peter.chuang, shawmin.lei}@mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

This contribution proposes a modified method to improve the efficiency for coding significance maps by further exploiting the new two-level diagonal sub-block scan structure. The proposed method attempts to further reduce the number of coded bins by additionally encoding the position of the last significant coefficient within a sub-block. Experimental results reportedly show Y BD-rate improvements 0.0%, 0.0 and 0.0% for HE-AI, HE-RA and HE-LDB, respectively, and 0.1%, 0.1 and 0.2% for LC-AI, LC-RA and LC-LDB, respectively, under the common test condition.

# Introduction

A method for two-level coding of significance maps of large transform units is adopted in the 7th HEVC meeting [1]. In this two-level coding method, a new syntax element significant_coeff_group_flag is coded for some sub-blocks to indicate if there is any significant coefficient within a given sub-block. In this way, significance flags for a large number of zero coefficients from the same sub-lock region can be compactly represented by coding one syntax element. Experimental results show this method can significantly reduce the number of coded bins, particularly in combination with the newly adopted diagonal sub-block scan method for coding the significance maps [2].

This contribution proposes a modified method to improve the efficiency for coding significance maps by further exploiting the new two-level diagonal sub-block scan structure.

# Proposed algorithm

The proposed method attempts to further reduce the number of coded bins by additionally encoding new syntax elements group_last_significant_coeff_x and group_last_significant_coeff_y for representing the position of the last significant coefficient within a significant sub-block (with significant_coeff_group_flag equal to 1). Coding significance flags can be skipped for all the coefficients after the last significant coefficient in the scan order within the sub-block. Similar to the idea in [1], we encode this additional information only for the sub-block expected to have a small number of significant coefficients. For the effective integration with current two-level coding method, syntax elements group_last_significant_coeff_x and group_last_significant_coeff_y are only coded for a sub-block with significant_coeff_group_flag explicitly coded and being equal to 1. That is, the proposed method skips coding the position of the last significant coefficient for the sub-block having both neighboring sub-blocks in the bottom and to the right being significant. Figure 1 illustrates the proposed method for coding significance maps. As we can see, coding significance flags for the zero coefficients in the yellow sub-blocks can be skipped using the proposed method.

The new syntax elements group_last_significant_coeff_x and group_last_significant_coeff_y are binarized by a truncated unary code for the BAC engine. Each bin is assigned a single context and three contexts are added for coding each proposed syntax element.

Figure 1: Illustration for the proposed method for coding significance maps.

# Experimental results

The proposed algorithm has been fully implemented using HM-5.0 as software base and experimentally evaluated according to the common test conditions defined in JCTVC-G1200 [3]. The experimental results are provided in the included spread sheets and summarized in Table 1.

Table 1: Summary of the BD-rate results according to the common test conditions

|  | All Intra HE |  |  | All Intra LC |  |  | All Intra HE-10 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A (8bit) | 0.0% | 0.0% | 0.1% | -0.1% | -0.2% | -0.2% |  |  |  |
| Class B | 0.0% | 0.0% | 0.1% | -0.2% | -0.4% | -0.4% |  |  |  |
| Class C | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% | -0.1% |  |  |  |
| Class D | 0.0% | 0.1% | 0.1% | -0.1% | -0.1% | -0.1% |  |  |  |
| Class E | 0.1% | 0.2% | 0.2% | -0.1% | -0.1% | -0.2% |  |  |  |
| Overall | 0.0% | 0.1% | 0.1% | -0.1% | -0.2% | -0.2% |  |  |  |
|  | 0.0% | 0.1% | 0.1% | -0.1% | -0.2% | -0.2% |  |  |  |
| Class F | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% | -0.1% |  |  |  |
| Enc Time[%] | 103.3% |  |  | 105.5% |  |  |  |  |  |
| Dec Time[%] | 103.5% |  |  | 104.7% |  |  |  |  |  |
|  | Random Access HE |  |  | Random Access LC |  |  | Random Access HE-10 |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A (8bit) | 0.0% | 0.0% | -0.1% | 0.0% | 0.0% | -0.3% | 0.0% | -0.1% | 0.5% |
| Class B | 0.0% | -0.1% | 0.0% | -0.1% | -0.2% | -0.3% | 0.0% | -0.1% | -0.1% |
| Class C | 0.0% | -0.1% | 0.1% | 0.0% | -0.1% | -0.1% |  |  |  |
| Class D | 0.0% | -0.2% | 0.0% | 0.0% | 0.3% | -0.2% |  |  |  |
| Class E |  |  |  |  |  |  |  |  |  |
| Overall | 0.0% | -0.1% | 0.0% | -0.1% | 0.0% | -0.2% | 0.0% | -0.1% | 0.2% |
|  | 0.0% | -0.1% | 0.0% | -0.1% | 0.0% | -0.2% | 0.0% | 0.0% | 0.2% |
| Class F | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% | 0.0% |  |  |  |
| Enc Time[%] | 100.9% |  |  | 104.5% |  |  | 102.0% |  |  |
| Dec Time[%] | 100.6% |  |  | 103.6% |  |  | 101.5% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  | Low delay B HE-10 |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |  |  |  |
| Class B | 0.1% | -0.1% | 0.0% | -0.3% | -0.4% | -0.1% |  |  |  |
| Class C | 0.0% | -0.1% | 0.1% | -0.1% | 0.0% | -0.2% |  |  |  |
| Class D | 0.0% | -0.4% | -0.2% | -0.1% | 0.9% | 0.2% |  |  |  |
| Class E | 0.1% | 0.3% | -0.4% | -0.2% | 0.0% | -0.2% |  |  |  |
| Overall | 0.0% | -0.1% | -0.1% | -0.2% | 0.1% | 0.0% |  |  |  |
|  | 0.0% | -0.1% | -0.1% | -0.2% | 0.1% | 0.0% |  |  |  |
| Class F | -0.1% | 0.0% | -0.6% | 0.0% | -0.5% | -0.3% |  |  |  |
| Enc Time[%] | 101.7% |  |  | 101.7% |  |  |  |  |  |
| Dec Time[%] | 101.7% |  |  | 100.7% |  |  |  |  |  |
|  | Low delay P HE |  |  | Low delay P LC |  |  | Low delay P HE-10 |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |  |  |  |
| Class B | 0.0% | 0.0% | 0.0% | -0.1% | 0.0% | -0.7% |  |  |  |
| Class C | 0.1% | 0.0% | -0.2% | -0.1% | 0.1% | 0.1% |  |  |  |
| Class D | 0.0% | 0.1% | -0.1% | -0.1% | 0.3% | 0.4% |  |  |  |
| Class E | -0.1% | 0.3% | -0.6% | -0.1% | -0.3% | -1.4% |  |  |  |
| Overall | 0.0% | 0.1% | -0.2% | -0.1% | 0.0% | -0.3% |  |  |  |
|  | 0.0% | 0.1% | -0.3% | -0.1% | 0.0% | -0.3% |  |  |  |
| Class F | 0.0% | 0.4% | 0.1% | 0.1% | 0.2% | 0.1% |  |  |  |
| Enc Time[%] | 100.7% |  |  | 104.1% |  |  |  |  |  |
| Dec Time[%] | 100.7% |  |  | 103.1% |  |  |  |  |  |

# References

- N. Nguyen, T. Ji, D. He, G. Martin-Cocher, L. Song “Multi-level significance maps for Large Transform Units,” Document of Joint Collaborative Team on Video Coding, JCTVC-G644, 7th Meeting: Geneva, CH, 21-30 November, 2011.

- J. Sole, R. Joshi, and M. Karczewicz, “Non-CE11: Diagonal sub-block scan for HE residual coding,” Document of Joint Collaborative Team on Video Coding, JCTVC-G323, 7th Meeting: Geneva, CH, 21-30 November, 2011.

- F. Bossen, “Common HM test conditions and software reference configurations,” Document of Joint Collaborative Team on Video Coding, Document of Joint Collaborative Team on Video Coding, JCTVC-G1200, 7th Meeting: Geneva, CH, 21-30 November, 2011.

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).