| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 8th Meeting: San José, CA, USA, 1–10 February, 2012 | Document: JCTVC-H0289-r1 |
| --- | --- |

| Title: | Non-CE4: Spatial QP prediction based on motion vector predictors (AMVP) |  |  |  |
| --- | --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |  |
| Purpose: | Information |  |  |  |
| Author(s) or Contact(s): | Pierre Andrivon, Philippe Bordes, Philippe Salmon (Technicolor) | Tel: | +33-2-99-27-38-07 pierre.andrivon@technicolor.com philippe.bordes@technicolor.com philippe.salmon@technicolor.com |  |
| Source: | Technicolor |  |  |  |

_____________________________

# Abstract

This contribution presents results of spatial QP prediction based on:

- Test 1: motion vector predictor cues (in Inter),

- Test 2: a combination of test 1 and an intra direction based method (JCTVC-G1204 Subtest 1.3),

- Test 3: a non mode dependant QP prediction (above and left QGCU averaging if within LCU otherwise last coded QP).

Simulations results show on average 0.2% to 0.5% BD-rate gains for test 1 and test 2 in RA, LB and AI cases respectively under the test condition defined in CE4 Subtest 1 (AVC like).

Compared to the non mode dependant QP prediction (test 3), gains are equivalent.

# Introduction

CE4: Subtest 1 (JCTVC-G1204) relates activities around QP prediction. JCTVC-G1028 and CE4: Subtest 1.3 namely describe a spatial QP prediction based on intra directions. This contribution proposes to study spatial QP prediction based on motion vector predictors of inter mode when available. Besides, in order to complete the intra mode proposition (CE4 Subtest 1.3) a combination with the inter mode AMVP-based method is proposed.

# Algorithms description

CU, PU and QGCUs employed in these methods are illustrated in Figure 1. Each QGCU which is outside of the LCU is regarded as unavailable, as shown in Figure 1.3.2 of JCTVC-G1204-r3.

## Test 1: Spatial QP prediction based on AMVP (inter prediction)

This method is inspired by CE4: Subtest 1.3 using intra mode directions to retrieve spatial QP predictor(s) of the current QGCU. The principle of the algorithm is to predict the QP from the same QGCU as above and/or left motion vector predictors are derived from, if they exist. Indeed, considering spatial QP prediction based on inter mode, only A1 and B1 QPs (within the LCU, Cf. Figure 1) are considered as QP predictors if their respective motion vector predictors are available. Brief algorithm of the proposed method is described as follows.

- if the top-left-most CU in the current QGCU is inter-coded and skip_flag = 0 for the current CU and merge_flag = 0  (as described in WD*) for the current TLPU, the predicted QP is derived depending on the availability of motion vector predictors of N1 and M1 relative to the TLPU as follows (with N, M being replaced by A or B) (Figure 1):

If (availableFlagL0N && mvpL0 == mvL0N1 && availableFlagL1M && mvpL1== mvL1M1) then predicted QP is calculated by averaging QPs of QGCU N1 and QGCU M1 (as depicted in 8.4.2.1.7 of WD*).

Otherwise if (availableFlagL0N && mvpL0 == mvL0N1 && !availableFlagL1M) then predicted QP is the QP of the QGCU N1 belongs to, where mvL0N1 = mvL0[ xN1 ][ yN1 ] (as depicted in 8.4.2.1.7 of WD*).

Otherwise if (availableFlagL1N && mvpL1 == mvL1N1 && !availableFlagL0M) then predicted QP is the QP of the QGCU N1 belongs to, where mvL1N1 = mvL1[ xN1 ][ yN1 ] (as depicted in 8.4.2.1.7 of WD*).

- Otherwise if at least either of the above CU or the left CU is available within the LCU, predicted QP is calculated by averaging QPs of those CUs available.

- Otherwise, QP of the preceding QGCU in scan order is employed as the predicted QP.

## Test 2: Combination of CE4 Subtest 1.3 and AMVP-based spatial QP prediction

The proposed method combines CE4 Subtest 1.3 (JCTVC-G1204) method with the inter method described just here above. Brief algorithm of the proposed method is described as follows.

- If the top-left-most CU (TLCU) in the current QGCU is intra-coded, the predicted QP is derived along intra prediction direction of the top-left-most PU (TLPU) with the JCTVC-G357 method, as shown in Figure 1.3.3. of JCTVC-G1204.

- Otherwise if the top-left-most CU in the current QGCU is inter-coded and skip_flag = 0 for the current CU and merge_flag = 0  (as described in WD*) for the current TLPU, the predicted QP is derived depending on the availability of motion vector predictors of N1 and M1 relative to the TLPU as follows (with N, M being replaced by A or B) (Figure 1):

If (availableFlagL0N && mvpL0 == mvL0N1 && availableFlagL1M && mvpL1== mvL1M1) then predicted QP is calculated by averaging QPs of QGCU N1 and QGCU M1 (as depicted in 8.4.2.1.7 of WD*).

Otherwise if (availableFlagL0N && mvpL0 == mvL0N1 && !availableFlagL1M) then predicted QP is the QP of the QGCU N1 belongs to, where mvL0N1 = mvL0[ xN1 ][ yN1 ] (as depicted in 8.4.2.1.7 of WD*).

Otherwise if (availableFlagL1N && mvpL1 == mvL1N1 && !availableFlagL0M) then predicted QP is the QP of the QGCU N1 belongs to, where mvL1N1 = mvL1[ xN1 ][ yN1 ] (as depicted in 8.4.2.1.7 of WD*).

- Otherwise if at least either of the above CU or the left CU is available within the LCU, predicted QP is calculated by averaging QPs of those CUs available.

- Otherwise, QP of the preceding QGCU in scan order is employed as the predicted QP.

Remark:

WD: Working Draft 5 of High-Efficiency Video Coding, JCTVC-G1103 d2

## Test 3: Spatial QP prediction without modes considerations

This method proposes to remove previously described inter (left and above AMVP-based) and intra (left and above direction based) based methods to derive spatial QP predictor.

- If at least either of the above QGCU or the left QGCU is available within the LCU, predicted QP is calculated by averaging QPs of those QGCUs available.

- Otherwise, QP of the preceding QGCU in scan order is employed as the predicted QP.

Figure 1: CU, PU and neighboring QGCUs potentially employed for spatial QP prediction.

Remarks:

QGCU: Quantization Group of Coding Units

- QPL: QP of the left neighboring QGCU

- QPA: QP of the above neighboring QGCU

- QPAL: QP of the above-left neighboring QGCU

- TLPU: Top-Left-Most Prediction Unit

# Experiments

## Setup

Same mandatory test conditions as related in CE4 Subtest 1 (JCTV-G1204) are used. Namely, TM-5 step 3, which has already been implemented on HM5.0 as an encoding method, shall be used for QP adaptation method with modulation by ±12. This will be enabled by command option “-aq 1 –aqr 12”. MaxCUDepthDQP = 3 (mandatory). Anchor is Subtest 1.1 (AVC-style QP prediction, last coded QP).

The proposed QP prediction algorithms have been implemented on top of CE4 Subtest 1 software.

The source code has been run on a cluster grid with the following characteristics:

- OS: Linux (RedHat 5.5)

- CPU: 2 x Intel(R) Xeon(R) E5680 @ 3.33GHz, 6 cores.

- RAM: 96 GB per 6 cores.

## Results

Results and their analysis with respect to algorithms description are exposed hereunder. Complete results are given in attached Excel file sheets.

### Test 1: Spatial QP prediction based on AMVP

Considering GoPs including P or B frames (RA and LD),  states BD-rate and dQP incr. average gains for the proposed method compared with AVC-style method (CE4 Subtest 1.1). Those gains are less than those provided by CE4 Subtest 1.3 (Figure 3). Then, both intra and inter based methods are combined to test if results can be cumulative. If so, note that it would infer that results of inter or intra methods perform better than rest of the algorithm (Cf. Test 3).

|  | All Intra HE |  |  |  | All Intra LC |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | dQP incr. | Y | U | V | dQP incr. |
| Class A (8bit) | -0,2% | -0,2% | -0,2% | -4,1% | -0,1% | -0,1% | -0,2% | -3,9% |
| Class B | -0,3% | -0,3% | -0,3% | -7,2% | -0,2% | -0,2% | -0,2% | -7,3% |
| Class C | -0,4% | -0,4% | -0,4% | -9,6% | -0,4% | -0,4% | -0,4% | -9,8% |
| Class D | -0,4% | -0,4% | -0,4% | -10,2% | -0,4% | -0,3% | -0,3% | -10,3% |
| Class E | -0,4% | -0,4% | -0,4% | -6,7% | -0,4% | -0,4% | -0,4% | -7,8% |
| Overall | -0,4% | -0,3% | -0,3% | -8,0% | -0,3% | -0,3% | -0,3% | -8,2% |
|  | -0,4% | -0,3% | -0,3% |  | -0,3% | -0,3% | -0,3% |  |
| Class F | -0,1% | -0,1% | -0,2% | -4,5% | -0,1% | -0,1% | -0,1% | -4,5% |
| Enc Time[%] | 100% |  |  |  | 101% |  |  |  |
| Dec Time[%] | 99% |  |  |  | 99% |  |  |  |
|  | Random Access HE |  |  |  | Random Access LC |  |  |  |
|  | Y | U | V | dQP incr. | Y | U | V | dQP incr. |
| Class A (8bit) | -0,2% | -0,2% | 0,0% | -3,0% | -0,3% | 0,0% | -0,2% | -2,8% |
| Class B | -0,2% | -0,1% | -0,2% | -4,5% | -0,2% | -0,1% | -0,2% | -4,2% |
| Class C | -0,3% | -0,1% | -0,4% | -6,5% | -0,3% | -0,2% | 0,0% | -6,3% |
| Class D | -0,3% | -0,1% | -0,4% | -6,4% | -0,3% | -0,3% | -0,6% | -6,3% |
| Class E |  |  |  |  |  |  |  |  |
| Overall | -0,2% | -0,1% | -0,3% | -5,3% | -0,3% | -0,2% | -0,2% | -5,1% |
|  | -0,2% | -0,1% | -0,2% |  | -0,3% | -0,1% | -0,2% |  |
| Class F | 0,0% | 0,0% | -0,1% | -3,4% | -0,2% | -0,1% | 0,0% | -3,3% |
| Enc Time[%] | 100% |  |  |  | 100% |  |  |  |
| Dec Time[%] | 98% |  |  |  | 100% |  |  |  |
|  | Low delay B HE |  |  |  | Low delay B LC |  |  |  |
|  | Y | U | V | dQP incr. | Y | U | V | dQP incr. |
| Class A |  |  |  |  |  |  |  |  |
| Class B | -0,2% | -0,1% | 0,0% | -3,4% | -0,2% | -0,2% | -0,7% | -2,6% |
| Class C | -0,3% | -0,2% | 0,0% | -5,2% | -0,3% | -0,3% | 0,1% | -4,8% |
| Class D | -0,2% | -0,6% | 0,1% | -5,0% | -0,3% | -0,8% | -0,5% | -4,8% |
| Class E | -0,2% | -1,2% | -0,8% | -2,4% | -0,2% | 0,3% | -0,4% | -3,1% |
| Overall | -0,2% | -0,4% | -0,1% | -4,1% | -0,2% | -0,3% | -0,4% | -3,8% |
|  | -0,2% | -0,4% | -0,2% |  | -0,2% | -0,3% | -0,3% |  |
| Class F | -0,1% | 0,0% | 0,1% | -2,3% | 0,1% | -0,3% | 0,3% | -1,5% |
| Enc Time[%] | 100% |  |  |  | 100% |  |  |  |
| Dec Time[%] | 103% |  |  |  | 102% |  |  |  |

Figure 2: Spatial QP prediction based on AMVP compared to CE4-subtest 1.1.

|  | All Intra HE |  |  |  | All Intra LC |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | dQP incr. | Y | U | V | dQP incr. |
| Class A (8bit) | -0,4% | -0,4% | -0,4% | -8,1% | -0,3% | -0,3% | -0,3% | -8,4% |
| Class B | -0,4% | -0,4% | -0,4% | -10,7% | -0,3% | -0,3% | -0,3% | -11,1% |
| Class C | -0,6% | -0,5% | -0,5% | -12,5% | -0,5% | -0,5% | -0,5% | -12,9% |
| Class D | -0,5% | -0,4% | -0,5% | -12,4% | -0,4% | -0,4% | -0,4% | -12,6% |
| Class E | -0,7% | -0,7% | -0,7% | -12,4% | -0,8% | -0,7% | -0,8% | -14,5% |
| Overall | -0,5% | -0,5% | -0,5% | -11,5% | -0,5% | -0,4% | -0,5% | -12,1% |
|  | -0,5% | -0,5% | -0,5% |  | -0,5% | -0,4% | -0,4% |  |
| Class F | -0,2% | -0,4% | -0,3% | -8,2% | -0,2% | -0,3% | -0,3% | -9,2% |
| Enc Time[%] | 100% |  |  |  | 101% |  |  |  |
| Dec Time[%] | 100% |  |  |  | 100% |  |  |  |
|  | Random Access HE |  |  |  | Random Access LC |  |  |  |
|  | Y | U | V | dQP incr. | Y | U | V | dQP incr. |
| Class A (8bit) | -0,2% | -0,1% | -0,3% | -4,9% | -0,4% | -0,2% | -0,2% | -5,0% |
| Class B | -0,3% | -0,2% | -0,2% | -6,1% | -0,3% | -0,1% | 0,0% | -5,8% |
| Class C | -0,4% | -0,1% | -0,4% | -7,9% | -0,4% | -0,1% | -0,2% | -7,7% |
| Class D | -0,3% | -0,6% | -0,5% | -7,7% | -0,4% | -0,6% | -0,3% | -7,6% |
| Class E |  |  |  |  |  |  |  |  |
| Overall | -0,3% | -0,3% | -0,4% | -6,9% | -0,3% | -0,2% | -0,2% | -6,7% |
|  | -0,3% | -0,3% | -0,3% |  | -0,3% | -0,2% | -0,2% |  |
| Class F | -0,2% | -0,3% | -0,3% | -5,9% | -0,3% | -0,4% | -0,2% | -6,3% |
| Enc Time[%] | 100% |  |  |  | 100% |  |  |  |
| Dec Time[%] | 98% |  |  |  | 97% |  |  |  |
|  | Low delay B HE |  |  |  | Low delay B LC |  |  |  |
|  | Y | U | V | dQP incr. | Y | U | V | dQP incr. |
| Class A |  |  |  |  |  |  |  |  |
| Class B | -0,2% | -0,2% | -0,2% | -3,9% | -0,2% | 0,0% | -0,3% | -3,2% |
| Class C | -0,3% | -0,4% | -0,3% | -5,9% | -0,4% | -0,4% | -0,1% | -5,6% |
| Class D | -0,3% | -0,7% | 0,3% | -5,7% | -0,3% | -0,6% | -1,0% | -5,5% |
| Class E | -0,1% | -0,4% | -0,3% | -3,0% | -0,3% | 0,2% | -1,3% | -4,1% |
| Overall | -0,2% | -0,4% | -0,1% | -4,7% | -0,3% | -0,2% | -0,6% | -4,5% |
|  | -0,2% | -0,4% | -0,1% |  | -0,3% | -0,3% | -0,6% |  |
| Class F | -0,2% | -0,2% | -0,2% | -3,4% | -0,1% | 0,9% | 0,5% | -3,2% |
| Enc Time[%] | 100% |  |  |  | 100% |  |  |  |
| Dec Time[%] | 98% |  |  |  | 98% |  |  |  |

Figure 3: Subtest 1.3 (Spatial QP prediction Intra) compared to CE4 Subtest 1.1

(courtesy from JVC Kenwood, JCTVC-H0205).

### Test 2: Combination of intra direction (CE4 Subtest 1.3) and proposed inter based methods for spatial QP prediction

Figure 3 and Figure 4 comparison states that gains are equivalent for combined method than for Subtest 1.3. Then, next method (test 3) proposes to break down Subtest 1.3 algorithm to identify where its gains come from.

|  | All Intra HE |  |  |  | All Intra LC |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | dQP incr. | Y | U | V | dQP incr. |
| Class A (8bit) | -0,4% | -0,4% | -0,4% | -8,1% | -0,3% | -0,3% | -0,3% | -8,4% |
| Class B | -0,4% | -0,4% | -0,4% | -10,7% | -0,3% | -0,3% | -0,3% | -11,1% |
| Class C | -0,6% | -0,5% | -0,5% | -12,5% | -0,5% | -0,5% | -0,5% | -12,9% |
| Class D | -0,5% | -0,4% | -0,5% | -12,4% | -0,4% | -0,4% | -0,4% | -12,6% |
| Class E | -0,7% | -0,8% | -0,7% | -12,4% | -0,8% | -0,7% | -0,8% | -14,5% |
| Overall | -0,5% | -0,5% | -0,5% | -11,5% | -0,5% | -0,4% | -0,5% | -12,1% |
|  | -0,5% | -0,5% | -0,5% |  | -0,5% | -0,4% | -0,4% |  |
| Class F | -0,2% | -0,4% | -0,3% | -8,2% | -0,2% | -0,3% | -0,3% | -9,2% |
| Enc Time[%] | 100% |  |  |  | 100% |  |  |  |
| Dec Time[%] | 100% |  |  |  | 100% |  |  |  |
|  | Random Access HE |  |  |  | Random Access LC |  |  |  |
|  | Y | U | V | dQP incr. | Y | U | V | dQP incr. |
| Class A (8bit) | -0,2% | -0,1% | -0,2% | -4,6% | -0,4% | -0,2% | -0,3% | -4,7% |
| Class B | -0,3% | -0,2% | -0,2% | -5,8% | -0,3% | -0,1% | -0,1% | -5,6% |
| Class C | -0,4% | -0,2% | -0,3% | -7,7% | -0,3% | -0,1% | -0,2% | -7,4% |
| Class D | -0,3% | -0,7% | -0,5% | -7,3% | -0,3% | -0,6% | -0,3% | -7,1% |
| Class E |  |  |  |  |  |  |  |  |
| Overall | -0,3% | -0,3% | -0,3% | -6,5% | -0,3% | -0,2% | -0,2% | -6,4% |
|  | -0,3% | -0,3% | -0,3% |  | -0,3% | -0,2% | -0,2% |  |
| Class F | -0,2% | -0,3% | -0,3% | -5,9% | -0,3% | -0,4% | -0,2% | -6,2% |
| Enc Time[%] | 100% |  |  |  | 100% |  |  |  |
| Dec Time[%] | 100% |  |  |  | 100% |  |  |  |
|  | Low delay B HE |  |  |  | Low delay B LC |  |  |  |
|  | Y | U | V | dQP incr. | Y | U | V | dQP incr. |
| Class A |  |  |  |  |  |  |  |  |
| Class B | -0,2% | -0,1% | -0,1% | -3,7% | -0,2% | -0,2% | -0,2% | -3,0% |
| Class C | -0,3% | -0,4% | -0,3% | -5,6% | -0,3% | -0,5% | 0,1% | -5,2% |
| Class D | -0,3% | -1,1% | 0,3% | -5,1% | -0,3% | -0,2% | 0,0% | -5,0% |
| Class E | -0,1% | -0,9% | -0,3% | -2,8% | -0,3% | -0,3% | -1,5% | -3,7% |
| Overall | -0,2% | -0,6% | -0,1% | -4,3% | -0,3% | -0,3% | -0,3% | -4,2% |
|  | -0,2% | -0,6% | -0,1% |  | -0,3% | -0,3% | -0,4% |  |
| Class F | -0,1% | 0,3% | 0,1% | -3,0% | 0,0% | 0,3% | 0,6% | -3,2% |
| Enc Time[%] | 100% |  |  |  | 100% |  |  |  |
| Dec Time[%] | 100% |  |  |  | 100% |  |  |  |

Figure 4: Combination of Spatial QP prediction intra directions (above and left) and inter (AMVP-based, above and left) compared to CE4 subtest 1.1.

### Test 3: Spatial QP prediction without mode considerations

Figure 3 and Figure 5 comparison states that gains of the non-intra-based related part of subtest 1.3 algorithm are equivalent to intra or inter mode dependant algorithms.

|  | All Intra HE |  |  |  | All Intra LC |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | dQP incr. | Y | U | V | dQP incr. |
| Class A (8bit) | -0,2% | -0,2% | -0,2% | -4,1% | -0,1% | -0,1% | -0,2% | -3,9% |
| Class B | -0,3% | -0,3% | -0,3% | -7,2% | -0,2% | -0,2% | -0,2% | -7,3% |
| Class C | -0,4% | -0,4% | -0,4% | -9,6% | -0,4% | -0,4% | -0,4% | -9,8% |
| Class D | -0,4% | -0,4% | -0,4% | -10,2% | -0,4% | -0,3% | -0,3% | -10,3% |
| Class E | -0,4% | -0,4% | -0,4% | -6,7% | -0,4% | -0,4% | -0,4% | -7,8% |
| Overall | -0,4% | -0,3% | -0,3% | -8,0% | -0,3% | -0,3% | -0,3% | -8,2% |
|  | -0,4% | -0,3% | -0,3% |  | -0,3% | -0,3% | -0,3% |  |
| Class F | -0,1% | -0,1% | -0,2% | -4,5% | -0,1% | -0,1% | -0,1% | -4,5% |
| Enc Time[%] | 100% |  |  |  | 102% |  |  |  |
| Dec Time[%] | 99% |  |  |  | 100% |  |  |  |
|  | Random Access HE |  |  |  | Random Access LC |  |  |  |
|  | Y | U | V | dQP incr. | Y | U | V | dQP incr. |
| Class A (8bit) | -0,2% | -0,3% | -0,1% | -3,2% | -0,3% | 0,0% | -0,3% | -3,1% |
| Class B | -0,2% | -0,2% | -0,2% | -4,7% | -0,2% | 0,0% | -0,2% | -4,4% |
| Class C | -0,3% | -0,1% | -0,4% | -6,8% | -0,3% | -0,2% | -0,1% | -6,6% |
| Class D | -0,3% | -0,3% | -0,4% | -6,8% | -0,3% | -0,3% | -0,6% | -6,8% |
| Class E |  |  |  |  |  |  |  |  |
| Overall | -0,3% | -0,2% | -0,3% | -5,6% | -0,3% | -0,1% | -0,3% | -5,4% |
|  | -0,3% | -0,2% | -0,2% |  | -0,3% | -0,1% | -0,3% |  |
| Class F | 0,0% | -0,1% | -0,1% | -3,6% | -0,2% | -0,2% | 0,0% | -3,4% |
| Enc Time[%] | 100% |  |  |  | 100% |  |  |  |
| Dec Time[%] | 101% |  |  |  | 103% |  |  |  |
|  | Low delay B HE |  |  |  | Low delay B LC |  |  |  |
|  | Y | U | V | dQP incr. | Y | U | V | dQP incr. |
| Class A |  |  |  |  |  |  |  |  |
| Class B | -0,2% | -0,1% | 0,3% | -3,7% | -0,2% | -0,4% | -0,5% | -2,9% |
| Class C | -0,3% | -0,3% | -0,1% | -5,5% | -0,3% | -0,4% | 0,2% | -5,2% |
| Class D | -0,3% | -0,2% | 0,3% | -5,5% | -0,3% | -0,8% | -0,3% | -5,4% |
| Class E | -0,2% | -0,7% | -1,2% | -2,8% | -0,2% | 1,1% | -0,5% | -3,5% |
| Overall | -0,2% | -0,3% | -0,1% | -4,4% | -0,3% | -0,2% | -0,3% | -4,2% |
|  | -0,2% | -0,3% | -0,1% |  | -0,3% | -0,3% | -0,2% |  |
| Class F | -0,2% | -0,3% | -0,2% | -2,4% | 0,1% | 1,2% | 1,2% | -1,9% |
| Enc Time[%] | 100% |  |  |  | 100% |  |  |  |
| Dec Time[%] | 101% |  |  |  | 102% |  |  |  |

Figure 5: Spatial QP prediction without modes considerations (above and left QGCU averaging if within LCU otherwise last coded QP) compared to CE4 subtest 1.1.

# Conclusions

This contribution presents results of spatial QP prediction based on motion vector predictor cues (test 1) and a combination of this latter and an intra direction based method (JCTVC-G1204 Subtest 1.3) (test 2). Simulation results show on average 0.2% to 0.3%, BD-rate gains for test 1 and test 2 in RA, LB and AI cases respectively under the test condition defined in CE4 Subtest 1 (AVC like).

Compared to a non mode dependant QP prediction (above and left QGCU averaging if within LCU otherwise last coded QP) (test 3), gains are equivalent.

# References

[1] “Description of Core Experiment 4: Quantization”, JCTVC-G1204, Nov. 2011

[2] “Non-CE4: Rate control friendly spatial QP prediction”, JCTVC-G1028, Nov. 2011

[3] “Working Draft 5 of High-Efficiency Video Coding”, JCTVC-G1103, Nov. 2011

# Patent rights declaration(s)

Technicolor may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).