| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 8th Meeting: San José, CA, USA, 1–10 February, 2012 | Document: JCTVC-H0290 WG11 Number: m23165 |
| --- | --- |

| Title: | Non-CE11:Simplification of context selection for significant_coeff_flag |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Kengo Terada Hisao Sasai Takahiro Nishi 1006 Kadoma, Kadoma Osaka, Japan | Tel: Email: | +81 6 6900 9689 { terada.kengo, sasai.hisao, nishi.takahiro }@jp.panasonic.com |
| Source: | Panasonic Corporation |  |  |

_____________________________

# Abstract

In HEVC WD5.0, transformed coefficients are scanned in 4x4 sub-block basis, and CABAC context is adaptively selected triangle shaped region basis in 16x16 and 32x32 TU for luminance component. This contribution proposes to use 4x4 sub-block based CABAC context selection for simplification, which is aligned with the transformed coefficients scanning unit. Two different patterns of regions for the context selection were evaluated on top of HM5.0. Experimental result reportedly showed that proposed modification can be introduced into the HM5.0 with less than 0.1% BD-rate loss.

# Introduction

In HEVC WD5.0, three regions are defined for the CABAC context selection of significant_coeff_flag coding in 16x16 and 32x32 TU for luminance component as shown in Fig.1. significant_coeff_flag is scanned in 4x4 sub-block basis as shown in Fig.2 [1]. If these processing units were aligned, some of decision branches in the CABAC context selection process can be removed.

Fig.1 : Regions for significant_coeff_flag context selection in HM5.0

Fig.2 : significant_coeff_flag coding order in HM5.0

# Proposed Solution

## 4x4 sub-block based CABAC context selection (Pattern 1)

In this proposal, region1/2 boundary is changed to 4x4 sub-block boundary as shown in Fig.3. With this modification, the CABAC context can be decided based on the sub-block position, and thus coefficient position based decision is not necessary anymore, except for DC coefficient.

Fig.3 : Regions for significant_coeff_flag context selection in Pattern1

## Unified context selection for 16x16 and 32x32 TUs (Pattern 2)

In addition to pattern 1, a unified one for 16x16 and 32x32 TUs is proposed as shown in Fig.4. Some decision branches can be further reduced.

Fig.4 : Regions for significant_coeff_flag context selection in Pattern2

# Results

The proposed method was evaluated under the common test condition described in JCTVC-G1200 [2]. It was also checked under the CE11 low QP (12, 17, 22, 27) test condition described in JCTVC-G1211 [3]. The computing platform is Linux 64bits and the executables were compiled on 64-bit Linux with gcc 4.4.3.

Table 1 : BD BR luminance for Pattern1 and Pattern2

Table 2 : BD BR for Pattern1 ( Common Conditions )

|  | All Intra HE |  |  | All Intra LC |  |  | All Intra HE-10 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A (8bit) | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% |  |  |  |
| Class B | 0.0% | 0.1% | 0.1% | 0.0% | 0.1% | 0.1% |  |  |  |
| Class C | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| Class D | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| Class E | 0.0% | 0.1% | 0.1% | 0.0% | 0.1% | 0.1% |  |  |  |
| Overall | 0.0% | 0.1% | 0.1% | 0.0% | 0.1% | 0.0% |  |  |  |
|  | 0.0% | 0.1% | 0.1% | 0.0% | 0.1% | 0.0% |  |  |  |
| Class F | 0.0% | -0.1% | 0.0% | 0.0% | 0.1% | 0.0% |  |  |  |
| Enc Time[%] | 99% |  |  | 99% |  |  |  |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |  |  |  |
|  | Random Access HE |  |  | Random Access LC |  |  | Random Access HE-10 |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A (8bit) | 0.0% | 0.0% | -0.1% | 0.0% | 0.1% | -0.1% | 0.0% | 0.2% | 0.8% |
| Class B | 0.0% | 0.0% | 0.0% | 0.1% | 0.1% | 0.1% | 0.1% | 0.0% | 0.0% |
| Class C | 0.0% | -0.1% | 0.0% | 0.0% | 0.0% | 0.1% |  |  |  |
| Class D | 0.0% | -0.2% | 0.0% | 0.0% | 0.4% | -0.3% |  |  |  |
| Class E |  |  |  |  |  |  |  |  |  |
| Overall | 0.0% | -0.1% | 0.0% | 0.0% | 0.2% | 0.0% | 0.1% | 0.1% | 0.3% |
|  | 0.0% | -0.1% | 0.0% | 0.0% | 0.2% | -0.1% | 0.2% | 0.1% | 0.4% |
| Class F | -0.1% | -0.1% | -0.1% | 0.0% | 0.0% | -0.1% |  |  |  |
| Enc Time[%] | 100% |  |  | 100% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  | 100% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  | Low delay B HE-10 |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |  |  |  |
| Class B | 0.0% | 0.2% | -0.1% | 0.0% | -0.2% | 0.4% |  |  |  |
| Class C | 0.0% | 0.1% | 0.2% | 0.0% | 0.3% | 0.1% |  |  |  |
| Class D | 0.0% | -0.3% | 0.2% | 0.0% | 0.7% | -0.3% |  |  |  |
| Class E | 0.0% | 0.5% | -0.9% | 0.0% | -0.9% | 0.2% |  |  |  |
| Overall | 0.0% | 0.1% | -0.1% | 0.0% | 0.0% | 0.1% |  |  |  |
|  | 0.0% | 0.1% | -0.1% | 0.0% | 0.0% | 0.2% |  |  |  |
| Class F | -0.2% | 0.0% | -0.3% | 0.1% | -0.7% | -0.3% |  |  |  |
| Enc Time[%] | 100% |  |  | 100% |  |  |  |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |  |  |  |

Table 3 : BD BR for Pattern1 ( Low QP (12, 17, 22, 27) )

|  | All Intra HE |  |  | All Intra LC |  |  | All Intra HE-10 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A (8bit) | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| Class C | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| Class D | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| Class E | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% |  |  |  |
| Overall | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
|  | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| Class F | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| Enc Time[%] | 99% |  |  | 99% |  |  |  |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |  |  |  |
|  | Random Access HE |  |  | Random Access LC |  |  | Random Access HE-10 |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A (8bit) | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% | 0.0% | 0.0% | 0.1% |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class C | 0.0% | 0.1% | 0.1% | 0.0% | 0.0% | 0.0% |  |  |  |
| Class D | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| Class E |  |  |  |  |  |  |  |  |  |
| Overall | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% |
|  | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% |
| Class F | -0.1% | 0.0% | -0.1% | 0.1% | 0.0% | 0.0% |  |  |  |
| Enc Time[%] | 99% |  |  | 100% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  | 100% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  | Low delay B HE-10 |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |  |  |  |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| Class C | 0.0% | 0.0% | 0.1% | 0.0% | 0.1% | 0.0% |  |  |  |
| Class D | 0.0% | -0.1% | 0.0% | 0.0% | 0.1% | -0.1% |  |  |  |
| Class E | 0.0% | 0.2% | 0.0% | 0.0% | 0.0% | -0.2% |  |  |  |
| Overall | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% |  |  |  |
|  | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% |  |  |  |
| Class F | 0.0% | -0.1% | -0.1% | 0.2% | 0.3% | 0.0% |  |  |  |
| Enc Time[%] | 100% |  |  | 100% |  |  |  |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |  |  |  |

Table 4 : BD BR for Pattern2 ( Common Conditions )

|  | All Intra HE |  |  | All Intra LC |  |  | All Intra HE-10 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A (8bit) | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% |  |  |  |
| Class B | 0.1% | 0.1% | 0.1% | 0.1% | 0.1% | 0.1% |  |  |  |
| Class C | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| Class D | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| Class E | 0.0% | 0.1% | 0.2% | 0.1% | 0.1% | 0.1% |  |  |  |
| Overall | 0.0% | 0.1% | 0.1% | 0.0% | 0.1% | 0.1% |  |  |  |
|  | 0.0% | 0.1% | 0.1% | 0.0% | 0.1% | 0.1% |  |  |  |
| Class F | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% |  |  |  |
| Enc Time[%] | 99% |  |  | 99% |  |  |  |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |  |  |  |
|  | Random Access HE |  |  | Random Access LC |  |  | Random Access HE-10 |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A (8bit) | 0.0% | 0.0% | -0.2% | 0.0% | 0.1% | 0.0% | 0.1% | -0.3% | -0.1% |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% | 0.1% | 0.1% | 0.0% | 0.0% |
| Class C | 0.0% | -0.1% | -0.1% | 0.0% | 0.2% | 0.1% |  |  |  |
| Class D | 0.1% | -0.1% | 0.0% | 0.0% | 0.5% | -0.3% |  |  |  |
| Class E |  |  |  |  |  |  |  |  |  |
| Overall | 0.0% | -0.1% | 0.0% | 0.0% | 0.2% | 0.0% | 0.1% | -0.1% | 0.0% |
|  | 0.0% | -0.1% | 0.0% | 0.0% | 0.2% | 0.0% | 0.1% | -0.1% | 0.0% |
| Class F | 0.0% | -0.1% | 0.0% | 0.0% | -0.1% | -0.1% |  |  |  |
| Enc Time[%] | 100% |  |  | 100% |  |  | 100% |  |  |
| Dec Time[%] | 99% |  |  | 100% |  |  | 99% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  | Low delay B HE-10 |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |  |  |  |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% | 0.1% |  |  |  |
| Class C | 0.0% | -0.2% | 0.2% | 0.0% | 0.3% | -0.1% |  |  |  |
| Class D | 0.1% | -0.4% | -0.1% | 0.0% | 0.7% | 0.2% |  |  |  |
| Class E | 0.1% | 0.2% | -0.5% | 0.0% | -0.6% | 0.2% |  |  |  |
| Overall | 0.0% | -0.1% | -0.1% | 0.0% | 0.1% | 0.1% |  |  |  |
|  | 0.0% | -0.1% | -0.2% | 0.0% | 0.1% | 0.1% |  |  |  |
| Class F | -0.1% | 0.1% | -0.2% | 0.0% | -1.0% | -0.2% |  |  |  |
| Enc Time[%] | 100% |  |  | 100% |  |  |  |  |  |
| Dec Time[%] | 99% |  |  | 99% |  |  |  |  |  |

Table 5 : BD BR for Pattern2 ( Low QP (12, 17, 22, 27) )

|  | All Intra HE |  |  | All Intra LC |  |  | All Intra HE-10 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A (8bit) | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% | 0.1% |  |  |  |
| Class C | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| Class D | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| Class E | 0.0% | 0.0% | 0.1% | 0.0% | 0.1% | 0.1% |  |  |  |
| Overall | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
|  | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| Class F | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| Enc Time[%] | 99% |  |  | 99% |  |  |  |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |  |  |  |
|  | Random Access HE |  |  | Random Access LC |  |  | Random Access HE-10 |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A (8bit) | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% | -0.1% | -0.1% |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% |
| Class C | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| Class D | 0.0% | -0.1% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| Class E |  |  |  |  |  |  |  |  |  |
| Overall | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% |
|  | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class F | -0.1% | -0.1% | 0.0% | 0.0% | -0.1% | 0.0% |  |  |  |
| Enc Time[%] | 99% |  |  | 100% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  | 100% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  | Low delay B HE-10 |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |  |  |  |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| Class C | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% |  |  |  |
| Class D | 0.0% | -0.1% | -0.1% | 0.0% | 0.1% | -0.1% |  |  |  |
| Class E | 0.0% | 0.0% | -0.1% | 0.0% | 0.0% | 0.4% |  |  |  |
| Overall | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% |  |  |  |
|  | 0.0% | 0.0% | -0.1% | 0.0% | 0.1% | 0.1% |  |  |  |
| Class F | 0.0% | 0.0% | 0.0% | 0.1% | 0.2% | 0.0% |  |  |  |
| Enc Time[%] | 99% |  |  | 100% |  |  |  |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |  |  |  |

# Conclusion

In this contribution, 4x4 sub-block based CABAC context selection for significant_coeff_flag is proposed. Proposed processing unit, which is aligned with transformed coefficients scanning, can simplify the CABAC context decision. It was confirmed that proposed two patterns of regions for the context selection can be introduced into the HM5.0 without significant impact on coding efficiency (in range of 0.1%). It is suggested to consider the inclusion of this proposal in the HM/CD of HEVC.

# References

- J.Sole, “Diagonal sub-block scan for HE residual coding”, JCTVC-G323, JCT-VC 7th Meeting, Geneva, November, 2011.

- Frank Bossen, “Common test conditions and software reference configurations”, JCTVC-G1200, JCT-VC 7th Meeting, Geneva, November, 2011.

- V.Sze, “Description of Core Experiment (CE11): Coefficient scanning and coding”, JCTVC-G1211, JCT-VC 7th Meeting, Geneva, November, 2011.

# Patent rights declaration(s)

Panasonic Corporation may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Proposed Text for Committee Draft

## For 4x4 sub-block based CABAC context selection (Pattern1)

###### 9.3.3.1.1.5 Derivation process of ctxIdxInc for the syntax element significant_coeff_flag

Inputs to this process are the color component index cIdx, the current coefficient scan position ( xC , yC ), the transform block width log2TrafoWidth and the transform block height log2TrafoHeight.

Output of this process is ctxIdxInc.

….

- The variable sigCtx is modified as follows.

  - If cIdx is equal to 0 and xC + yC are greater than (1 << (max(log2TrafoWidth, log2TrafoHeight) − 2)) − 1, the following applies.

  - If cIdx is equal to 0 and (xC>>2) + (yC>>2) are greater than (1 << (max(log2TrafoWidth, log2TrafoHeight) − 4)) − 1, the following applies.

sigCtx = ( (sigCtx + 1) >> 1 ) + 24		(9-63)

  - Otherwise, the following applies.

sigCtx = ( (sigCtx + 1) >> 1 ) + ( (cIdx > 0) ? 18 : 21 )	(9-64)

## For unified context selection for 16x16 and 32x32 TUs (Pattern2)

###### 9.3.3.1.1.5 Derivation process of ctxIdxInc for the syntax element significant_coeff_flag

Inputs to this process are the color component index cIdx, the current coefficient scan position ( xC , yC ), the transform block width log2TrafoWidth and the transform block height log2TrafoHeight.

Output of this process is ctxIdxInc.

….

- The variable sigCtx is modified as follows.

  - If cIdx is equal to 0 and xC + yC are greater than (1 << (max(log2TrafoWidth, log2TrafoHeight) − 2)) − 1, the following applies.

  - If cIdx is equal to 0 and (xC>>2) + (yC>>2) are greater than 0, the following applies.

sigCtx = ( (sigCtx + 1) >> 1 ) + 24		(9-63)

  - Otherwise, the following applies.

sigCtx = ( (sigCtx + 1) >> 1 ) + ( (cIdx > 0) ? 18 : 21 )	(9-64)