| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 8th Meeting: San José, CA, USA, 1–10 February, 2012 | Document: JCTVC-H0295 WG11 Number: m23170 |
| --- | --- |

| Title: | CE6.a: Cross-channel prediction refinement to improve intra chroma prediction |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yi-Jen Chiu 2200 Mission College Blvd. Santa Clara CA USA 95054 | Tel: Email: | 408-765-9494 yi-jen.chiu@intel.com |
|  | Yu Han 8F Raycom Infotech Park A, No. 2 KeXueYuan South Rd., Haidian District, Beijing, China, 100190 | Tel: Email: | 86-10-82171388 yu.han@intel.com |
|  | Lidong Xu 8F Raycom Infotech Park A, No. 2 KeXueYuan South Rd., Haidian District, Beijing, China, 100190 | Tel: Email: | 86-10-82171425 lidong.xu@intel.com |
|  | Wenhao Zhang 8F Raycom Infotech Park A, No. 2 KeXueYuan South Rd., Haidian District, Beijing, China, 100190 | Tel: Email: | 86-10-82171657 wenhao.zhang@intel.com |
|  | Hong Jiang 2200 Mission College Blvd. Santa Clara CA USA 95054 | Tel: Email: | 916-356-0457 hong.h.jiang@intel.com |
| Source: | Intel Corp. |  |  |

_____________________________

# Abstract

This contribution reports the test result of the cross-channel prediction refinement technique to improve intra chroma prediction of HM 5.0. Presented at JCT-VC meeting at Geneva, the cross-channel prediction refinement technique produces a refinement signal based on a fixed weighting of the reconstructed cross-channel chroma residual signal. Two test data points of the chroma cross-channel prediction refinement are reported: global weighting prediction and mode-dependent weighting prediction. Compared to HM5.0 anchor of the common test condition, the mode-dependent weighting prediction reports an average BD bitrate improvement of -0.3% on Y, -0.6% on Cb, and -3.4% on Cr on the AI_HE test cases A-E, of -0.3% on Y, -0.8% on Cb, and -3.3% on Cr on AI_LC test cases A-E, and of -0.3% on Y, -0.6% on Cb, and -3.3% on Cr on AI_HE-10 test cases A-F, without significant impact on the CPU run time (1% decrease in Enc time, 0% increase in Dec time for AI_HE case, 0% increase in Enc time, 1% increase in Dec time for AI_LC case, and 1% decrease in Enc time, 0% increase in Dec time for AI_HE-10 case).

# Introduction

This contribution presents a cross-channel prediction refinement technique to improve the intra chroma prediction of HM 5.0. This proposed chroma cross-channel prediction refinement technique produces a prediction refinement signal of Cr channel based upon a fixed weighting of the collocated reconstructed Cb residual pixels. That is, the residual of Cb pixel can be linearly refined from the collocated reconstructed residual of Cb.

Two scenarios of fixed weighting of cross-channel prediction refinement are tested:  global fixed weighting prediction and mode-dependent weighting prediction. The global fixed weighting prediction reported in [1] applies an identical weighting value across all of intra chroma prediction modes. The mode-dependent weighting prediction applies a different weighting value for different intra chroma prediction mode. The detail of the work is described in next section, the test result is provided in section 3, and section 4 concludes the report.

# Cr residual predicted from reconstructed Cb residual

The proposed prediction refinement technique of estimating the Cr residual based upon the reconstructed Cb residual is always applied on the process of intra chroma prediction excluding the occurrence of LM mode. No additional control flag is required to indicate the occurrence of this cross-channel chroma residual.

The flow of the proposed techniques is described in the following steps:

- At the first step, the Cr pixels are predicted by the regular intra chroma prediction mode. For the pixel located at (x, y), the prediction of Cr pixel is denoted asPredCr[x,y].

- If the intra chroma prediction mode at the first step does not belong to LM mode, a prediction refinement of Cr pixels is produced by the reconstructed residual of Cb pixels of the collocated PU. The formula prediction refinement is proposed to be a linear model for the pixel located at (x, y) as follows,

Mod_PredCr[x,y]=α⋅ResiRecCb'[x,y]	               (1)

where Mod_PredCr is the prediction refinement of Cr pixels, ResiRecCb´ is the reconstructed residual value of Cb pixels, and α is a weighting parameter.

- The final prediction of Cr pixel located at (x, y) is

final_PredCr[x,y]=Clip(PredCr[x,y]+Mod_PredCr[x,y])        	(2)

We mark that this cross-channel prediction refinement is not applied in LM mode.

Global fixed weighting prediction:

The weighting parameter of α of the global fixed weighting version utilizes an identical fixed value for all of intra prediction modes (-1/2 by default), tested in [1]. The final prediction of Cr pixel located at (x, y) is as bellow:

final_PredCr[x,y]=Clip(PredCr[x,y]+(−12)⋅ResiRecCb'[x,y])      	(3)

Mode-dependent weighting prediction:

The mode-dependent weighting version applies different weighting parameters α to different intra chroma prediction modes. The final prediction of Cr pixel located at (x, y) is

final_PredCr[x,y]=Clip(PredCr[x,y]+α⋅ResiRecCb'[x,y])      	(4)

where α=-1/4 for DC mode, α=-3/4 for Planar mode, and α=-1/2 for the remaining modes as specified in table 1.

Table 1 – weighting parameter for different intra chroma prediction modes

| α value | Intra prediction mode |
| --- | --- |
| -1/4 | DC mode |
| -3/4 | Planar mode |
| Not Applied | LM Mode |
| -1/2 | Remaining modes |

# Experimental results

Tables 2-5 summarize the coding efficiency results under the JCT-VC common test conditions for Intra HE, Intra LC and Intra HE10 configurations. The BD bitrate of YUV is generated as the formula specified in JCTVC-F386 [2].

Experimental results of global fixed weighting prediction:

Table 2 provides the test result based on HM 5.0 reference software for mandatory test. Table 3 provides the test result based on HM 5.0including optional test (Class F and All Intra HE-10 test case).

Table 2 - the performance of global fixed weighting prediction on HM 5.0 reference software for mandatory test

|  | All Intra HE |  |  |  | All Intra LC |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | YUV | Y | U | V | YUV |
| Class A (8bit) | -0.4% | 0.1% | -3.9% | -0.6% | 0.0% | 0.1% | -1.6% | -0.2% |
| Class B | 0.0% | 0.1% | -2.7% | -0.2% | 0.4% | 0.8% | 0.5% | 0.4% |
| Class C | -0.5% | -1.3% | -3.2% | -0.8% | -0.6% | -1.4% | -3.1% | -0.9% |
| Class D | -0.5% | -1.4% | -3.3% | -0.8% | -0.6% | -1.3% | -3.6% | -0.9% |
| Class E | -0.2% | -0.2% | -2.6% | -0.4% | -0.3% | -0.7% | -2.3% | -0.5% |
| Overall | -0.3% | -0.6% | -3.1% | -0.6% | -0.2% | -0.5% | -1.9% | -0.4% |
|  | -0.3% | -0.6% | -3.0% | -0.6% | -0.2% | -0.5% | -1.9% | -0.4% |
| Class F | -0.5% | -1.4% | -2.0% | -0.7% | -0.6% | -1.1% | -3.1% | -0.9% |
| Enc Time[%] | 100% |  |  |  | 100% |  |  |  |
| Dec Time[%] | 101% |  |  |  | 101% |  |  |  |

Table 3 - the performance of global fixed weighting prediction on HM 5.0 reference software including optional test

|  | All Intra HE |  |  |  | All Intra LC |  |  |  | All Intra HE-10 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | YUV | Y | U | V | YUV | Y | U | V | YUV |
| Class A | -0.2% | 0.4% | -2.2% | -0.3% | 0.2% | 0.3% | -1.0% | 0.1% | -0.2% | 0.5% | -2.6% | -0.4% |
| Class B | 0.0% | 0.1% | -2.7% | -0.2% | 0.4% | 0.8% | 0.5% | 0.4% | -0.1% | 0.0% | -3.1% | -0.3% |
| Class C | -0.5% | -1.3% | -3.2% | -0.8% | -0.6% | -1.4% | -3.1% | -0.9% | -0.5% | -1.3% | -3.3% | -0.8% |
| Class D | -0.5% | -1.4% | -3.3% | -0.8% | -0.6% | -1.3% | -3.6% | -0.9% | -0.5% | -1.4% | -3.3% | -0.8% |
| Class E | -0.2% | -0.2% | -2.6% | -0.4% | -0.3% | -0.7% | -2.3% | -0.5% | -0.2% | -0.4% | -3.1% | -0.5% |
| Class F | -0.5% | -1.4% | -2.0% | -0.7% | -0.6% | -1.1% | -3.1% | -0.9% | -0.5% | -1.3% | -2.2% | -0.7% |
| Overall | -0.3% | -0.6% | -2.7% | -0.5% | -0.2% | -0.5% | -2.0% | -0.4% | -0.3% | -0.6% | -2.9% | -0.6% |
|  | -0.3% | -0.6% | -2.7% | -0.5% | -0.2% | -0.5% | -1.9% | -0.4% | -0.3% | -0.6% | -2.9% | -0.6% |
| Enc Time[%] | 100% |  |  |  | 100% |  |  |  | 100% |  |  |  |
| Dec Time[%] | 101% |  |  |  | 101% |  |  |  | 100% |  |  |  |

Experimental results of mode-dependent weighting prediction:

Table 4 provides the test result based on HM 5.0 reference software for mandatory test. Table 5 provides the test result based on HM 5.0 including optional test.

Table 4 - the performance of mode-dependent weighting prediction on HM 5.0 reference software for mandatory test

|  | All Intra HE |  |  |  | All Intra LC |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | YUV | Y | U | V | YUV |
| Class A (8bit) | -0.3% | 0.1% | -4.1% | -0.6% | -0.1% | -0.4% | -3.5% | -0.4% |
| Class B | 0.0% | 0.2% | -2.9% | -0.2% | 0.3% | 0.1% | -1.6% | 0.1% |
| Class C | -0.5% | -1.3% | -3.6% | -0.9% | -0.7% | -1.5% | -4.5% | -1.1% |
| Class D | -0.5% | -1.3% | -3.8% | -0.9% | -0.7% | -1.4% | -4.8% | -1.1% |
| Class E | -0.2% | -0.3% | -2.7% | -0.4% | -0.2% | -0.7% | -2.6% | -0.5% |
| Overall | -0.3% | -0.6% | -3.4% | -0.6% | -0.3% | -0.8% | -3.3% | -0.6% |
|  | -0.3% | -0.6% | -3.3% | -0.6% | -0.3% | -0.8% | -3.3% | -0.6% |
| Class F | -0.5% | -1.4% | -2.4% | -0.8% | -0.7% | -1.0% | -4.0% | -1.1% |
| Enc Time[%] | 99% |  |  |  | 100% |  |  |  |
| Dec Time[%] | 100% |  |  |  | 101% |  |  |  |

Table 5 - the performance of mode-dependent weighting prediction on HM 5.0 reference software including optional test

|  | All Intra HE |  |  |  | All Intra LC |  |  |  | All Intra HE-10 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | YUV | Y | U | V | YUV | Y | U | V | YUV |
| Class A | -0.2% | 0.4% | -2.4% | -0.3% | 0.1% | -0.1% | -2.3% | -0.1% | -0.2% | 0.5% | -2.9% | -0.3% |
| Class B | 0.0% | 0.2% | -2.9% | -0.2% | 0.3% | 0.1% | -1.6% | 0.1% | -0.1% | 0.1% | -3.3% | -0.3% |
| Class C | -0.5% | -1.3% | -3.6% | -0.9% | -0.7% | -1.5% | -4.5% | -1.1% | -0.5% | -1.3% | -3.8% | -0.9% |
| Class D | -0.5% | -1.3% | -3.8% | -0.9% | -0.7% | -1.4% | -4.8% | -1.1% | -0.5% | -1.3% | -3.9% | -0.9% |
| Class E | -0.2% | -0.3% | -2.7% | -0.4% | -0.2% | -0.7% | -2.6% | -0.5% | -0.2% | -0.4% | -3.1% | -0.5% |
| Class F | -0.5% | -1.4% | -2.4% | -0.8% | -0.7% | -1.0% | -4.0% | -1.1% | -0.5% | -1.3% | -2.6% | -0.8% |
| Overall | -0.3% | -0.6% | -3.0% | -0.6% | -0.3% | -0.7% | -3.3% | -0.6% | -0.3% | -0.6% | -3.3% | -0.6% |
|  | -0.3% | -0.6% | -3.0% | -0.6% | -0.3% | -0.7% | -3.2% | -0.6% | -0.3% | -0.6% | -3.3% | -0.6% |
| Enc Time[%] | 99% |  |  |  | 100% |  |  |  | 99% |  |  |  |
| Dec Time[%] | 100% |  |  |  | 101% |  |  |  | 100% |  |  |  |

# Conclusion

This contribution reports the test result cross-channel prediction refinement technique to improve the intra chroma prediction of HM 5.0. The cross-channel prediction refinement technique produces a refinement signal based on a fixed weighting of the reconstructed cross-channel chroma residual signal. Two test data points of the chroma cross-channel prediction refinement are reported: global weighting prediction and mode-dependent weighting prediction. Compared to HM5.0 anchor of the common test condition, the mode-dependent weighting prediction reports an average BD bitrate improvement of -0.3% on Y, -0.6% on Cb, and -3.4% on Cr on the AI_HE test cases A-E, of -0.3% on Y, -0.8% on Cb, and -3.3% on Cr on AI_LC test cases A-E, and of -0.3% on Y, -0.6% on Cb, and -3.3% on Cr on AI_HE-10 test cases A-F, without significant impact on the CPU run time (1% decrease in Enc time, 0% increase in Dec time for AI_HE case, 0% increase in Enc time, 1% increase in Dec time for AI_LC case, and 1% decrease in Enc time, 0% increase in Dec time for AI_HE-10 case).

# References

- Y. Chiu, Y. Han, L. Xu, W. Zhang, and H. Jiang, “Cross-channel intra chroma residual prediction,” Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-G173, 7th Meeting: Geneva, Switzerland, 21-30 November, 2011.

- TK Tan and F. Bossen, “Chroma RD cost computation in HM3.0,” Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-F386, 6th Meeting: Torino, Italy, 14-22 July, 2011.

- F. Bossen, “Common test conditions and software reference configurations,” Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-G1200, 7th Meeting: Geneva, Switzerland, 21-30 November, 2011.

# Patent rights declaration(s)

Intel Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Appendix: text for working draft change (add)

8.3.3.1.9 Specification of cross-channel intra chroma prediction

Inputs to this process are:

–	a sample location ( xB, yB ) specifying the top-left sample of the current block relative to the top-left sample of the current picture,

–	neighbouring chroma samples pCb[ x, y ] and pCr[ x, y ], with x, y = -1..nS-1,

–	a variable nS specifying the prediction size,

–	a variable cIdx specifying the chroma component of the current block.

Output of this process is:

–	the predicted samples predSamples[ x, y ], with x, y =0..nS-1.

This process is invoked when decoding cIdx=1 and intraPredMode is not equal to 35 (LM mode).

The values of the prediction samples predSamples[ x, y ], with x, y = 0..nS-1, are derived as the follows:

predSamples[ x, y ] = Clip1C ( predSamplesCr[x, y] + α * resSamplesCb [ x, y ] ), with x, y = 0..nS-1

where, predSamplesCr is the array of Cr samples predicted by intra directional prediction. α is specified as bellows:

Global fixed weighting prediction:

α=-1/2, for all of intra prediction modes

Mode-dependent weighting prediction:

α=-1/4, for DC mode

α=-3/4, for Planar mode

α=-1/2, for remaining modes