| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 8th Meeting: San José, CA, USA, 1–10 February, 2012 | Document: JCTVC-H0296 WG11 Number: m23171 |
| --- | --- |

| Title: | CE6.a: Cross-check report for CE6a on intra chroma prediction |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Yi-Jen Chiu 2200 Mission College Blvd. Santa Clara CA USA 95054 | Tel: Email: | 408-765-9494 yi-jen.chiu@intel.com |
|  | Yu Han 8F Raycom Infotech Park A, No. 2 KeXueYuan South Rd., Haidian District, Beijing, China, 100190 | Tel: Email: | 86-10-82171388 yu.han@intel.com |
|  | Lidong Xu 8F Raycom Infotech Park A, No. 2 KeXueYuan South Rd., Haidian District, Beijing, China, 100190 | Tel: Email: | 86-10-82171425 lidong.xu@intel.com |
|  | Wenhao Zhang 8F Raycom Infotech Park A, No. 2 KeXueYuan South Rd., Haidian District, Beijing, China, 100190 | Tel: Email: | 86-10-82171657 wenhao.zhang@intel.com |
| Source: | Intel Corp. |  |  |

_____________________________

# Abstract

CE6.a aims to improve the intra chroma prediction of the HEVC Test Model. This contribution presents a summary of the results by Intel.

# Introduction

The following tools are considered in this cross-check for CE6.a:

- Cross-channel intra chroma residual prediction (JCTVC-G173)

- Luma-based chroma prediction - Model correction (JCTVC-G244)

- Chroma intra prediction based on residual luma samples (JCTVC-G346)

- New modes for chroma intra prediction (JCTVC-G358)

This contribution reports the cross-check results of different combinations of these tools. It is used to study the potential optimal combination in terms of trade-off coding efficiency/complexity. When the tools in G173 and G358 are both enabled, cross-channel intra chroma residual prediction will applied to the new intra chroma prediction mode proposed in G358.

# Simulation Results

These combinations experiments are tested under the common test conditions for AI_HE, AI_LC and AI_HE10 cases. The LM mode is removed from AI_LC case since last Geneva meeting. Because the tools of JCTVC-G244 and JCTVC-G358 are similar to the LM mode, the combinations include these two proposals don’t test in AI_LC case. A summary of results for AI_HE and AI_LC cases is listed in Error: Reference source not found, where the anchor is HM5.0. Complete results are available in the accompanied spread sheets.

Table 1 Crosscheck results of CE6.a

| All Class A-E | HE |  |  |  | LC |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | YUV | Y | U | V | YUV |
| G173 | -0.30% | -0.58% | -3.06% | -0.55% | -0.21% | -0.47% | -1.92% | -0.38% |
| Enc Time[%] | 100% |  |  |  | 100% |  |  |  |
| Dec Time[%] | 101% |  |  |  | 101% |  |  |  |
| G244 | -0.07% | -1.74% | -1.29% | -0.32% |  |  |  |  |
| Enc Time[%] | 99% |  |  |  |  |  |  |  |
| Dec Time[%] | 100% |  |  |  |  |  |  |  |
| G358 | -0.11% | -2.22% | -2.21% | -0.47% |  |  |  |  |
| Enc Time[%] | 104% |  |  |  |  |  |  |  |
| Dec Time[%] | 101% |  |  |  |  |  |  |  |
| G173_G244 | -0.36% | -2.22% | -3.84% | -0.80% |  |  |  |  |
| Enc Time[%] | 99% |  |  |  |  |  |  |  |
| Dec Time[%] | 101% |  |  |  |  |  |  |  |
| G173_G346 | -0.36% | -3.16% | -5.15% | -1.01% | -0.49% | -4.96% | -5.99% | -1.39% |
| Enc Time[%] | 101% |  |  |  | 101% |  |  |  |
| Dec Time[%] | 102% |  |  |  | 103% |  |  |  |
| G173_G358 | -0.39% | -2.79% | -5.05% | -0.99% |  |  |  |  |
| Enc Time[%] | 104% |  |  |  |  |  |  |  |
| Dec Time[%] | 101% |  |  |  |  |  |  |  |
| G244_G346 | -0.11% | -3.60% | -3.23% | -0.68% |  |  |  |  |
| Enc Time[%] | 100% |  |  |  |  |  |  |  |
| Dec Time[%] | 102% |  |  |  |  |  |  |  |
| G244_G358 | -0.13% | -3.48% | -3.30% | -0.69% |  |  |  |  |
| Enc Time[%] | 104% |  |  |  |  |  |  |  |
| Dec Time[%] | 101% |  |  |  |  |  |  |  |
| G346_G358 | -0.15% | -3.96% | -3.96% | -0.82% |  |  |  |  |
| Enc Time[%] | 105% |  |  |  |  |  |  |  |
| Dec Time[%] | 102% |  |  |  |  |  |  |  |
| G173_G244_G346 | -0.36% | -4.06% | -5.59% | -1.13% |  |  |  |  |
| Enc Time[%] | 101% |  |  |  |  |  |  |  |
| Dec Time[%] | 103% |  |  |  |  |  |  |  |
| G173_G244_G358 | -0.40% | -4.00% | -5.76% | -1.16% |  |  |  |  |
| Enc Time[%] | 104% |  |  |  |  |  |  |  |
| Dec Time[%] | 102% |  |  |  |  |  |  |  |
| G244_G346_G358 | -0.16% | -4.81% | -4.69% | -0.96% |  |  |  |  |
| Enc Time[%] | 105% |  |  |  |  |  |  |  |
| Dec Time[%] | 102% |  |  |  |  |  |  |  |
| G173_G346_G358 | -0.41% | -4.50% | -6.52% | -1.29% |  |  |  |  |
| Enc Time[%] | 105% |  |  |  |  |  |  |  |
| Dec Time[%] | 102% |  |  |  |  |  |  |  |
| G173_G244_G346_G358 | -0.41% | -5.31% | -6.99% | -1.40% |  |  |  |  |
| Enc Time[%] | 106% |  |  |  |  |  |  |  |
| Dec Time[%] | 102% |  |  |  |  |  |  |  |

# References

- Y. Chiu, Y. Han, L. Xu, W. Zhang, and H. Jiang, “Cross-channel intra chroma residual prediction,” Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-G173, 7th Meeting: Geneva, Switzerland, 21-30 November, 2011.

- C. Gisquet and E. François, “Non-CE6a: Luma-based chroma prediction - Model correction,” Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-G173, 7th Meeting: Geneva, Switzerland, 21-30 November, 2011.

- K. Kawamura, T. Yoshino, H. Kato, S. Naito, “Chroma intra prediction based on residual luma samples,” Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-G173, 7th Meeting: Geneva, CH, 21-30 November, 2011.

- X. Zhang, O. C. Au, J. Dai, F. Zou, C. Pang, X. Wen, “New modes for chroma intra prediction,” Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-G358, 7th Meeting: Geneva, Switzerland, 21-30 November, 2011.

- F. Bossen, “Common test conditions and software reference configurations”, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-G1200, 7th Meeting: Geneva, Switzerland, 21-30 November, 2011.