| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 8th Meeting: San José, CA, USA, 1–10 February, 2012 | Document: JCTVC-H0297 WG11 Number: m23172 |
| --- | --- |

| Title: | CE6.b: Cross verification of MediaTek’s rectangular (2NxN and Nx2N) intra prediction (JCTVC-H0437) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Yi-Jen Chiu 2200 Mission College Blvd. Santa Clara CA USA 95054 | Tel: Email: | 408-765-9494 yi-jen.chiu@intel.com |
|  | Yu Han 8F Raycom Infotech Park A, No. 2 KeXueYuan South Rd., Haidian District, Beijing, China, 100190 | Tel: Email: | 86-10-82171388 yu.han@intel.com |
|  | Lidong Xu 8F Raycom Infotech Park A, No. 2 KeXueYuan South Rd., Haidian District, Beijing, China, 100190 | Tel: Email: | 86-10-82171425 lidong.xu@intel.com |
|  | Wenhao Zhang 8F Raycom Infotech Park A, No. 2 KeXueYuan South Rd., Haidian District, Beijing, China, 100190 | Tel: Email: | 86-10-82171657 wenhao.zhang@intel.com |
| Source: | Intel Corp. |  |  |

_____________________________

# Abstract

This contribution provides the cross-check results of rectangular (2NxN and Nx2N) intra prediction proposed by MediaTek. Two prediction unit (PU) types (i.e. 2NxN and Nx2N) were added for Intra coding units (CU) in addition to the existing PU types (i.e. 2Nx2N and NxN) in current HM5.0. The purpose of this experiment is to test the coding efficiency and run-time of the proposed method and its harmonization with the NSQT based on HM5.0 software.

# Simulation Results

The software package obtained from the proponent was successfully compiled and tested under the common test conditions for AI_HE case. The BD rate results match exactly to the data provided by the proponent, and the dec time is close. The enc time is 6-7% less than that provided by the proponent. This cross-check includes 3 tests.

- Test 1: G135 only

The software from the proponents was complied with macro settings as follows. Table 1 shows the performance data of the proposed technique.

#define MTK_G135                     1

#if MTK_G135

#define MTK_G135_NSQT                0

#if MTK_G135_NSQT

#define MTK_G135_NSQT_82_28          0

#endif

#endif

Table 1 Crosscheck results of G135 only

|  | All Intra HE |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Class A (8bit) | -1.0% | -2.1% | -2.7% |
| Class B | -1.4% | -2.3% | -2.4% |
| Class C | -1.8% | -1.9% | -2.0% |
| Class D | -1.8% | -1.6% | -1.6% |
| Class E | -2.4% | -5.5% | -4.8% |
| Overall | -1.7% | -2.5% | -2.6% |
|  | -1.7% | -2.5% | -2.5% |
| Class F | -5.1% | -4.4% | -4.2% |
| Enc Time[%] | 126% |  |  |
| Dec Time[%] | 103% |  |  |

- Test 2: harmonization of G135 with NSQT

The software from the proponents was complied with macro settings as follows. Table 2 shows the performance data of the proposed technique.

#define MTK_G135                     1

#if MTK_G135

#define MTK_G135_NSQT                1

#if MTK_G135_NSQT

#define MTK_G135_NSQT_82_28          0

#endif

#endif

Table 2 Crosscheck results of the harmonization of G135 with NSQT

|  | All Intra HE |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Class A (8bit) | -0.8% | -2.0% | -2.6% |
| Class B | -0.9% | -2.1% | -2.2% |
| Class C | -0.9% | -1.6% | -1.7% |
| Class D | -0.8% | -1.1% | -1.1% |
| Class E | -1.6% | -4.6% | -4.0% |
| Overall | -1.0% | -2.2% | -2.2% |
|  | -1.0% | -2.1% | -2.2% |
| Class F | -0.9% | -1.2% | -1.0% |
| Enc Time[%] | 119% |  |  |
| Dec Time[%] | 102% |  |  |

- Test 3: harmonization of G135 with NSQT plus 8x2/2x8 TU for 8x4/4x8 PU (optional)

The software from the proponents was complied with macro settings as follows. Table 3 shows the performance data of the proposed technique.

#define MTK_G135                     1

#if MTK_G135

#define MTK_G135_NSQT                1

#if MTK_G135_NSQT

#define MTK_G135_NSQT_82_28          1

#endif

#endif

Table 3 Crosscheck results of the harmonization of G135 with NSQT plus 8x2/2x8 TU for 8x4/4x8 PU

|  | All Intra HE |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Class A (8bit) | -0.9% | -2.0% | -2.6% |
| Class B | -1.2% | -2.1% | -2.2% |
| Class C | -1.7% | -1.8% | -2.0% |
| Class D | -1.7% | -1.5% | -1.5% |
| Class E | -1.9% | -4.8% | -4.1% |
| Overall | -1.5% | -2.3% | -2.4% |
|  | -1.5% | -2.3% | -2.3% |
| Class F | -5.0% | -4.1% | -4.1% |
| Enc Time[%] | 126% |  |  |
| Dec Time[%] | 103% |  |  |

# References

- S. Liu, X. Zhang, Z. Zhou and S. Lei, “Non-CE6: Rectangular (2NxN and Nx2N) Intra Prediction”, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-G135, 7th Meeting: Geneva, Switzerland, 21-30 November, 2011.

- F. Bossen, “Common test conditions and software reference configurations”, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-G1200, 7th Meeting: Geneva, Switzerland, 21-30 November, 2011.