| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 8th Meeting: San José, CA, USA, 1–10 February, 2012 | Document: JCTVC-H0298 WG11 Number: m23173 |
| --- | --- |

| Title: | CE10 Subtest1: Cross verification of MediaTek’s chroma deblocking filter (JCTVC-H0310) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Yi-Jen Chiu 2200 Mission College Blvd. Santa Clara CA USA 95054 | Tel: Email: | 408-765-9494 yi-jen.chiu@intel.com |
|  | Lidong Xu 8F Raycom Infotech Park A, No. 2 KeXueYuan South Rd., Haidian District, Beijing, China, 100190 | Tel: Email: | 86-10-82171425 lidong.xu@intel.com |
|  | Yu Han 8F Raycom Infotech Park A, No. 2 KeXueYuan South Rd., Haidian District, Beijing, China, 100190 | Tel: Email: | 86-10-82171388 yu.han@intel.com |
|  | Wenhao Zhang 8F Raycom Infotech Park A, No. 2 KeXueYuan South Rd., Haidian District, Beijing, China, 100190 | Tel: Email: | 86-10-82171657 wenhao.zhang@intel.com |
| Source: | Intel Corp. |  |  |

_____________________________

# Abstract

This contribution provides the cross-check results of MediaTek’s proposed chroma deblock filter, which applies independent filtering decision and 8x8 filtering unit for chroma deblocking, in CE10 subtest 1. The software package obtained from the proponent was successfully compiled and tested under CE10  common test conditions (including the optional tests of LD_P cases). The BD rate results match exactly to the data provided by the proponent, and the differences of enc/dec time are within normal variations. Table 1-4 show the simulation results.

Informal subjective viewing showed similar visual quality with HM5.0 anchor results. Some areas in the proposal results showed better visual quality and some areas showed slightly worse. It is hard to say it is caused by the proposal or by the change of motion prediction.

Table 1 Simulation results of AI cases

|  | All Intra HE |  |  | All Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A (8bit) | 0.0% | -0.3% | -0.3% | 0.0% | -0.1% | 0.0% |
| Class B | 0.0% | -0.4% | -0.4% | 0.0% | -0.1% | 0.0% |
| Class C | 0.0% | -0.7% | -0.7% | 0.0% | 0.0% | 0.0% |
| Class D | 0.0% | -0.6% | -0.6% | 0.0% | 0.1% | 0.1% |
| Class E | 0.0% | 0.0% | -0.1% | 0.0% | 0.0% | 0.1% |
| Overall | 0.0% | -0.4% | -0.5% | 0.0% | 0.0% | 0.0% |
|  | 0.0% | -0.4% | -0.5% | 0.0% | 0.0% | 0.0% |
| Class F | 0.0% | -0.8% | -0.9% | 0.0% | -0.5% | -0.7% |
| Enc Time[%] | 99% |  |  | 98% |  |  |
| Dec Time[%] | 100% |  |  | 99% |  |  |

Table 2 Simulation results of RA cases

|  | Random Access HE |  |  | Random Access LC |  |  | Random Access HE-10 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A (8bit) | 0.0% | -0.4% | -0.4% | 0.0% | -0.5% | -0.4% | 0.0% | -0.5% | -0.5% |
| Class B | 0.0% | -0.5% | -0.5% | 0.0% | -0.9% | -0.8% | 0.0% | -0.6% | -0.6% |
| Class C | 0.0% | -0.7% | -0.7% | 0.0% | -1.1% | -1.1% |  |  |  |
| Class D | 0.0% | -0.8% | -0.8% | 0.0% | -0.9% | -1.1% |  |  |  |
| Class E |  |  |  |  |  |  |  |  |  |
| Overall | 0.0% | -0.6% | -0.6% | 0.0% | -0.9% | -0.9% | 0.0% | -0.6% | -0.5% |
|  | 0.0% | -0.6% | -0.6% | 0.0% | -0.9% | -0.9% | 0.0% | -0.6% | -0.5% |
| Class F | -0.1% | -1.1% | -1.2% | 0.0% | -1.2% | -1.3% |  |  |  |
| Enc Time[%] | 99% |  |  | 99% |  |  | 99% |  |  |
| Dec Time[%] | 100% |  |  | 99% |  |  | 100% |  |  |

Table 3 Simulation results of low delay B cases

|  | Low delay B HE |  |  | Low delay B LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.0% | -0.3% | -0.2% | 0.0% | -1.2% | -0.7% |
| Class C | 0.0% | -0.6% | -0.6% | 0.0% | -0.7% | -1.1% |
| Class D | 0.0% | -0.6% | -0.4% | -0.1% | -0.3% | -0.7% |
| Class E | 0.0% | -1.0% | -1.8% | -0.1% | -3.3% | -2.2% |
| Overall | 0.0% | -0.6% | -0.7% | 0.0% | -1.2% | -1.1% |
|  | 0.0% | -0.6% | -0.7% | 0.0% | -1.2% | -1.0% |
| Class F | -0.2% | -0.7% | -1.1% | -0.1% | -1.2% | -1.5% |
| Enc Time[%] | 99% |  |  | 99% |  |  |
| Dec Time[%] | 100% |  |  | 101% |  |  |

Table 4 Simulation results of low delay P cases

|  | Low delay P HE |  |  | Low delay P LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.0% | -0.6% | -0.5% | 0.0% | -1.4% | -1.4% |
| Class C | 0.0% | -0.5% | -0.8% | 0.0% | -0.9% | -1.0% |
| Class D | -0.1% | -0.2% | -0.6% | -0.1% | -1.6% | -0.4% |
| Class E | -0.1% | -1.0% | -1.5% | 0.0% | -3.8% | -2.7% |
| Class F | 0.0% | -0.6% | -0.8% | -0.1% | -1.2% | -1.3% |
| Overall | -0.1% | -0.6% | -0.8% | 0.0% | -1.7% | -1.3% |
|  | -0.1% | -0.6% | -0.8% | 0.0% | -1.7% | -1.3% |
| Enc Time[%] | 99% |  |  | 99% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |