| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 8th Meeting: San José, CA, USA, 1–10 February, 2012 | Document: JCTVC-H0305 M23180 |
| --- | --- |

| Title: | The core spatial transform with fast algorithm |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Hongbo Zhu | Email: | hurricanezhb@hotmail.com |
| Source: | None. |  |  |

_____________________________

# Abstract

New odd parts for the 8x8, 16x16 and 32x32 core spatial transforms denoted by 8-bits signed integer are proposed in this document. When compared to fast algorithms of the odd parts of the HM core spatial transform, the new odd parts are with about 10% less addition complexity. The coding efficiency is about same with the current design.

# Introduction

The current design of the core spatial transform has the following features:

- 8-bits signed digital transform matrix.

- The transform matrix is highly near-orthogonal to its transpose matrix.

- The NxN transform matrix is the even part of the 2Nx2N transform matrix.

- The transform could be implemented as full matrix multiplication or partial butterfly.

- The odd parts is symmetric matrix so that the reduced area when FDCT and IDCT are implemented in one chip.

- Highly similar norm across different lines of the transform matrix and different size transform.

- Good coding efficiency.

There are several hundreds of the odd parts for the 8x8 transform matrices, more than several hundred thousands of the odd parts for the 16x16 transform matrices and more than several hundred thousands of the odd part for the 32x32 transform matrices that satisfy all of the upper features. So an optimization program is coded that can find the fast algorithm for any integer transform matrix to find the one that is with least addition complexity. The new found 8x8, 16x16 and 32x32 transform matrices are shown in fig. 1, fig. 2 and fig. 3 respectively. Their partial butterfly flowcharts are shown in figure 4 and figure 5.

The odd part of the 8x8 transform needs 24 addition operations and some shift operations (the odd part of the HM 8x8 transform needs 27 addition operations and some shift operations).

The odd part of the 16x16 transform needs 89 addition operations and some shift operations (the odd part of the HM 16x16 transform needs 102 addition operations and some shift operations).

The odd part of the 32x32 transform needs 344 addition operations and some shift operations (the odd part of the HM 32x32 transform needs 377 addition operations and some shift operations).

All the fast algorithms of the odd parts for the new transforms and the HM transforms are listed in the file FstFct_BSTandHM.cpp.

64, 64, 64, 64, 64, 64, 64, 64,   89, 75, 50, 18,-18,-50,-75,-89,   83, 36,-36,-83,-83,-36, 36, 83,   75,-18,-89,-50, 50, 89, 18,-75,   64,-64,-64, 64, 64,-64,-64, 64,   50,-89, 18, 75,-75,-18, 89,-50,   36,-83, 83,-36,-36, 83,-83, 36,   18,-50, 75,-89, 89,-75, 50,-18    Figure 1: 8x8 transform matrix   64, 64, 64, 64, 64, 64, 64, 64, 64, 64, 64, 64, 64, 64, 64, 64,   90, 87, 80, 70, 57, 43, 25,  9, -9,-25,-43,-57,-70,-80,-87,-90,   89, 75, 50, 18,-18,-50,-75,-89,-89,-75,-50,-18, 18, 50, 75, 89,   87, 57,  9,-43,-80,-90,-70,-25, 25, 70, 90, 80, 43, -9,-57,-87,   83, 36,-36,-83,-83,-36, 36, 83, 83, 36,-36,-83,-83,-36, 36, 83,   80,  9,-70,-87,-25, 57, 90, 43,-43,-90,-57, 25, 87, 70, -9,-80,   75,-18,-89,-50, 50, 89, 18,-75,-75, 18, 89, 50,-50,-89,-18, 75,   70,-43,-87,  9, 90, 25,-80,-57, 57, 80,-25,-90, -9, 87, 43,-70,   64,-64,-64, 64, 64,-64,-64, 64, 64,-64,-64, 64, 64,-64,-64, 64,   57,-80,-25, 90, -9,-87, 43, 70,-70,-43, 87,  9,-90, 25, 80,-57,   50,-89, 18, 75,-75,-18, 89,-50,-50, 89,-18,-75, 75, 18,-89, 50,   43,-90, 57, 25,-87, 70,  9,-80, 80, -9,-70, 87,-25,-57, 90,-43,   36,-83, 83,-36,-36, 83,-83, 36, 36,-83, 83,-36,-36, 83,-83, 36,   25,-70, 90,-80, 43,  9,-57, 87,-87, 57, -9,-43, 80,-90, 70,-25,   18,-50, 75,-89, 89,-75, 50,-18,-18, 50,-75, 89,-89, 75,-50, 18,    9,-25, 43,-57, 70,-80, 87,-90, 90,-87, 80,-70, 57,-43, 25, -9              Figure 2: 16x16 transform matrix

64, 64, 64, 64, 64, 64, 64, 64, 64, 64, 64, 64, 64, 64, 64, 64, 64, 64, 64, 64, 64, 64, 64, 64, 64, 64, 64, 64, 64, 64, 64, 64,   90, 90, 88, 85, 82, 78, 73, 67, 61, 54, 46, 38, 31, 22, 13,  4, -4,-13,-22,-31,-38,-46,-54,-61,-67,-73,-78,-82,-85,-88,-90,-90,   90, 87, 80, 70, 57, 43, 25,  9, -9,-25,-43,-57,-70,-80,-87,-90,-90,-87,-80,-70,-57,-43,-25, -9,  9, 25, 43, 57, 70, 80, 87, 90,   90, 82, 67, 46, 22, -4,-31,-54,-73,-85,-90,-88,-78,-61,-38,-13, 13, 38, 61, 78, 88, 90, 85, 73, 54, 31,  4,-22,-46,-67,-82,-90,   89, 75, 50, 18,-18,-50,-75,-89,-89,-75,-50,-18, 18, 50, 75, 89, 89, 75, 50, 18,-18,-50,-75,-89,-89,-75,-50,-18, 18, 50, 75, 89,   88, 67, 31,-13,-54,-82,-90,-78,-46, -4, 38, 73, 90, 85, 61, 22,-22,-61,-85,-90,-73,-38,  4, 46, 78, 90, 82, 54, 13,-31,-67,-88,   87, 57,  9,-43,-80,-90,-70,-25, 25, 70, 90, 80, 43, -9,-57,-87,-87,-57, -9, 43, 80, 90, 70, 25,-25,-70,-90,-80,-43,  9, 57, 87,   85, 46,-13,-67,-90,-73,-22, 38, 82, 88, 54, -4,-61,-90,-78,-31, 31, 78, 90, 61,  4,-54,-88,-82,-38, 22, 73, 90, 67, 13,-46,-85,   83, 36,-36,-83,-83,-36, 36, 83, 83, 36,-36,-83,-83,-36, 36, 83, 83, 36,-36,-83,-83,-36, 36, 83, 83, 36,-36,-83,-83,-36, 36, 83,   82, 22,-54,-90,-61, 13, 78, 85, 31,-46,-90,-67,  4, 73, 88, 38,-38,-88,-73, -4, 67, 90, 46,-31,-85,-78,-13, 61, 90, 54,-22,-82,   80,  9,-70,-87,-25, 57, 90, 43,-43,-90,-57, 25, 87, 70, -9,-80,-80, -9, 70, 87, 25,-57,-90,-43, 43, 90, 57,-25,-87,-70,  9, 80,   78, -4,-82,-73, 13, 85, 67,-22,-88,-61, 31, 90, 54,-38,-90,-46, 46, 90, 38,-54,-90,-31, 61, 88, 22,-67,-85,-13, 73, 82,  4,-78,   75,-18,-89,-50, 50, 89, 18,-75,-75, 18, 89, 50,-50,-89,-18, 75, 75,-18,-89,-50, 50, 89, 18,-75,-75, 18, 89, 50,-50,-89,-18, 75,   73,-31,-90,-22, 78, 67,-38,-90,-13, 82, 61,-46,-88, -4, 85, 54,-54,-85,  4, 88, 46,-61,-82, 13, 90, 38,-67,-78, 22, 90, 31,-73,   70,-43,-87,  9, 90, 25,-80,-57, 57, 80,-25,-90, -9, 87, 43,-70,-70, 43, 87, -9,-90,-25, 80, 57,-57,-80, 25, 90,  9,-87,-43, 70,   67,-54,-78, 38, 85,-22,-90,  4, 90, 13,-88,-31, 82, 46,-73,-61, 61, 73,-46,-82, 31, 88,-13,-90, -4, 90, 22,-85,-38, 78, 54,-67,   64,-64,-64, 64, 64,-64,-64, 64, 64,-64,-64, 64, 64,-64,-64, 64, 64,-64,-64, 64, 64,-64,-64, 64, 64,-64,-64, 64, 64,-64,-64, 64,   61,-73,-46, 82, 31,-88,-13, 90, -4,-90, 22, 85,-38,-78, 54, 67,-67,-54, 78, 38,-85,-22, 90,  4,-90, 13, 88,-31,-82, 46, 73,-61,   57,-80,-25, 90, -9,-87, 43, 70,-70,-43, 87,  9,-90, 25, 80,-57,-57, 80, 25,-90,  9, 87,-43,-70, 70, 43,-87, -9, 90,-25,-80, 57,   54,-85, -4, 88,-46,-61, 82, 13,-90, 38, 67,-78,-22, 90,-31,-73, 73, 31,-90, 22, 78,-67,-38, 90,-13,-82, 61, 46,-88,  4, 85,-54,   50,-89, 18, 75,-75,-18, 89,-50,-50, 89,-18,-75, 75, 18,-89, 50, 50,-89, 18, 75,-75,-18, 89,-50,-50, 89,-18,-75, 75, 18,-89, 50,   46,-90, 38, 54,-90, 31, 61,-88, 22, 67,-85, 13, 73,-82,  4, 78,-78, -4, 82,-73,-13, 85,-67,-22, 88,-61,-31, 90,-54,-38, 90,-46,   43,-90, 57, 25,-87, 70,  9,-80, 80, -9,-70, 87,-25,-57, 90,-43,-43, 90,-57,-25, 87,-70, -9, 80,-80,  9, 70,-87, 25, 57,-90, 43,   38,-88, 73, -4,-67, 90,-46,-31, 85,-78, 13, 61,-90, 54, 22,-82, 82,-22,-54, 90,-61,-13, 78,-85, 31, 46,-90, 67,  4,-73, 88,-38,   36,-83, 83,-36,-36, 83,-83, 36, 36,-83, 83,-36,-36, 83,-83, 36, 36,-83, 83,-36,-36, 83,-83, 36, 36,-83, 83,-36,-36, 83,-83, 36,   31,-78, 90,-61,  4, 54,-88, 82,-38,-22, 73,-90, 67,-13,-46, 85,-85, 46, 13,-67, 90,-73, 22, 38,-82, 88,-54, -4, 61,-90, 78,-31,   25,-70, 90,-80, 43,  9,-57, 87,-87, 57, -9,-43, 80,-90, 70,-25,-25, 70,-90, 80,-43, -9, 57,-87, 87,-57,  9, 43,-80, 90,-70, 25,   22,-61, 85,-90, 73,-38, -4, 46,-78, 90,-82, 54,-13,-31, 67,-88, 88,-67, 31, 13,-54, 82,-90, 78,-46,  4, 38,-73, 90,-85, 61,-22,   18,-50, 75,-89, 89,-75, 50,-18,-18, 50,-75, 89,-89, 75,-50, 18, 18,-50, 75,-89, 89,-75, 50,-18,-18, 50,-75, 89,-89, 75,-50, 18,   13,-38, 61,-78, 88,-90, 85,-73, 54,-31,  4, 22,-46, 67,-82, 90,-90, 82,-67, 46,-22, -4, 31,-54, 73,-85, 90,-88, 78,-61, 38,-13,    9,-25, 43,-57, 70,-80, 87,-90, 90,-87, 80,-70, 57,-43, 25, -9, -9, 25,-43, 57,-70, 80,-87, 90,-90, 87,-80, 70,-57, 43,-25,  9,    4,-13, 22,-31, 38,-46, 54,-61, 67,-73, 78,-82, 85,-88, 90,-90, 90,-90, 88,-85, 82,-78, 73,-67, 61,-54, 46,-38, 31,-22, 13, -4                                                   Figure 3: 32x32 transform matrix

# Experimental results

The HM5.0 r1682 is used as the platform to test the proposed transform. The simulation experiments were conducted following the testing conditions in [1]. The encoding/decoding times were estimated using two PC listed in table 1. The results are listed in table 2.

| CPU | AMD Phenom(tm) II x6 1075T@3.0GHz (6 cores) | AMD bulldozer FX-8150 @3.6GHz (8 cores) |
| --- | --- | --- |
| DRAM | 6G 1333MHz | 8G 1333MHz |
| BUS | 1333MHz | 4.8GT/s |
| HardDisk | WD 1TB | WD 512GB |
| OS | Win7x64 | Win7x64 |
| EXE | X64 version (VC2008) | X64 version (VC2008) |

Table 1: the computers to generate the results

|  | All Intra HE |  |  | All Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A (8bit) | 0.0% | 0.0% | 0.1% | -0.1% | 0.0% | 0.0% |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class C | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class D | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class E | 0.0% | 0.1% | 0.1% | 0.0% | 0.1% | 0.1% |
| Overall | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |

|  |  | Random Access HE |  |  |  |  | Random Access LC |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Y |  | U |  | V | Y | U | V |
| Class A (8bit) |  | 0.0% |  | 0.0% |  | -0.1% | 0.0% | 0.1% | -0.1% |
| Class B |  | 0.0% |  | 0.0% |  | 0.1% | -0.1% | 0.1% | 0.0% |
| Class C |  | 0.0% |  | -0.1% |  | 0.1% | 0.0% | 0.1% | 0.0% |
| Class D |  | 0.0% |  | -0.3% |  | 0.2% | 0.0% | 0.5% | -0.1% |
| Class E |  |  |  |  |  |  |  |  |  |
| Overall |  | 0.0% |  | -0.1% |  | 0.1% | 0.0% | 0.2% | 0.0% |
|  |  | 0.0% |  | -0.1% |  | 0.1% | 0.0% | 0.2% | -0.1% |
| Random Access HE-10 |  |  |  |  |  |  |  |  |  |
| Y | U |  | V |  |  |  |  |  |  |
| 0.0% | 0.0% |  | 0.0% |  |  |  |  |  |  |
| 0.0% | 0.0% |  | 0.0% |  |  |  |  |  |  |
| 0.0% | 0.0% |  | 0.0% |  |  |  |  |  |  |
| 0.0% | -0.2% |  | 0.2% |  |  |  |  |  |  |
| #VALUE! | #VALUE! |  | #VALUE! |  |  |  |  |  |  |
| 0.0% | -0.1% |  | 0.1% |  |  |  |  |  |  |
| 0.0% | 0.0% |  | 0.0% |  |  |  |  |  |  |

|  | Low delay B HE |  |  | Low delay B LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.0% | 0.2% | 0.2% | 0.0% | -0.2% | 0.2% |
| Class C | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% | -0.2% |
| Class D | 0.0% | -0.3% | -0.1% | 0.0% | 0.1% | 0.2% |
| Class E | 0.0% | 0.5% | -1.2% | 0.0% | 0.1% | 0.3% |
| Overall | 0.0% | 0.1% | -0.2% | 0.0% | 0.0% | 0.1% |
|  | 0.0% | 0.1% | -0.2% | 0.0% | 0.0% | 0.1% |

|  | Low delay P HE |  |  | Low delay P LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.0% | 0.3% | -0.1% | 0.0% | 0.1% | -0.6% |
| Class C | 0.0% | 0.2% | -0.1% | 0.0% | 0.1% | 0.1% |
| Class D | 0.1% | 0.2% | -0.3% | -0.1% | -0.3% | -0.1% |
| Class E | 0.0% | 0.1% | -1.1% | -0.1% | -0.7% | -0.3% |
| Class F | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Overall | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
|  | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |

Table 2: Results of the proposed transform

# Conclusion

The proposal provides the new core spatial transform that is with less addition complexity than the current HM transform.

# Reference

- F. Bossen, “Common test conditions and software reference configurations,” ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, Document JCTVC-G1200, Nov. 2011.

# Patent rights declaration(s)

Hongbo Zhu may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).